    - Turn ignition On.

### MIL will indicate DTC(s) by number of flashes. Acura: Accessing Diagnostic Trouble Codes

## 86-94 Acura

The Engine Control Unit (ECU) stores trouble codes. On the outside of the ECU case are one or two LEDs. If the ECU has 2 LEDs, ignore the Amber LED and look at the Red LED. On 87-89 models, the ECU is located beneath the front passenger seat. 1990-1994 models locate the ECU under the dashboard on the passenger's side beneath the carpet. Legend models provide a flip-out mirror that allows you to view the LEDs. Earlier models have a "window" at the base of the passenger front seat that allows you to view the LEDs.

image used with permission Copyright © 2002 ALLDATA LLC.

The Check Engine light will illuminate when the ECU stores a trouble code.

Counting the Flashes

To retrieve trouble codes, turn the ignition key to the "on" position with the engine off.

86-89 models:

The ECU will flash many short blinks. The total number of blinks equals the code number. For example, 12 short blinks equals code 12, nine short blinks equals code 9, etc.. If more than one code is stored, the ECU will flash the codes in sequence (lowest to highest) with a pause between each code. The codes will repeat until the key is turned to the OFF position.

90-94 models:

The ECU will flash a long blink to represent the first digit of the code, then a series of short blinks to represent the second digit. For example, 1 long blink, followed by 4 short blinks equals code 14. Four long blinks followed by five short blinks equals code 45.

NOTE: Legend Models: If code 7 or 17 is stored and the S4 light on the transaxle indicator panel is also flashing, it may be necessary to diagnose the automatic transaxle control unit.

### 86–90 Legend & 86–91 Integra

In the event of a PGM-FI system malfunction, the engine warning lamp will light and stay lit. When this condition occurs, it will be necessary to inspect the ECU LED display. On Integra models, the ECU contains a single red LED display. On Legend models, the ECU contains a red and a yellow LED display. The red LED should be used for both models when diagnosing PGM-FI system malfunctions.

When the engine warning lamp is illuminated, proceed as follows:

- On all models except 1991 Integra, move front passenger seat to rear position.

- On 1991 Integra models, pull down passenger side carpet from under dashboard.

- On all models, turn ignition On and observe red LED on front or top of ECU. The LED will indicate diagnostic trouble codes (DTCs) by number of flashes.

### NSX, Vigor, 91–95 Legend, 92–95 Integra & 91–96 NSX

In the event of a PGM-FI system malfunction, the Check Engine lamp will light and stay On. When this condition occurs, proceed as follows:

- Using a jumper wire, connect terminals of service check connector.

| Service check connector location. 91–95 Legend & 91–96 NSX | Service check connector location. Vigor | Service check connector location. 92–95 Integra |
| --- | --- | --- |

  - The Check Engine lamp will indicate DTCs by the number of flashes.

### 96–01 Integra

When the Malfunction Indicator Lamp (MIL) has been on or if there is driveability problem, proceed as follows:

- To use OBD II scan tool, proceed as follows:

  - Connect scan tool to 16-pin data link connector (DLC) located behind RH side of front console.

Scan tool connection location. 96–01 Integra

    - Turn ignition On.

    - Follow scan tool manufacturer’s operating instructions to access DTCs.

  - To use MIL, proceed as follows:

    - Connect SCS service connector special tool No. 07PAZ-0010100, or equivalent, to service check connector located under passenger’s side of dash.

Service check connector location. 96–01 Integra

### 97–01 NSX

When Malfunction Indicator Lamp (MIL) has been on or there is driveability condition, proceed as follows:

  - To use OBD II scan tool, proceed as follows:

    - Connect scan tool to 16-pin data link connector (DLC) located under glove compartment under a removable cover.

    - Turn ignition On.

    - Follow scan tool manufacturer’s operating instructions to access DTCs.

  - To use MIL, proceed as follows:

    - Connect SCS service connector special tool No. 07PAZ-0010100, or equivalent, to service check connector located under passenger’s side of dash

    - Turn ignition On.

    - MIL will indicate DTC(s) by number of flashes.

### 2.2CL, 2.3CL, 2.5TL, 3.0CL, 3.2CL, 3.2TL 3.5MDX & 3.5RL

When Malfunction Indicator Lamp (MIL) has been on or there is driveability problem, proceed as follows:

- To use OBD II scan tool, proceed as follows:

  - Connect scan tool to 16-pin data link connector (DLC) located behind or under the ashtray on center console.

  - Turn ignition On.

  - Follow scan tool manufacturer’s operating instructions to access DTCs.

- To use MIL, proceed as follows:

  - Connect SCS service connector special tool No. 07PAZ-0010100, or equivalent, to service check connector located under passenger’s side of dash.

  - Turn ignition On.

    - MIL will indicate DTC(s) by number of flashes.

### SLX

Connect a suitable scan tool to the DLC (Data Link Connector), located at the lower LH of the instrument panel behind a small square cover.

## Acura: Clearing Diagnostic Trouble Codes

### Legend, Vigor, 86–95 Integra & 91–96 NSX

After making all necessary repairs, the DTCs should be cleared as follows from the ECU memory.

- On 86–89 Integra models, remove hazard fuse at battery positive terminal for approximately 10 seconds to clear all stored DTCs.

- On 90 Integra models, remove hazard fuse in main fuse box for approximately 10 seconds to clear all stored DTCs.

- On 91–95 Integra models, remove back-up fuse from under hood fuse box for approximately 10 seconds to clear all stored DTCs.

- On 86–90 Legend models, remove alternator sense fuse in underhood relay box for 10 seconds to clear all DTCs.

- On 91–95 Legend models, remove No. 15 fuse from under dash fuse box for 10 seconds to clear all DTCs. This will also cancel power seat settings.

- On NSX models, remove clock fuse from main relay box, located in luggage compartment on RH side, for approximately 10 seconds to clear all DTCs. This will also cancel radio presets and clock setting.

- On Vigor models, remove back-up fuse from under hood fuse/relay box for approximately 10 seconds to clear all DTCs. This will also cancel radio presets and clock setting

### 2.2CL, 2.3CL, 2.5TL, 3.0CL, 3.2CL, 3.2TL, 3.5 MDX, 3.5 RL, 
96–01 Integra & 97–01 NSX

If using scan tool, follow scan tool instructions to clear ECM/PCM memory.

If using MIL, turn ignition Off. On models except NSX and 99–01 3.2TL,remove Backup Radio Fuse from underhood fuse relay box for 10 seconds. On NSX models, remove Clock Fuse from underhood fuse relay box for 10 seconds. On all models, disconnect SCS service connector. On 99–01 3.2TL models, remove Clock Backup Fuse from passenger compartment under dash fuse relay box for 10 seconds. On all models, disconnect SCS service connector.

### SLX

Follow scan tool instructions to clear PCM memory.

## Trouble codes

|  | Vigor, Legend |
| --- | --- |
|  | 2.2CL |
|  | 2.3CL |
|  | 2.5TL |
|  | 3.0CL |
|  | 3.2CL |
|  | 3.5RL |
|  | SLX |
| 86-94 | Acura Basic Codes |
| 88–95 | Integra |
| 91–96 | NSX |
| 96–01 | Integra |
| 96–98 | 3.2TL |
| 97–01 | NSX |
| 99–01 | 3.2TL & 3.5L MDX |

Information provided by AlldataLLC 
Copyright © 2002 ALLDATA LLC.

batauto.com©2002

96 & newer vehicles are OBD2 systems 
and REQUIRE A SCANNER, unless otherwise stated.
Warning: Please check for manufacturer specific codes and 
confirm the trouble code descriptions with an appropriate manual. 
Disclaimer: We cannot guarantee the accuracy of all trouble codes.
We try and make them as accurate as we can, but sometimes we all make mistakes.

## Vigor, Legend, 88–95 Integra & 91–96 NSX

| Integra | Legend | NSX | Vigor |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | ECM |
| 1 | — | — | 1 | HO2S |
| — | 1{1} | — | — | LH HO2S |
| — | 1{2} | 1 | — | Front HO2S |
| — | 2{1} | — | — | RH HO2S |
| — | 2{2} | 2 | — | Rear HO2S |
| 3 | 3 | 3 | 3 | MAP Sensor |
| 4 | — | — | — | CKP Sensor |
| — | 4 | — | — | CKP Sensor Position 1 |
| — | — | 4 | — | CKP Sensor Position A |
| — | 5 | 5 | 5 | MAP Sensor |
| 6 | 6 | 6 | 6 | ECT Sensor |
| 7 | 7 | 7 | 7 | TP Sensor |
| 8 | — | — | 8 | TDC Sensor |
| 9 | — | — | 9 | CYP Sensor |
| — | 9 | — | — | CYP Sensor Position 1 |
| — | — | 9 | — | CYP Sensor Position A |
| 10 | 10 | 10 | 10 | IAT Sensor |
| — | 12 | 12 | 12 | EGR Valve Lift Sensor |
| 13 | 13 | 13 | 13 | BARO Sensor |
| 14 | 14 | 14 | 14 | IAC Valve |
| 15 | 15 | 15 | 15 | Ignition Output Signal |
| 16 | 16 | 16 | 16 | Fuel Injection |
| 17 | 17 | 17 | 17 | VSS |
| — | 18 | 18 | 18 | Ignition Timing Adjuster |
| 20 | — | — | 20 | ELD |
| 21 | — | — | — | VTEC Solenoid Valve |
| — | — | 21 | — | Front VTEC Solenoid Valve |
| 22 | — | — | — | VTEC Pressure Switch |
| — | — | 22 | — | Front VTEC Pressure Switch |
| 23 | — | — | — | KS |
| 23 | 23 | — | — | LH KS |
| — | — | 23 | 23 | Front KS |
| 30 | — | 30 | 30 | A/T FI Signal A |
| 31 | — | 31 | 31 | A/T FI Signal B |
| — | 35 | 35 | — | TC STB Signal |
| — | 36 | 36 | — | TCFC Signal |
| 41 | — | — | 41 | HO2S Heater |
| — | 41 | — | — | LH HO2S Heater |
| — | — | 41 | — | Front HO2S Heater |
| — | 42 | — | — | RH HO2S Heater |
| — | — | 42 | — | Rear HO2S Heater |
| 43 | — | — | 43 | Fuel Supply System |
| — | 43 | — | — | LH Fuel Supply System |
| — | — | 43 | — | Front Fuel Supply System |
| — | 44 | — | — | RH Fuel Supply System |
| — | — | 44 | — | Rear Fuel Supply System |
| — | — | — | 45 | Fuel Metering |
| — | 45 | — | 45 | LH Fuel Metering |
| — | — | 45 | — | Front Fuel Metering |
| — | 46 | — | — | RH Fuel Metering |
| — | — | 46 | — | Rear Fuel Metering |
| — | — | 47 | — | Fuel Pump |
| — | — | 51 | — | Rear VTEC Solenoid Valve |
| — | — | 52 | — | Rear VTEC Pressure Switch |
| — | 53 | — | 53 | RH KS |
| — | — | 53 | 53 | Rear KS |
| — | 54 | — | — | CKP Sensor Position 2 |
| — | — | 54 | — | CKP Sensor Position B |
| — | 59 | — | — | CYP Sensor Position 2 |
| — | — | 59 | — | CYP Sensor Position B |

{1}—1991–95 models.
{2}—1988–90 models.

# 2.2CL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10 | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 1 | Sensor 1 Primary HO2S Circuit Low Voltage |
| P0132 | 1 | Sensor 1 Primary HO2S Circuit High Voltage |
| P0133 | 61 | Primary HO2S Slow Response |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage |
| P0138 | 63 | Secondary HO2S Circuit High Voltage |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0335 | 4 | CKP Sensor Circuit Malfunction |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0441 | 92 | EVAP Improper Purge Flow |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Low Input |
| P0501 | 17 | VSS Circuit Range/Performance |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0730, P0740, P0753 & P0758 | 70 | A/T |
| P1106 | 13 | BARO Circuit Range/Performance Problem |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | Manifold Absolute Pressure Lower Than Expected |
| P1129 | 5 | Throttle Position Higher Than Expected |
| P1253 | 21 | VTEC System Malfunction |
| P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1300 | — | Random Misfire |
| P1359 | 8 | CKP/TDC Sensor Circuit Connector Disconnection |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC 1 Sensor No Signal |
| P1381 | 9 | CYP Sensor Intermittent Interruption |
| P1382 | 9 | CYP Sensor No Signal |
| P1456 | 90 | EVAP System Leak Detected (Fuel Tank System) |
| P1457 | 90 | EVAP System Leak Detected (Control Canister System) |
| P1459 | 92 | EVAP Purge Flow Switch Malfunction |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1519 | 14 | Idle Air Control Valve Circuit |
| P1607 | — | ECM/PCM Internal Circuit Failure |
| P1705, P1706, P1738, P1739, P1753, P1758, P1768, P1773, P1791 | 70 | A/T |

## 2.3CL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 1 | Sensor 1 Primary HO2S Circuit Low Voltage |
| P0132 | 1 | Sensor 1 Primary HO2S Circuit High Voltage |
| P0133 | 61 | Sensor 1 Primary HO2S Slow Response |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage |
| P0138 | 63 | Secondary HO2S Circuit High Voltage |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0300 | — | Random Misfire |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0325 | 23 | KS Sensor Circuit Malfunction |
| P0335 | 4 | CKP Sensor Circuit Malfunction |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Low Input |
| P0501 | 17 | VSS Circuit Range/Performance |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0730, P0740, P0753 & P0758 | 70 | A/T |
| P1106 | 13 | BARO Circuit Range/Performance Problem |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | Manifold Absolute Pressure Lower Than Expected |
| P1129 | 5 | Manifold Absolute Pressure Higher Than Expected |
| P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1300 | — | Random Misfire |
| P1359 | 8 | CKP/TDC Sensor Circuit Connector Disconnection |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC Sensor No Signal |
| P1381 | 9 | CYP Sensor Intermittent Interruption |
| P1382 | 9 | CYP Sensor No Signal |
| P1456 | 90 | EVAP System Leak Detected (Fuel Tank System) |
| P1457 | 90 | EVAP System Leak Detected (Control Canister System) |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1519 | 14 | Idle Air Control Valve Circuit |
| P1607 | — | ECM/PCM Internal Circuit Failure |
| P1705, P1706, P1738, P1739, P1753, P1768, P1773, P1791 | 70 | A/T |

## 2.5TL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0101 | 50 | MAF Circuit Range/Performance |
| P0102 | 50 | MAF Circuit Low Input |
| P0103 | 50 | MAF Circuit High Input |
| P0111 | 10 | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0130 | 1 | Primary HO2S Circuit Malfunction |
| P0133 | 61 | Primary HO2S Slow Response |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0300 | 76 | Random Misfire Detected |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0305 | 75 | Cylinder 5 Misfire |
| P0327 | 23 | Front KS Circuit Low Input |
| P0328 | 23 | Front KS Circuit High Input |
| P0332 | 53 | Rear KS Circuit Low Input |
| P0333 | 53 | Rear KS Circuit High Input |
| P0335 | 4 | CKP Sensor Circuit Low Input |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0440 | 90 | EVAP System Leak Detected |
| P0441 | 92 | EVAP Improper Purge Flow |
| P0452 | 91 | EVAP System Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | EVAP System Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Low Input |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0725, P0730, P0740, P0753 & P0758 | 70 | A/T |
| P1102 | 50 | MAF Lower Than Expected |
| P1103 | 50 | MAF Higher Than Expected |
| P1106 | 13 | BARO Circuit Range/Performance |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | TP Lower Than Expected |
| P1122 | 7 | TP Higher Than Expected |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1336 | 54 | Crankshaft Speed Fluctuation Sensor Intermittent Interruption |
| P1337 | 54 | Crankshaft Speed Fluctuation Sensor No Signal |
| P1359 | 8 | CKP/TDC Sensor Circuit Connector Disconnection |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC Sensor No Signal |
| P1381 | 9 | CYP Sensor Intermittent Interruption |
| P1382 | 9 | CYP Sensor No Signal |
| P1459 | 92 | EVAP Purge Flow Switch Malfunction |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1660 | 70 | A/T FI Data Line Failure |
| P1681 | 30 | A/T FI Signal A Low Input |
| P1682 | 30 | A/T FI Signal A High Input |
| P1686 | 31 | A/T FI Signal B Low Input |
| P1687 | 31 | A/T FI Signal B High Input |
| P1705, P1706, P1753, P1758, P1768, P1786, P1787, P1790, P1791, P1792 & P1794 | 70 | A/T |

## 3.0CL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10 | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 1 | Sensor 1 Primary HO2S Circuit Low Voltage |
| P0132 | 1 | Sensor 1 Primary HO2S Circuit High Voltage |
| P0133 | 61 | Primary HO2S Slow Response |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage |
| P0138 | 63 | Secondary HO2S Circuit High Voltage |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0300 | — | Random Misfire |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0305 | 75 | Cylinder 5 Misfire |
| P0306 | 76 | Cylinder 6 Misfire |
| P0335 | 4 | CKP Sensor Circuit Malfunction |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Low Input |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0725, P0730, P0740, P0753, P0758 & P0763 | 70 | A/T |
| P1106 | 13 | BARO Sensor Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1300 | — | Random Misfire |
| P1301 | 71 | Cylinder 1 Random Misfire |
| P1302 | 72 | Cylinder 2 Random Misfire |
| P1303 | 73 | Cylinder 3 Random Misfire |
| P1304 | 74 | Cylinder 4 Random Misfire |
| P1305 | 75 | Cylinder 5 Random Misfire |
| P1306 | 76 | Cylinder 6 Random Misfire |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC 1 Sensor No Signal |
| P1366 | 58 | TDC 2 Sensor Intermittent |
| P1367 | 58 | TDC 2 Sensor No Signal |
| P1456 | 90 | EVAP System Leak Detected (Fuel Tank System) |
| P1457 | 90 | EVAP Control System Leak Detected (Control Canister System) |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1519 | 14 | IAC Valve Circuit Failure |
| P1607 | — | ECM/PCM Internal Circuit Failure |
| P1655 | — | SEAF/SEFA Signal Line Failure |
| P1705, P1706, P1738, P1739, P1753, P1768, P1773 & P1791 | 70 | A/T |

## 3.2CL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0128 | 87 | Thermostat Range/Performance Problem |
| P0131 | 1 | Sensor 1 Primary HO2S Circuit Low Voltage |
| P0132 | 1 | Sensor 1 Primary HO2S Circuit High Voltage |
| P0133 | 61 | Primary HO2S Slow Response |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage |
| P0138 | 63 | Secondary HO2S Circuit High Voltage |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0300 | — | Random Misfire |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0305 | 75 | Cylinder 5 Misfire |
| P0306 | 76 | Cylinder 6 Misfire |
| P0325 | 23 | Knock Sensor (KS) Circuit Malfunction |
| P0335 | 4 | CKP Sensor Circuit Malfunction |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0451 | 91 | Fuel Tank Pressure Sensor Circuit Range/Performance Problem |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Low Input |
| P0505 | 14 | IAC System Malfunction |
| P0710 P0715 P0720 P0730 P0740 P0753 P0758 P0760 P0780 | 70 | A/T |
| P1106 | 13 | BARO Sensor Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC 1 Sensor No Signal |
| P1366 | 58 | TDC 2 Sensor Intermittent |
| P1367 | 58 | TDC 2 Sensor No Signal |
| P1456 | 90 | EVAP System Leak Detected (Fuel Tank System) |
| P1457 | 90 | EVAP Control System Leak Detected (Control Canister System) |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1519 | 14 | IAC Valve Circuit Failure |
| P1607 | — | ECM/PCM Internal Circuit Failure |
| P1656 | 70 | SEAF/SEFA Signal Line Failure |
| P1676 | 35 | FPTDR Signal Line Failure |
| P1678 | 35 | FPTDR Signal Line Failure |
| P1705 P1706 P1709 P1710 P1713 P1738 P1739 P1740 P1753 P1768 P1773 P1778 | 70 | A/T |

## 3.5RL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0106 | 5{3} | MAP Circuit Range/Performance |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10{4} | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86{2} | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 2{2} | Sensor Primary HO2S Circuit Low Voltage (Bank 1 Sensor 1) |
| P0132 | 2{2} | Sensor Primary HO2S Circuit High Voltage (Bank 1 Sensor 1) |
| P0133 | 62{2} | RH Primary HO2S Slow Response (Bank 1 Sensor 1) |
| P0135 | 42 | RH Primary HO2S Heater Circuit Malfunction (Bank 1 Sensor 1) |
| P0137 | 63{2} | Secondary HO2S Circuit Low Voltage (Senor 2) |
| P0138 | 63{2} | Secondary HO2S Circuit High Voltage (Sensor 2) |
| P0139 | 63{2} | Secondary HO2S Slow Response (Sensor 2) |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction (Sensor 2) |
| P0151 | 1{2} | LH Primary HO2S Circuit Low Voltage (Bank 2 Sensor 1) |
| P0152 | 1{2} | LH Primary HO2S Circuit High Voltage (Bank 2 Sensor 1) |
| P0153 | 61{2} | LH Primary HO2S Slow Response (Bank 2 Sensor 1) |
| P0155 | 41 | LH Primary HO2S Heater Circuit Malfunction (Bank 2 Sensor 1) |
| P0171 | 46{2} | System Too Lean, RH Bank (Bank 1) |
| P0172 | 46{2} | System Too Rich, RH Bank (Bank 1) |
| P0174 | 45{2} | System Too Lean, LH Bank (Bank 2) |
| P0175 | 45{2} | System Too Rich, LH Bank (Bank 2) |
| P0300{9} | 71 thru 76 | Random Misfire |
| P0325 | 53 | RH KS (Bank 1) Circuit Malfunction |
| P0330 | 23 | LH Knock Sensor (Bank 2) Circuit Malfunction |
| P0335 | 4 | CKP Sensor 1 Circuit Malfunction |
| P0336 | 4 | CKP Sensor 1 Range/Performance |
| P0401 | 80{2} | EGR Insufficient Flow |
| P0420 | 67{2} | Catalyst System Efficiency Below Threshold |
| P0451{10} | 91 |  |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0505 | 14 | IAC System Malfunction |
| P0715 P0720 P0730 P0740 P0753 P0758 | 70{1}{5} | A/T |
| P1106 | 13{7} | BARO Sensor Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7{7} | Throttle Position Lower Than Expected |
| P1122 | 7{7} | Throttle Position Higher Than Expected |
| P1128 | 5{7} | MAP Lower Than Expected |
| P1129 | 5{7} | MAP Higher Than Expected |
| P1201 | 71 | Cylinder 1 Misfire |
| P1202 | 72 | Cylinder 2 Misfire |
| P1203 | 73 | Cylinder 3 Misfire |
| P1204 | 74 | Cylinder 4 Misfire |
| P1205 | 75 | Cylinder 5 Misfire |
| P1206 | 76 | Cylinder 6 Misfire |
| P1300{8} | — | Random Misfire |
| P1301 | 71 | Cylinder 1 Misfire |
| P1302 | 72 | Cylinder 2 Misfire |
| P1303 | 73 | Cylinder 3 Misfire |
| P1304 | 74 | Cylinder 4 Misfire |
| P1305 | 75 | Cylinder 5 Misfire |
| P1306 | 76 | Cylinder 6 Misfire |
| P1316 | 79 | Spark Plug Voltage Detection Circuit Malfunction, LH Bank (Bank 2) |
| P1317 | 79 | Spark Plug Voltage Detection Circuit Malfunction, RH Bank (Bank 2) |
| P1318 | 79 | Spark Plug Voltage Detection Module Reset Circuit Malfunction, LH Bank (Bank 2) |
| P1319 | 79 | Spark Plug Voltage Detection Module Reset Circuit Malfunction, RH Bank (Bank 1) |
| P1336 | 54 | CKP Sensor 2 Intermittent Interruption |
| P1337 | 54 | CKP Sensor 2 No Signal |
| P1381 | 9 | CYP Sensor 1 Intermittent Interruption |
| P1382 | 9 | CYP Sensor 1 No Signal |
| P1386 | 59 | CYP Sensor 2 Intermittent Interruption |
| P1387 | 59 | CYP Sensor 2 No Signal |
| P1456 | 90{2} | Evaporative Emission Control System Leak Detected (Fuel Tank System) |
| P1457 | 90{2} | Evaporative Emission Control System Leak Detected (Control Canister System) |
| P1486 | 87 | Cooling System Malfunction |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1607 | — | PCM Internal Circuit Failure |
| P1690 | 35{6} | Traction Control System Stand-By Line Failure |
| P1696 | 36{6} | Traction Control Fuel Cut Signal Low Input |
| P1697 | 36{6} | Traction Control Fuel Cut Signal High Input |
| P1705 P1706 P1753 P1758 P1768 P1791 | 70{5} | A/T |

{1}—These DTCs will be indicated by the blinking of the MIL with the SCS service connector connected.
{2}—These DTCs require two ‘‘trips’’ or two driving cycles unless the SCS service connector is connected.
{3}—1996 model only.
{4}—1997 model only.
{5}—The D4 indicator light and the MIL may come on simultaneously.
{6}—With TCS, VSA
{7}—1997–2001 models.
{8}—1996–1997 models
{9}—1998–2001 models
{10}—2000–2001 models

## SLX

| OBD2 |  |
| --- | --- |
| P0101 | MAF System Performance |
| P0102 | MAF Sensor Circuit Low Frequency |
| P0103 | MAF Sensor Circuit High Frequency |
| P0106 | MAP System Performance |
| P0107 | MAP Circuit Low Voltage |
| P0108 | MAP Circuit High Voltage |
| P0112 | IAT Sensor Circuit Low Voltage |
| P0113 | IAT Sensor Circuit High Voltage |
| P0117 | ECT Sensor Circuit Low Voltage |
| P0118 | ECT Sensor Circuit High Voltage |
| P0121 | TP System Performance |
| P0122 | TP Sensor Circuit Low Voltage |
| P0123 | TP Sensor Circuit High Voltage |
| P0125 | ECT Excessive Time To Closed Loop Fuel Control |
| P0131 | HO2S Circuit Low Voltage (Bank 1 Sensor 1) |
| P0132 | HO2S Circuit High Voltage (Bank 1 Sensor 1) |
| P0133 | HO2S Slow Response (Bank 1 Sensor 1) |
| P0134 | HO2S Circuit Insufficient Activity (Bank 1 Sensor 1) |
| P0135 | HO2S Heater Circuit Malfunction (Bank 1 Sensor 1) |
| P0137 | HO2S Circuit Low Voltage (Bank 1 Sensor 2) |
| P0138 | HO2S Circuit High Voltage (Bank 1 Sensor 2) |
| P0140 | HO2S Circuit Insufficient Activity (Bank 1 Sensor 2) |
| P0141 | HO2S Heater Circuit Malfunction (Bank 1 Sensor 2) |
| P0143 | HO2S Circuit Low Voltage (Bank 1 Sensor 3) M/T Only |
| P0144 | HO2S Circuit High Voltage (Bank 1 Sensor 3) M/T Only |
| P0146 | HO2S Circuit Insufficient Activity (Bank 1 Sensor 3) M/T Only |
| P0147 | HO2S Heater Circuit Malfunction (Bank 1 Sensor 3) M/T Only |
| P0151 | HO2S Circuit Low Voltage (Bank 2 Sensor 1) |
| P0152 | HO2S Circuit High Voltage (Bank 2 Sensor 1) |
| P0153 | HO2S Slow Response (Bank 2 Sensor 1) |
| P0154 | HO2S Circuit Insufficient Activity (Bank 2 Sensor 1) |
| P0155 | HO2S Heater Circuit Malfunction (Bank 2 Sensor 1) |
| P0157 | HO2S Circuit Low Voltage (Bank 2 Sensor 2) |
| P0158 | HO2S Circuit High Voltage (Bank 2 Sensor 2) |
| P0160 | HO2S Circuit Insufficient Activity (Bank 2 Sensor 2) |
| P0161 | HO2S Heater Circuit Malfunction (Bank 2 Sensor 2) |
| P0171 | Fuel Trim System Lean (Bank 1) |
| P0172 | Fuel Trim System Rich (Bank 1) |
| P0174 | Fuel Trim System Lean (Bank 2) |
| P0175 | Fuel Trim System Rich (Bank 2) |
| P0201 | Injector 1 Control Circuit |
| P0202 | Injector 2 Control Circuit |
| P0203 | Injector 3 Control Circuit |
| P0204 | Injector 4 Control Circuit |
| P0205 | Injector 5 Control Circuit |
| P0206 | Injector 6 Control Circuit |
| P0218 | Transmission Over Temperature Condition, A/T Only |
| P0300 | Misfire Detected |
| P0301 | Cylinder 1 Misfire |
| P0302 | Cylinder 2 Misfire |
| P0303 | Cylinder 3 Misfire |
| P0304 | Cylinder 4 Misfire |
| P0305 | Cylinder 5 Misfire |
| P0306 | Cylinder 6 Misfire |
| P0325 | KS Module Circuit Malfunction |
| P0327 | Front KS Circuit |
| P0336 | 58X Reference Signal Circuit |
| P0337 | CKP Sensor Circuit Low Frequency |
| P0341 | CMP Sensor Circuit Performance |
| P0342 | CMP Sensor Circuit Low |
| P0351 | Ignition 1 Control Circuit |
| P0352 | Ignition 2 Control Circuit |
| P0353 | Ignition 3 Control Circuit |
| P0354 | Ignition 4 Control Circuit |
| P0355 | Ignition 5 Control Circuit |
| P0356 | Ignition 6 Control Circuit |
| P0401 | EGR Insufficient Flow |
| P0402 | EGR Pintle Crank Open Error |
| P0404 | EGR Stuck Open |
| P0405 | EGR Low Voltage |
| P0406 | EGR High Voltage |
| P0420 | TWC System Low Efficiency (Bank 1) |
| P0430 | TWC System Low Efficiency (Bank 2) A/T Only |
| P0440 | EVAP System No Flow During Purge |
| P0441 | EVAP No Flow During Purge |
| P0442 | EVAP System Small Leak |
| P0446 | EVAP Canister Vent Blocked |
| P0452 | Tank Pressure Sensor Low Voltage |
| P0453 | Tank Pressure Sensor High Voltage |
| P0462 | Fuel Level Sensor Circuit Low Voltage |
| P0463 | Fuel Level Sensor Circuit High Voltage |
| P0502 | VSS Circuit Low Input |
| P0506 | IAC System Low RPM |
| P0507 | IAC System High RPM |
| P0560 | System Voltage Low |
| P0562 | System Voltage Low |
| P0563 | System Voltage High |
| P0601 | PCM Memory |
| P0705 | Transmission Range Switch Circuit |
| P0706 | Transmission Range Switch Performance |
| P0712 | Transmission Fluid Temperature System Low Voltage |
| P0713 | Transmission Fluid Temperature System High Voltage |
| P0719 | Brake Switch Circuit Low, A/T Only |
| P0722 | Output Speed Sensor Circuit No Signal, A/T Only |
| P0723 | Output Speed Sensor Circuit Intermittent Signal, A/T Only |
| P0724 | Brake Switch Circuit High, A/T Only |
| P0730 | Improper Gear Ratio |
| P0742 | TCC Circuit Stuck On |
| P0748 | Transmission Pressure Control Solenoid Electrical Circuit Fault |
| P0751 | Transmission Shift Solenoid ‘‘A’’ Performance Or Stuck Off, A/T Only |
| P0753 | Transmission Shift Solenoid ‘‘A’’ Electrical Circuit Fault, A/T Only |
| P0756 | Transmission Shift Solenoid ‘‘B’’ Performance Or Stuck Off, A/T Only |
| P0758 | Transmission Shift Solenoid ‘‘B’’ Electrical Circuit Fault, A/T Only |
| P1106 | MAP Sensor Circuit Intermittent High Voltage |
| P1107 | MAP Sensor Circuit Intermittent Low Voltage |
| P1111 | IAT Sensor Circuit Intermittent High Voltage |
| P1112 | IAT Sensor Circuit Intermittent Low Voltage |
| P1114 | ECT Sensor Circuit Intermittent Low Voltage |
| P1115 | ECT Sensor Circuit Intermittent High Voltage |
| P1121 | TP Sensor Circuit Intermittent High Voltage |
| P1122 | TP Sensor Circuit Intermittent Low Voltage |
| P1133 | HO2S Insufficient Switching (Bank 1 Sensor 1) |
| P1134 | HO2S Transition Time Ratio (Bank 1 Sensor 1) |
| P1153 | HO2S Insufficient Switching (Bank 2 Sensor 1) |
| P1154 | HO2S Transition Time Ratio (Bank 2 Sensor 1) |
| P1171 | Fuel System Lean During Acceleration |
| P1380 | ABS Rough Road System Fault |
| P1381 | ABS Rough Class 2 Serial Link Error |
| P1390 | G-Sensor Intermittent Low Voltage |
| P1391 | G-Sensor Performance |
| P1392 | G-Sensor Low Voltage |
| P1393 | G-Sensor High Voltage |
| P1394 | G-Sensor Intermittent High Voltage |
| P1404 | EGR Stuck Closed |
| P1406 | EGR Valve Pintle Position Circuit |
| P1441 | EVAP System Flow During Non-Purge |
| P1442 | EVAP Vacuum Switch High Voltage During ignition On |
| P1508 | IAC System Low RPM |
| P1509 | IAC System High RPM |
| P1618 | Serial Peripheral Interface PCM Interprocessor Communication Error, A/T Only |
| P1625 | PCM Unexpected Reset |
| P1640 | Driver 1 Input Voltage High |
| P1650 | Quad Driver Module A fault |
| P1790 | TRANS ROM Checksum Error |
| P1792 | TRANS EEPROM Checksum Error |
| P1835 | TRANS Kick Down Switch Malfunction |
| P1850 | Brake Band Apply Solenoid Malfunction |
| P1860 | TCC PCM Solenoid Circuit Fault |
| P1870 | Transmission Component Slipping |

## 86-94 Acura Basic Codes

| MIL |  |
| --- | --- |
| 1 | Oxygen sensor or circuit (Integra) |
| 1 | Front oxygen sensor or circuit (Legend) |
| 2 | Rear Oxygen sensor or Circuit (Legend) |
| 3 | MAP Sensor or circuit |
| 4 | Crank Angle Sensor or circuit (Integra) |
| 4 | Crank Angle Sensor No.1 (Legend) |
| 5 | MAP Sensor or circuit |
| 6 | Coolant Temperature Sensor or circuit |
| 7 | Throttle Angle Sensor or circuit |
| 8 | TDC sensor or circuit (Integra) |
| 9 | Cylinder position sensor No.1 (Legend) |
| 9 | Crank Angle Sensor or circuit (Integra) |
| 10 | Intake Air Temperature Sensor or circuit |
| 12 | EGR Control system |
| 13 | Atmospheric Pressure Sensor or circuit |
| 14 | Idle Control System |
| 15 | Ignition Output signal |
| 16 | Fuel Injector |
| 17 | Vehicle Speed Sensor or circuit |
| 18 | Ignition timing adjustment (Legend only) |
| 19 | Lock-up control solenoid valve (Integra) |
| 20 | Electric Load (Integra) |
| 21 | Front spool solenoid valve (Integra) |
| 22 | Front timing oil pressure switch (Integra) |
| 23 | Front Knock Sensor (Legend) |
| 30 | ATF FL signal A (Legend) |
| 31 | ATF FL signal B (Legend) |
| 35 | TC Standby signal A (Legend) |
| 36 | TC FC signal (Legend) |
| 41 | Oxygen Sensor heater (Integra) |
| 41 | Front Oxygen Sensor heater (Legend) |
| 42 | Rear Oxygen Sensor heater (Legend) |
| 43 | Fuel supply system (Integra) |
| 43 | Front fuel supply system (Legend) |
| 44 | Rear fuel supply system (Legend) |
| 45 | Font fuel metering (Legend) |
| 46 | Rear fuel metering (Legend) |
| 47 | Fuel pump (Legend) |
| 51 | Rear spool solenoid valve (Legend) |
| 52 | Rar valve timing oil pressure switch (Legend) |
| 53 | Rear Knock Sensor (Legend) |
| 54 | Crank Angle B (Legend) |
| 59 | No. 1 Cylinder position (Legend) |

## Vigor, Legend, 88–95 Integra & 91–96 NSX

| Integra | Legend | NSX | Vigor |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | ECM |
| 1 | — | — | 1 | HO2S |
| — | 1{1} | — | — | LH HO2S |
| — | 1{2} | 1 | — | Front HO2S |
| — | 2{1} | — | — | RH HO2S |
| — | 2{2} | 2 | — | Rear HO2S |
| 3 | 3 | 3 | 3 | MAP Sensor |
| 4 | — | — | — | CKP Sensor |
| — | 4 | — | — | CKP Sensor Position 1 |
| — | — | 4 | — | CKP Sensor Position A |
| — | 5 | 5 | 5 | MAP Sensor |
| 6 | 6 | 6 | 6 | ECT Sensor |
| 7 | 7 | 7 | 7 | TP Sensor |
| 8 | — | — | 8 | TDC Sensor |
| 9 | — | — | 9 | CYP Sensor |
| — | 9 | — | — | CYP Sensor Position 1 |
| — | — | 9 | — | CYP Sensor Position A |
| 10 | 10 | 10 | 10 | IAT Sensor |
| — | 12 | 12 | 12 | EGR Valve Lift Sensor |
| 13 | 13 | 13 | 13 | BARO Sensor |
| 14 | 14 | 14 | 14 | IAC Valve |
| 15 | 15 | 15 | 15 | Ignition Output Signal |
| 16 | 16 | 16 | 16 | Fuel Injection |
| 17 | 17 | 17 | 17 | VSS |
| — | 18 | 18 | 18 | Ignition Timing Adjuster |
| 20 | — | — | 20 | ELD |
| 21 | — | — | — | VTEC Solenoid Valve |
| — | — | 21 | — | Front VTEC Solenoid Valve |
| 22 | — | — | — | VTEC Pressure Switch |
| — | — | 22 | — | Front VTEC Pressure Switch |
| 23 | — | — | — | KS |
| 23 | 23 | — | — | LH KS |
| — | — | 23 | 23 | Front KS |
| 30 | — | 30 | 30 | A/T FI Signal A |
| 31 | — | 31 | 31 | A/T FI Signal B |
| — | 35 | 35 | — | TC STB Signal |
| — | 36 | 36 | — | TCFC Signal |
| 41 | — | — | 41 | HO2S Heater |
| — | 41 | — | — | LH HO2S Heater |
| — | — | 41 | — | Front HO2S Heater |
| — | 42 | — | — | RH HO2S Heater |
| — | — | 42 | — | Rear HO2S Heater |
| 43 | — | — | 43 | Fuel Supply System |
| — | 43 | — | — | LH Fuel Supply System |
| — | — | 43 | — | Front Fuel Supply System |
| — | 44 | — | — | RH Fuel Supply System |
| — | — | 44 | — | Rear Fuel Supply System |
| — | — | — | 45 | Fuel Metering |
| — | 45 | — | 45 | LH Fuel Metering |
| — | — | 45 | — | Front Fuel Metering |
| — | 46 | — | — | RH Fuel Metering |
| — | — | 46 | — | Rear Fuel Metering |
| — | — | 47 | — | Fuel Pump |
| — | — | 51 | — | Rear VTEC Solenoid Valve |
| — | — | 52 | — | Rear VTEC Pressure Switch |
| — | 53 | — | 53 | RH KS |
| — | — | 53 | 53 | Rear KS |
| — | 54 | — | — | CKP Sensor Position 2 |
| — | — | 54 | — | CKP Sensor Position B |
| — | 59 | — | — | CYP Sensor Position 2 |
| — | — | 59 | — | CYP Sensor Position B |

{1}—1991–95 models.

## Vigor, Legend, 88–95 Integra & 91–96 NSX

| Integra | Legend | NSX | Vigor |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | ECM |
| 1 | — | — | 1 | HO2S |
| — | 1{1} | — | — | LH HO2S |
| — | 1{2} | 1 | — | Front HO2S |
| — | 2{1} | — | — | RH HO2S |
| — | 2{2} | 2 | — | Rear HO2S |
| 3 | 3 | 3 | 3 | MAP Sensor |
| 4 | — | — | — | CKP Sensor |
| — | 4 | — | — | CKP Sensor Position 1 |
| — | — | 4 | — | CKP Sensor Position A |
| — | 5 | 5 | 5 | MAP Sensor |
| 6 | 6 | 6 | 6 | ECT Sensor |
| 7 | 7 | 7 | 7 | TP Sensor |
| 8 | — | — | 8 | TDC Sensor |
| 9 | — | — | 9 | CYP Sensor |
| — | 9 | — | — | CYP Sensor Position 1 |
| — | — | 9 | — | CYP Sensor Position A |
| 10 | 10 | 10 | 10 | IAT Sensor |
| — | 12 | 12 | 12 | EGR Valve Lift Sensor |
| 13 | 13 | 13 | 13 | BARO Sensor |
| 14 | 14 | 14 | 14 | IAC Valve |
| 15 | 15 | 15 | 15 | Ignition Output Signal |
| 16 | 16 | 16 | 16 | Fuel Injection |
| 17 | 17 | 17 | 17 | VSS |
| — | 18 | 18 | 18 | Ignition Timing Adjuster |
| 20 | — | — | 20 | ELD |
| 21 | — | — | — | VTEC Solenoid Valve |
| — | — | 21 | — | Front VTEC Solenoid Valve |
| 22 | — | — | — | VTEC Pressure Switch |
| — | — | 22 | — | Front VTEC Pressure Switch |
| 23 | — | — | — | KS |
| 23 | 23 | — | — | LH KS |
| — | — | 23 | 23 | Front KS |
| 30 | — | 30 | 30 | A/T FI Signal A |
| 31 | — | 31 | 31 | A/T FI Signal B |
| — | 35 | 35 | — | TC STB Signal |
| — | 36 | 36 | — | TCFC Signal |
| 41 | — | — | 41 | HO2S Heater |
| — | 41 | — | — | LH HO2S Heater |
| — | — | 41 | — | Front HO2S Heater |
| — | 42 | — | — | RH HO2S Heater |
| — | — | 42 | — | Rear HO2S Heater |
| 43 | — | — | 43 | Fuel Supply System |
| — | 43 | — | — | LH Fuel Supply System |
| — | — | 43 | — | Front Fuel Supply System |
| — | 44 | — | — | RH Fuel Supply System |
| — | — | 44 | — | Rear Fuel Supply System |
| — | — | — | 45 | Fuel Metering |
| — | 45 | — | 45 | LH Fuel Metering |
| — | — | 45 | — | Front Fuel Metering |
| — | 46 | — | — | RH Fuel Metering |
| — | — | 46 | — | Rear Fuel Metering |
| — | — | 47 | — | Fuel Pump |
| — | — | 51 | — | Rear VTEC Solenoid Valve |
| — | — | 52 | — | Rear VTEC Pressure Switch |
| — | 53 | — | 53 | RH KS |
| — | — | 53 | 53 | Rear KS |
| — | 54 | — | — | CKP Sensor Position 2 |
| — | — | 54 | — | CKP Sensor Position B |
| — | 59 | — | — | CYP Sensor Position 2 |
| — | — | 59 | — | CYP Sensor Position B |

{1}—1991–95 models.
{2}—1988–90 models.

## 96–01 Integra

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10 | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 1 | Primary HO2S Circuit Low Voltage (Sensor 1) |
| P0132 | 1 | Primary HO2S Circuit High Voltage (Sensor 1) |
| P0133 | 61 | Primary HO2S Slow Response (Sensor 1) |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction (Sensor 1) |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage (Sensor 2) |
| P0138 | 63 | Secondary HO2S Circuit High Voltage (Sensor 2) |
| P0139 | 63 | Secondary HO2S Slow Response (Sensor 2) |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction (Sensor 2) |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0325 | 23 | KS Circuit Malfunction |
| P0335 | 4 | CKP Sensor Circuit Malfunction |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0441 | 92 | EVAP Improper Purge Flow |
| P0500 | 17 | VSS Circuit Low Input |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0725, P0730, P0740, P0753 & P0758 | 70 | A/T |
| P1106 | 13 | BARO Sensor Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1300 | 71 thru 74 | Random Misfire |
| P1336 | 54 | Crankshaft Speed Fluctuation Sensor Intermittent Interruption |
| P1337 | 54 | Crankshaft Speed Fluctuation Sensor No Signal |
| P1359 | 8 | CKP/TDC Sensor Circuit Connector Disconnection |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC 1 Sensor No Signal |
| P1381 | 9 | CYP Sensor Intermittent Interruption |
| P1382 | 9 | CYP Sensor No Signal |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1607 | — | ECM Internal Circuit Failure |
| P1660 | 70 | A/T FI Data Line Failure |
| P1681 | 30 | A/T FI Signal A Low Input |
| P1682 | 30 | A/T FI Signal A High Input |
| P1686 | 31 | A/T FI Signal B Low Input |
| P1687 | 31 | A/T FI Signal B High Input |
| P0700, P1705, P1706, P1753, P1758, P1786, P1790, P1791, P1792 & P1794 | — | A/T |

## 96–98 3.2TL

| OBD2 | MIL |  |
| --- | --- | --- |
| P0106 | 5 | MAP Circuit Range/Performance Problem |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10 | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 2 | Sensor 1 Primary HO2S Circuit Low Voltage |
| P0132 | 2 | Sensor 1 Primary HO2S Circuit High Voltage |
| P0133 | 62 | Primary HO2S Slow Response |
| P0135 | 42 | Primary HO2S Heater Circuit Malfunction |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage |
| P0138 | 63 | Secondary HO2S Circuit High Voltage |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction (Sensor 2) |
| P0151 | 1 | LH Primary HO2S Circuit Low Voltage (Bank 2 Sensor 1) |
| P0152 | 1 | LH Primary HO2S Circuit High Voltage (Bank 2 Sensor 1) |
| P0153 | 61 | LH Primary HO2S Slow Response (Bank 2 Sensor 1) |
| P0155 | 41 | LH Primary HO2S Heater Circuit Malfunction (Bank 2 Sensor 1) |
| P0171 | 46 | System Too Lean RH Bank (Bank 1) |
| P0172 | 46 | System Too Rich RH Bank (Bank 1) |
| P0174 | 45 | System Too Lean LH Bank (Bank 2) |
| P0175 | 45 | System Too Rich LH Bank (Bank 2) |
| P0325 | 53 | RH KS (Bank 1) Circuit Malfunction |
| P0330 | 23 | LH KS (Bank 2) Circuit Malfunction |
| P0335 | 4 | CKP Sensor 1 Circuit Malfunction |
| P0336 | 4 | CKP Sensor 1 Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0441 | 92 | EVAP Improper Purge Flow |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0730, P0740, P0753, P0758 | 70 | A/T |
| P1106 | 13 | Barometric Pressure Sensor Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1201 | 71 | Cylinder 1 Misfire |
| P1202 | 72 | Cylinder 2 Misfire |
| P1203 | 73 | Cylinder 3 Misfire |
| P1204 | 74 | Cylinder 4 Misfire |
| P1205 | 75 | Cylinder 5 Misfire |
| P1206 | 76 | Cylinder 6 Misfire |
| P1300 | — | Random Misfire |
| P1301 | 71 | Cylinder 1 Misfire |
| P1302 | 72 | Cylinder 2 Misfire |
| P1303 | 73 | Cylinder 3 Misfire |
| P1304 | 74 | Cylinder 4 Misfire |
| P1305 | 75 | Cylinder 5 Misfire |
| P1306 | 76 | Cylinder 6 Misfire |
| P1316 | 79 | Spark Plug Voltage Detection Circuit Malfunction, LH Bank (Bank 2) |
| P1317 | 79 | Spark Plug Voltage Detection Circuit Malfunction, RH Bank (Bank 1) |
| P1318 | 79 | Spark Plug Voltage Detection Module Reset Circuit Malfunction, LH Bank (Bank 2) |
| P1319 | 79 | Spark Plug Voltage Detection Module Reset Circuit Malfunction, RH Bank (Bank 2) |
| P1336 | 54 | CKP Sensor 2 Intermittent Interruption |
| P1337 | 54 | CKP Sensor 2 No Signal |
| P1381 | 9 | CYP Sensor 1 Intermittent Interruption |
| P1382 | 9 | CYP Sensor 1 No Signal |
| P1386 | 59 | CYP Sensor 2 Intermittent Interruption (3.2TL) |
| P1387 | 59 | CYP Sensor 2 No Signal (3.2TL) |
| P1456 | 90 | Evaporative Emission Control System Leak Detected (Fuel Tank System) |
| P1457 | 90 | Evaporative Emission Control System Leak Detected (Control Canister System) |
| P1459 | 92 | EVAP Purge Flow Switch Malfunction |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1607 | — | PCM Internal Circuit Failure |
| P1690 | 35 | Traction Control System STB Line Failure |
| P1696 | 36 | Traction Control Fuel Cut Signal Low Input |
| P1697 | 36 | Traction Control Fuel Cut Signal High Input |
| P1705, P1706, P1753, P1758, P1768, P1791 | 70 | A/T |

## 97–01 NSX

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10 | IAT Range/Performance Problem |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 2 | Rear Primary HO2S Circuit Low Voltage (Bank Sensor 1) |
| P0132 | 2 | Rear Primary HO2S Circuit High Voltage (Bank 1 Sensor 1) |
| P0133 | 62 | Rear Primary HO2S Slow Response (Bank 1 Sensor 1) |
| P0135 | 42 | Rear Primary HO2S Heater Circuit Malfunction (Bank 1 Sensor 1) |
| P0137 | 64 | Rear Secondary HO2S Circuit Low Voltage (Bank 1 Sensor 2) |
| P0138 | 64 | Rear Secondary HO2S Circuit High Voltage (Bank 1 Sensor 2) |
| P0139 | 64 | Rear Secondary HO2S Slow Response (Bank 1 Sensor 2) |
| P0141 | 66 | Rear Secondary HO2S Heater Circuit Malfunction (Bank 1 Sensor 2) |
| P0151 | 1 | Front Primary HO2S Circuit Low Voltage (Bank 2 Sensor 1) |
| P0152 | 1 | Front Primary HO2S Circuit High Voltage (Bank 2 Sensor 1) |
| P0153 | 61 | Front Primary HO2S Slow Response (Bank 2 Sensor 1) |
| P0155 | 41 | Front Primary HO2S Heater Circuit Malfunction (Bank 2 Sensor 1) |
| P0157 | 63 | Front Secondary HO2S Circuit Low Voltage (Bank 2 Sensor 2) |
| P0158 | 63 | Front Secondary HO2S Circuit High Voltage (Bank 2 Sensor 2) |
| P0159 | 63 | Front Secondary HO2S Circuit Slow Response (Bank 2 Sensor 2) |
| P0161 | 65 | Front Secondary HO2S Heater Circuit Malfunction (Bank 2 Sensor 2) |
| P0171 | 46 | System Too Lean, Rear Bank (Bank 1) |
| P0172 | 46 | System Too Rich, Rear Bank (Bank 1) |
| P0174 | 45 | System Too Lean, Front Bank (Bank 2) |
| P0175 | 45 | System Too Rich, Front Bank (Bank 2) |
| P0300 | 71, 72, 73, 74, 75, 76 | Random Misfire |
| P0325 | 53 | Rear KS Circuit Malfunction |
| P0330 | 23 | Front KS Circuit Malfunction |
| P0335 | 4 | CKP Sensor A Circuit Malfunction |
| P0336 | 4 | CKP Sensor A Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 68 | Rear Catalyst System Efficiency Below Threshold |
| P0430 | 67 | Front Catalyst System Efficiency Below Threshold |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Low Input |
| P0505 | 14 | IAC System Malfunction |
| P0700, P0715, P0720, P0725, P0730, P0740, P0753 & P0758 | 70 | A/T |
| P1106 | 13 | BARO Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1201 | 71 | Cylinder 1 Misfire |
| P1202 | 72 | Cylinder 2 Misfire |
| P1203 | 73 | Cylinder 3 Misfire |
| P1204 | 74 | Cylinder 4 Misfire |
| P1205 | 75 | Cylinder 5 Misfire |
| P1206 | 76 | Cylinder 6 Misfire |
| P1241 | 40 | Throttle Vent Control Motor Circuit 1 Malfunction |
| P1242 | 40 | Throttle Vent Control Motor Circuit 2 Malfunction |
| P1243 | 40 | Throttle Position Insufficient |
| P1244 | 40 | Closed Throttle Position Insufficient |
| P1246 | 37 | Accelerator Position Sensor 1 Circuit Malfunction |
| P1247 | 37 | Accelerator Position Sensor 2 Circuit Malfunction |
| P1248 | 37 | Accelerator Position Sensors 1 & 2 Improper Correlation |
| P1259 | 52 | VTEC System Malfunction, Rear Bank (Bank 1) |
| P1279 | 22 | VTEC System Malfunction, Front Bank (Bank 2) |
| P1300 | 71 thru 76 | Random Misfire |
| P1301 | 71 | Cylinder 1 Misfire |
| P1302 | 72 | Cylinder 2 Misfire |
| P1303 | 73 | Cylinder 3 Misfire |
| P1304 | 74 | Cylinder 4 Misfire |
| P1305 | 75 | Cylinder 5 Misfire |
| P1306 | 76 | Cylinder 6 Misfire |
| P1316 | 79 | Spark Plug Voltage Detection Circuit Malfunction, Front Bank |
| P1317 | 79 | Spark Plug Voltage Detection Circuit Malfunction, Rear Bank |
| P1318 | 79 | Spark Plug Voltage Detection Module Reset Circuit Malfunction, Front Bank |
| P1319 | 79 | Spark Plug Voltage Detection Module Reset Circuit Malfunction, Rear Bank |
| P1336 | 54 | CKP Sensor B Range/Performance |
| P1337 | 54 | CKP Sensor B Low Input |
| P1381 | 9 | CYP Sensor A Intermittent Interruption |
| P1382 | 9 | CYP Sensor A No Signal |
| P1386 | 59 | CYP Sensor B Intermittent Interruption |
| P1387 | 59 | CYP Sensor B No Signal |
| P1456 | 90 | Evaporative Emission Control System Leak Detected (Fuel Tank System) |
| P1457 | 90 | Evaporative Emission Control System Leak Detected (Control Canister System) |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1607 | — | ECM Internal Circuit Failure A |
| P1608 | — | ECM Internal Circuit Failure B |
| P1671 | 31 | A/T FI Data Line No Signal |
| P1672 | 31 | A/T FI Data Line Failure |
| P1676 | 35 | Traction Control System FI Data Line No Signal |
| P1677 | 35 | Traction Control System FI Data Line Failure |
| P1705, P1706, P1753, P1758, P1768, P1788, P1790, P1791, P1792, P1793 & P1795 | 70 | A/T |

## 99–01 3.2TL & 3.5L MDX

| OBD2 | MIL |  |
| --- | --- | --- |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0112 | 10 | IAT Circuit Low Input |
| P0113 | 10 | IAT Circuit High Input |
| P0116 | 86 | ECT Circuit Range/Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Circuit Low Input |
| P0123 | 7 | TP Circuit High Input |
| P0131 | 1 | Sensor 1 Primary HO2S Circuit Low Voltage |
| P0132 | 1 | Sensor 1 Primary HO2S Circuit High Voltage |
| P0133 | 61 | Primary HO2S Slow Response |
| P0135 | 41 | Primary HO2S Heater Circuit Malfunction |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage |
| P0138 | 63 | Secondary HO2S Circuit High Voltage |
| P0139 | 63 | Secondary HO2S Slow Response |
| P0141 | 65 | Secondary HO2S Heater Circuit Malfunction |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0300 | — | Random Misfire |
| P0301 | 71 | Cylinder 1 Misfire |
| P0302 | 72 | Cylinder 2 Misfire |
| P0303 | 73 | Cylinder 3 Misfire |
| P0304 | 74 | Cylinder 4 Misfire |
| P0305 | 75 | Cylinder 5 Misfire |
| P0306 | 76 | Cylinder 6 Misfire |
| P0325 | 23 | KS Circuit Malfunction |
| P0335 | 4 | CKP Sensor Circuit Malfunction |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0451 | 91 | Fuel Tank Pressure Sensor Circuit/Range Performance |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0505 | 14 | IAC System Malfunction |
| P0715, P0720, P0730, P0740, P0753, P0758 & P0763 | 70 | A/T System Malfunction |
| P1106 | 13 | BARO Sensor Circuit Range/Performance Malfunction |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | ELD Circuit Low Input |
| P1298 | 20 | ELD Circuit High Input |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC 1 Sensor No Signal |
| P1366 | 58 | TDC 2 Sensor Intermittent |
| P1367 | 58 | TDC 2 Sensor No Signal |
| P1456 | 90 | EVAP System Leak Detected (Fuel Tank System) |
| P1457 | 90 | EVAP Control System Leak Detected (Control Canister System) |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1519 | 14 | IAC Valve Circuit Failure |
| P1607 | — | ECM/PCM Internal Circuit Failure |
| P1676 | 35 | FPTDR Signal Line Failure |
| P1678 | 35 | FPTDR Signal Line Failure |
| P1656 P1705, P1706, P1709, P1738, P1739, P1753, P1768 & P1773 | 70 | A/T System Malfunction |