### Alfa Romeo: Diagnostic Trouble Codes

##### Spider (Motronic ML4.1)

To access DTCs, with engine running, depress auto diagnostic push button located under rear seat. DTCs of four digits are indicated by the Check Engine lamp in a series of flashes and pauses.

Motronic ML4.1 auto diagnostic push button location. Spider models

##### 164 (Motronic ML4.1)

To access DTCs, turn ignition On, depress auto diagnostic push button (T4), located on the lower LH side of the instrument panel, for approximately 2 1/2 to five seconds. DTCs of four digits are indicated by the Check Engine lamp in a series of flashes and pauses.

Motronic ML4.1 auto diagnostic push button location. 164 models

### Diagnostic Trouble Code Interpretation

The Check Engine lamp will display DTCs by flashing frequencies and 2 1/2 second pauses. As an example, one flash, followed by a 2 1/2 second pause, followed by two flashes and another 2 1/2 second pause, followed by four flashes and a 2 1/2 second pause, followed by three flashes and another 2 1/2 second pause, indicates a DTC 1243. The DTC will be repeated until the auto diagnostic push button is depressed to proceed to the next DTC.

### Clearing Diagnostic Trouble Codes

After performing all necessary repairs, the DTCs should be cleared from the ECU memory by depressing the auto diagnostic push button for more than 10 seconds, or by disconnecting and isolating the battery ground cable for approximately 10 seconds.

## Trouble Codes

| 1211 | Battery Voltage Improper |
| --- | --- |
| 1212 | Idle Throttle Switch Or Circuit |
| 1213 | Full Throttle Switch Or Circuit |
| 1214 | Engine Temperature Sensor |
| 1221 | AFM Or Circuit |
| 1222 | Idle Adjustment Actuator Or Circuit |
| 1223 | HO2S Regulator Inner Adjuster Reaches Minimum Or Maximum Value Of Correction |
| 1224 | HO2S |
| 1225 | Air Temperature Sensor |
| 1243 | Fuel Pump Relay Or Circuit |
| 1244 | Purge Control Solenoid Or Circuit |
| 1245 | Output Stage Of ECU For Variable Valve Timing |
| 1251 | ECU Faulty |
| 1265 | Output Stage Of ECU For Check Engine Lamp |
| 4444 | No Recognized Malfunction |

96 & newer vehicles are OBD2 systems 
and REQUIRE A SCANNER, unless otherwise stated.