## BMW: Accessing Trouble Codes

This information applies to 1989-1990 3, 5, and 7 series BMW models.

The following procedure will enable you to retrieve "basic" or "flash" trouble codes only. Other codes may also be stored, but they can only be retrieved using an appropriate Scan Tool.

3 Series

Turn the ignition key to the "on" position, but do not start the engine. Wait 3 seconds for the code(s) to flash. The check engine light will flash the first code, pause for 3 seconds, then flash the next code. For example, if codes 2 and 4 are stored, the check engine light will flash twice, pause for three seconds, then flash four times. When all codes have been displayed, the check engine light will remain ON.

Code Definitions

| Code 1 | Airflow Meter |
| --- | --- |
| Code 2 | Oxygen Sensor |
| Code 3 | Coolat Temperature Sensor |
| Code 4 | Throttle Position Sensor |

### Less Diagnostic Tool

### 90-later 3 series models

- Turn ignition On.

- Depress accelerator pedal five times to WOT within five seconds.

- DTCs will display as flashes on ‘‘Check Engine’’ lamp.

- On models equipped with V-12 engine, depress accelerator pedal six times to WOT to display cylinder 7–12 DTCs.

5 and 7 Series

Turn the ignition key to the "on" position, engine off. Depress the accelerator pedal completely to the floor five times. The check engine light will begin flashing the trouble codes. These models use 4 digit codes. The codes are displayed as follows:

First digit (always "1"), pause, 2nd digit, pause, 3rd digit, pause, 4th digit. The light will now pause again and then will begin flashing out the next stored code, or it will repeat the first code if no other codes are stored.

| 1000 | End of diagnosis |
| --- | --- |
| 1211 | Electronic Control Unit (ECU) |
| 1215 | Airflow Sensor |
| 1221 | Oxygen Sensor |
| 1222 | Oxygen Sensor regulation |
| 1223 | Coolant Temperature Sensor |
| 1224 | Air Temperature Sensor |
| 1231 | Battery Voltage out of range |
| 1232 | Idle Switch |
| 1251 | Fuel Injectors (final stage 1) |
| 1252 | Fuel Injectors (final stage 2) |
| 1261 | Fuel Pump Relay |
| 1262 | Idle Speed Controller |
| 1263 | Tank Vent |
| 1264 | Oxygen Sensor heater relay |
| 1444 | No faults in memory |

### Using Scan Tool

Connect scan tool to diagnostic link connector located at right rear of engine compartment. Follow tool manufacturer’s instructions to obtain DTCs.

## Reading the Trouble Codes

### Using Check Engine Lamp

‘‘Check Engine’’ lamp will stay on five seconds, flash once, then begin flashing fault codes with 2.5 seconds between numbers. When DTC has been displayed, lamp remains on. Repeat access procedure to display next DTC. If first DTC is 1444, 2444 or 4444, no DTCs are stored. DTC 1000 or 2000 followed by one or two flashes, a long paused and then lamp remaining on indicates no more DTCs are stored. Refer to ‘‘Diagnostic Trouble Code Identification’’ for DTC interpretation.

## Clearing Trouble Codes

To clear trouble codes, momentarily disconnect multi-pin connector from ECM, disconnect battery, start engine 60 times after repair without repeat failure, or follow diagnostic tool manufacturer’s instructions.

NOTE: IF YOUR CAR IS EQUIPPED WITH A FACTORY ANTI-THEFT RADIO, MAKE SURE YOU HAVE THE RADIO ACTIVATION CODE BEFORE YOU DISCONNECT THE BATTERY CABLE!

Codes using Check Engine Light

Codes using Scan Tool

# Check Engine Lamp

| M1.2, M1.3 & M1.7{1} |  | M3.1 | M3.3 | M3.3.1 |  |
| --- | --- | --- | --- | --- | --- |
| 1st ECU | 2nd ECU |  |  |  |  |
| 1211 | 2211 | 1211 | 1211 | 1211 | ECM |
| — | — | — | 1212 | — | O2S No. 2 |
| — | — | — | 1213 | — | O2S No. 2 Control Range Limit |
| 1215 | 2215 | 1215 | 1215 | 1215 | MAF Sensor |
| 1216 | — | 1216 | 1216 | 1216 | TP Sensor |
| 1221 | 2221 | 1221 | 1221 | 1221 | O2S No. 1 |
| 1222 | 2222 | 1222 | 1222 | 1222 | O2S No. 1 Control Range Limit |
| 1223 | 2223 | 1223 | 1223 | 1223 | ECT |
| 1224 | 2224 | 1224 | 1224 | 1224 | IAT |
| 1231 | 2231 | 1231 | 1231 | 1231 | Battery Voltage |
| 1232 | — | — | — | — | TP Sensor (Idle) |
| 1233 | — | — | — | — | TP Sensor (WOT) |
| 1251 | 2251 | — | — | — | Fuel Injection Valves (Group 1) |
| 1252 | 2252 | — | — | — | Fuel Injection Valves (Group 2) |
| — | — | 1251–1256 | 1251–1256 | 1251–1256 | Fuel Injection Valve Cylinder 1–6 |
| — | — | — | 1257–1258 | — | Fuel Injection Valve Cylinder 7–8 |
| 1261 | 22261 | 1261 | 1261 | 1261 | Fuel Pump Relay |
| 1262 | — | 1262 | 1262 | 1262 | ISC Valve |
| 1263 | 2263 | 1263 | 1263 | 1263 | Purge Valve |
| 1264 | 2264 | 1264 | 1264 | 1264 | O2S Heating Relay |
| — | — | — | 1267 | 1267 | Air Pump |
| — | — | 1283 | 1283 | 1283 | Undefined Fuel Injection Valve |
| 1444 | 2444 | 1444 | 1444 | 1444 | No Failure |

{1}—On DME M1.7, DTCs beginning with ‘‘1’’ affect cylinders 1–6. DTCs beginning with ‘‘2’’ affect cylinders 7–12.

# Using Scan Tool

| M1.1 & M1.3 | M1.2 | M1.7 | M3.1 | M3.3.1 |  |
| --- | --- | --- | --- | --- | --- |
| — | — | 00 | 00 | 00 | Undefined Fault |
| 01 | 01 | — | — | — | ECM |
| — | — | 01 | 01 | 01 | Fuel Pump Relay |
| — | — | — | 02 | 02 | Idle Actuator |
| 03 | 03 | — | — | — | Fuel Pump Relay |
| — | — | 03 | — | — | Fuel Injectors 2, 4 & 6 or 8, 10 & 12 |
| — | — | — | 03 | 03 | Fuel Injector 1 |
| 04 | — | — | — | — | Idle Speed Actuator, Open |
| — | — | — | 04 | 04 | Fuel Injector 3 |
| 05 | 05 | — | — | — | EVAP Valve |
| — | — | — | 05 | 05 | Fuel Injector 2 |
| — | — | — | 06 | 06 | Injector Output Stage |
| 07 | 07 | — | — | — | MAF |
| — | — | 08 | 08 | 08 | Check Engine Lamp |
| 10 | 10 | — | — | — | O2 Control |
| — | — | — | 12 | 12 | TP Sensor |
| 15 | 15 | — | — | — | Check Engine Lamp |
| 16 | — | — | — | — | Fuel Injectors 1, 3 & 5 |
| — | 16 | — | — | — | Fuel Injectors 1, 3 & 5 or 7, 9 & 11 |
| — | — | 16 | — | — | Ignition |
| — | — | — | 16 | 16 | CMP |
| 17 | — | — | — | — | Fuel Injectors 2, 4 & 6 |
| — | 17 | — | — | — | Fuel Injectors 2, 4 & 6 or 8, 10 & 12 |
| — | — | 18 | 18 | 18 | Final Stage Pin 18 |
| — | — | — | 19 | 19 | Output Stage |
| 22 | — | — | — | — | Idle Speed Actuator, Closed |
| 23 | 23 | — | — | — | O2S Heating Replay |
| — | — | — | 23 | 23 | Ignition Cylinder No. 2 |
| — | — | — | 24 | 24 | Ignition Cylinder No. 3 |
| — | — | — | 25 | 25 | Ignition Cylinder No. 1 |
| — | — | — | 26 | 26 | Power Supply |
| 28 | 28 | — | — | — | O2S |
| 29 | — | — | — | — | VSS |
| — | — | — | 29 | 29 | Idle Actuator |
| — | — | — | 31 | 31 | Fuel Injector 5 |
| — | — | 32 | — | — | Fuel Injectors 1, 3 & 5 or 7, 9 & 11 |
| — | — | — | 32 | 32 | Fuel Injector 6 |
| 33 | — | — | — | — | Solenoid Valve Kickdown Prevent |
| — | — | — | 33 | 33 | Fuel Injector 4 |
| — | — | — | — | 34 | Output Stage |
| — | — | 36 | 36 | 36 | EVAP Valve |
| 37 | 37 | — | — | — | Voltage Supply |
| — | — | 37 | 37 | 37 | O2S Heater Relay |
| 38 | — | — | — | — | Automatic Stability Control/Anti-Theft System |
| 40 | — | — | — | — | A/C Compressor |
| — | — | 41 | 41 | 41 | MAF Sensor |
| — | 43 | — | — | — | Idle CO Sensor |
| 44 | 44 | — | — | — | IAT Sensor |
| 45 | 45 | — | — | — | ECT Sensor |
| — | — | — | — | 46 | Output Stage |
| — | — | 48 | 48 | 48 | A/C Compressor Shut off |
| 50 | — | — | — | — | Engine Drag Torque Control |
| — | — | — | 50 | 50 | Ignition Cylinder No. 4 |
| 51 | 51 | — | — | — | Ignition Angle |
| — | — | — | 51 | 51 | Ignition Cylinder No. 6 |
| 52 | — | — | — | — | Idle Switch |
| — | — | — | 52 | 52 | Ignition Cylinder No. 5 |
| 53 | — | — | — | — | Full Load Switch |
| 54 | 54 | — | — | — | Torque Converter Clutch Unit |
| — | — | 54 | 54 | 54 | Voltage Supply |
| — | — | — | 55 | 55 | Ignition Output Stage |
| — | — | — | 62 | — | Electronic Throttle Control Signal |
| — | — | 63 | — | — | Converter Lockup Clutch |
| — | — | 64 | — | — | Ignition Timing Tap |
| — | — | — | 64 | 64 | Ignition Timing Intervention During Downshifts |
| — | — | — | 67 | 67 | CKP Sensor |
| — | — | 70 | 70 | 70 | O2S |
| — | — | 73 | 73 | 73 | VSS |
| — | — | 77 | 77 | 77 | IAT Sensor |
| — | — | 78 | 78 | 78 | ECT Sensor |
| — | — | — | — | 81 | Anti-Theft System |
| — | — | 82 | 82 | — | Engine Drag Torque Control |
| — | — | 83 | 83 | — | Automatic Stability Control |
| — | — | — | 85 | 85 | A/C Compressor |
| 100 | 100 | 100 | 100 | 100 | Output Stage |
| 101 | 101 | — | — | — | Engine Operations Not Possible |
| — | — | 200 | 200 | 200 | ECM |
| — | — | 201 | — | — | O2 Control |
| — | — | — | 201 | — | Emission Control |
| — | — | — | — | 201 | DTC Memory |
| — | — | — | 202 | — | DTC Memory |
| — | — | — | — | 202 | Emission Control |
| — | — | — | 203 |  | Ignition Circuit Monitoring |
| — | — | — | 204 |  | Stall Protection |

Cadillac Brougham BCM and PCM Codes and Diagnostic Procedures

Diagnostics Basic Operation

| Step | Action |
| --- | --- |
| 1 | Enter self diagnostics by simultaneously pressing and holding the "OFF" and "WARMER" buttons on the Electronic Climate Control panel until the automatic segment check begins. |
| 2 | Malfunction codes are displayed in the following order: ..- ECC current codes ( parameter number -00 ) 1..- ECC history codes ( parameter number -00 ) |
| 3 | To select a specific parameter, press "HI" to increment and "LO" to decrement the parameter number. |
| 4 | After the status light display begins, ECC error codes are displayed. PCM codes may be displayed by selection parameter -01 and then pressing the "OUTSIDE TEMP" button. |
| 5 | Exit diagnostics by pressing the "AUTO" button |
| 6 | Pressing "OUTSIDE TEMP" switches between parameter number and parameter data |
| 7 | To over-ride a program number, while in program number display, parameter -11, pressing "WARMER" or "COLDER" will increase or decrease the program number. |

Parameter Number -00

ECC Diagnostic Codes

| Code | Malfunction |
| --- | --- |
| 00 | No ECC system fault |
| 10 | Out-side ambient temp sensor malfunction |
| 12 | A/C evaporator inlet temp sensor malfunction |
| 13 | In-car tem sensor malfunction |
| 19 | Blower motor failure |
| 32 | PCM-ECC data problem |
| 40 | Air mix valve problem |
| 48 | Very low refrigerant pressure level |
| 49 | High engine temp |
| 50 | EVO system fault |

Parameter Number -01

PCM Diagnostic Codes

| Code | Malfunction |
| --- | --- |
| 00 | No PCM faults |
| 12 | No engine RPM References pulses |
| 13 | Oxygen sensor circuit fault |
| 14 | Shorted coolant sensor circuit fault |
| 15 | Open coolant sensor circuit fault |
| 21 | Open throttle position sensor circuit fault |
| 22 | Shorted throttle position sensor circuit fault |
| 23 | Open MAT sensor circuit fault |
| 24 | VSS circuit fault |
| 25 | Shorted MAT sensor circuit fault |
| 32 | EGR system fault |
| 33 | Open MAP sensor circuit fault |
| 34 | Shorted MAP sensor circuit fault |
| 42 | EST/bypass circuit fault |
| 43 | ESC failure |
| 44 | Lean O2 signal |
| 45 | Rich O2 signal |
| 51 | PROM error |
| 52 | Backup fuel cal-pack missing |
| 54 | Fuel pump relay malfunction |
| 55 | A/D Converter malfunction |

Parameters

| Number | Description | Range | Units |
| --- | --- | --- | --- |
| -00 | ECC fault codes | 00-150 | Code |
| -01 | PCM fault codes | 00-55 | Code |
| -02 | Program number | 0-255 | Counts |
| -03 | IAC motor position | 0-255 | Counts |
| -04 | MAP | 0-255 | KPA |
| -06 | TPS | 0-255 | Counts |
| -07 | Oxygen sensor |  | Volts |
| -08 | MAT |  | C |
| -10 | Ignition voltage | 9.0-16.0 | Volts |
| -11 | Program number | 0-100 | % |
| -12 | Vehicle speed | 0-199 | MPH |
| -19 | Actual blower voltage | -3.3-18 | Volts |
| -20 | Commanded blower voltage | -3.3-18 | Volts |
| -21 | Engine coolant temp | -40-151 | C |
| -22 | Commanded blend door position | 0-255 | Counts |
| -23 | Actual blend door position | 0-255 | Counts |
| -24 | Air delivery mode | 0-6 | Code |
| -25 | In-car temp | -40-102 | C |
| -26 | Outside air temp | -40-93 | C |
| -28 | Evaporator inlet temp | -40-93 | C |
| -30 | Ignition cycle counter | 0-99 | Counts |
| -31 | PROM ID | 0-9999 | Code |

Air Delivery Mode

| Code | Mode |
| --- | --- |
| 0 | Max A/C |
| 1 | A/C |
| 2 | Intermediate |
| 3 | Heater |
| 4 | Defrost |
| 5 | Blower delay |
| 6 | Off |