# Chrysler Code Retrieval

### Accessing Trouble Codes 
96 & later are OBD2 & requires an OBD2 scanner.

Code retrieval can be accessed by the ignition key or by a diagnostic readout box(DRB).To obtain the codes using the key method,set the park brake,and put the vehicle in Park/neutral. 
Some 95 and later OBDII vehicles, you may be able to retrieve OBDII codes with this procedure, if equipped with digital dash.

- Turn the ignition key ON,OFF,ON,OFF,and finally ON again(ENGINE NOT RUNNING)

- The codes will start to flash on the POWER LOSS or CHECK ENGINE light.

- FLASH,pause,FLASH,FLASH=code 12

### Clearing Trouble Codes

Diagnostic trouble codes may be cleared by disconnecting the battery ground cable for at least 20 seconds.
OBD2 vehicles require a scanner to clear the trouble codes.

## Diagnostic Codes

## Chrysler

| 84–88 | 2.0L |
| --- | --- |
| 84-95 | TBI |
| 85–95 | 2.2L, 2.5L/L4 Turbo & 2.5L/L4 Flexible Fuel Engines |
| 87-89 | Lt Truck |
| 87-95 | MPFI Lt Duty Trucks & Vans |
| 88 | 1.5L Colt Code Retreival |
| 88–95 | 3.0L (Exc. Monaco, Premier & Stealth), 3.3L, 3.5L & 3.8L Engines |
| 89–90 | 3.0L Monaco & Eagle Premier |
| 89–95 | 1.5L, 1.6L, 1.8L, 2.4L (VIN G) |
| 89-95 | Jeep |
| 89–94 | 2.0L |
| 91–92 | 3.0L Monaco & Eagle Premier |
| 89-93 | 1.5L Colt Code Retrieval |
| 91–95 | 3.0L Stealth |
| 95 | 2.4L (VIN X) |
| 95 | 2.5L w/SFI |
| 95–96 | 1.5L, 1.8L, 2.4L (VIN B) & 3.0L W/OBD II |
| 95–98 | 2.0L Turbo |
| 95–98 | 2.0L Non-Turbo, 2.4L (VIN X), 2.5L/V6, 2.7L, 3.2L, 3.3L & 3.5L w/OBD II |
| 95–98 | Lt Duty Truck, Van & Jeep |
| 99–01 | Exc. Avenger & Sebring Coupe |
| 00 | Avenger & Sebring Coupe |
| 99–01 | Light Duty Truck, Van & Jeep |
| 96-01 | Chrysler generic. |
|  | Jeep |
|  | Ram Raider & Ram 50 |

## 96–01

| P0112 | Intake Air Temperature Sensor Voltage Low |
| --- | --- |
| P0113 | Intake Air Temperature Sensor Voltage High |
| P0117 | ECT Sensor Voltage Too Low |
| P0118 | ECT Sensor Voltage Too High |
| P0121 | Accelerator Pedal Position Sensor Signal Volts Do Not Agree w/Idle Validation Signal |
| P0122 | Accelerator Pedal Position Sensor Signal Voltage Too Low |
| P0123 | Accelerator Pedal Position Sensor Signal Voltage Too High |
| P0125 | Engine Is Cold Too Long |
| P0168 | Decreased Engine Performance Due To High Injection Pump Fuel Temperature |
| P0177 | Water In Fuel Sensor Voltage Too Low |
| P0181 | Fuel Injection Pump Failure |
| P0215 | Fuel Injection Pump Control Circuit |
| P0216 | Fuel Injection Pump Timing Failure |
| P0217 | Decreased Engine Performance Due To Engine Overheating Condition |
| P0219 | Camshaft Position Sensor Overspeed Signal |
| P0222 | Idle Validation Signals Both Low |
| P0223 | Idle Validation Signals Both High (Above 5 Volts) |
| P0230 | Transfer pump Circuit Out Of Range |
| P0232 | Fuel Shut-Off Voltage Too High |
| P0234 | Turbo Boost Limit Exceeded |
| P0236 | MAP Sensor Too High Too Long |
| P0237 | MAP Sensor Voltage Too Low |
| P0238 | MAP Sensor Voltage Too High |
| P0251 | Fuel Injection Pump Mechanical Failure Fuel Valve Feedback Circuit |
| P0253 | Fuel Injection Pump Fuel Valve Open Circuit |
| P0254 | Fuel Injection Pump Fuel Valve Current Too High |
| P0300 | Multiple Cylinder Misfire |
| P0301 | Misfire Detected, Cylinder No. 1 |
| P0302 | Misfire Detected, Cylinder No. 2 |
| P0303 | Misfire Detected, Cylinder No. 3 |
| P0304 | Misfire Detected, Cylinder No. 4 |
| P0305 | Misfire Detected, Cylinder No. 5 |
| P0306 | Misfire Detected, Cylinder No. 9 |
| P0310 | Misfire Detected, Cylinder No. 6 |
| P0320 | No RPM Signal To PCM |
| P0336 | Crankshaft Position Sensor Signal |
| P0341 | Camshaft Position Sensor Signal |
| P0370 | Fuel Injection Pump Speed/Position Sensor Signal Lost |
| P0380 | Intake Air Heater Relay No. 1 Control Circuit |
| P0381 | Wait To Start Lamp Inoperative |
| P0382 | Intake Air Heater Relay No. 2 Control Circuit |
| P0387 | Crankshaft Position Sensor Supply Voltage Too Low |
| P0388 | Crankshaft Position Sensor Supply Voltage Too High |
| P0400 | Exhaust Gas Recirculation (EGR) Flow Malfunction |
| P0460 | Fuel Level Unit No Change Over Miles |
| P0462 | Fuel Level Sending unit Volts Too Low |
| P0463 | Fuel Level Sending unit Volts Too High |
| P0500 | No Vehicle Speed Sensor Signal |
| P0522 | Oil Pressure Voltage Too Low |
| P0523 | Oil Pressure Voltage Too High |
| P0524 | Oil Pressure Too Low |
| P0545 | A/C Clutch Relay Circuit |
| P0562 | Charging System Voltage Too Low |
| P0563 | Charging System Voltage Too Low |
| P0601 | PCM Internal Controller Failure |
| P0622 | Alternator Field Improper Switching |
| P0712 | Trans Temp Sensor Voltage Too Low |
| P0713 | Trans Temp Sensor Voltage Too High |
| P0720 | Low Output Speed Sensor RPM Above 15 M.P.H. |
| P0743 | TCC Solenoid/Trans Relay Circuits |
| P0748 | Governor Pressure Sol/Control Trans Relay Circuits |
| P0751 | OD Switch Pressed (Lo) For More Than 5 Minutes |
| P0753 | Trans 3–4 Shift Sol/Trans Relay Circuits |
| P1283 | Idle Select Signal Invalid |
| P1284 | Fuel Injection Pump Battery Voltage Out Of Range |
| P1285 | Fuel Injection Pump Controller Always On |
| P1286 | Accelerator Pedal Position Sensor Supply Voltage Too High |
| P1287 | Fuel Injection Pump Controller Supply Voltage Low |
| P1291 | No Temperature Rise Seen From Intake Air Heaters |
| P1295 | Accelerator Pedal Position Sensor Supply Voltage Too Low |
| P1388 | Auto Shutdown (ASD) Relay Control Circuit |
| P1389 | No Auto Shutdown (ASD) Relay Output Voltage At PCM |
| P1475 | Aux. 5 Volt Output Too High |
| P1488 | Aux. 5 Volt Output Too Low |
| P1492 | Battery Temperature Sensor Voltage Too High |
| P1493 | Battery Temperature Sensor Voltage Too Low |
| P1594 | Charging System Voltage Too High |
| P1595 | Speed Control Solenoid Circuits |
| P1597 | Speed Control Switch Always Low |
| P1682 | Charging System Voltage Too Low |
| P1683 | Speed Control Power Relay Or Speed Control 12 Volt Driver Circuit |
| P1688 | Internal Fuel Injection Pump Controller Failure |
| P1689 | No Communication Between ECM & Injection Pump Module |
| P1690 | Fuel injection pump CKP Sensor Does Not Agree With ECM CKP Sensor |
| P1691 | Fuel Injection Pump Controller Calibration Failure |
| P1693 | DTC Detected In ECM Or PCM |
| P1694 | No CCD Messages Received From ECM |
| P1698 | No CCD Messages Received From PCM |
| P1740 | TCC Or OD Solenoid Performance |
| P1756 | Governor Pressure Not Equal To Target At 15–20 PSI |
| P1757 | Governor Pressure Above 3 PSI When Request Is 0 PSI |
| P1762 | Governor Pressure Sensor Offset Improper Voltage |
| P1763 | Governor Pressure Sensor Voltage Too High |
| P1764 | Governor Pressure Sensor Voltage Too Low |
| P1765 | Trans 12 Volt Supply Relay Control Circuit |
| P1899 | PNP Switch Failure |

# 84–88 2.0L Engine

| DTC | Circuit Or Condition |
| --- | --- |
| 0 | Normal |
| 1 | Oxygen Sensor |
| 2 | Crank Angle Sensor |
| 3 | Air Flow Sensor |
| 4 | Atmospheric Pressure Sensor |
| 5 | Throttle Position Sensor |
| 6 | Motor Position Switch |
| 7 | Coolant Temperature Sensor |
| 8 | No. 1 Cylinder TDC Sensor |

## Throttle Body (Single Point) Fuel Injection

| 1984–88 1.6L & 2.6L |  |
| --- | --- |
| 1 | Oxygen Sensor |
| 2 | Ignition Signal |
| 3 | Air Flow Sensor |
| 4 | Pressure Sensor |
| 5 | Throttle Position Sensor |
| 6 | ISC Motor Position Switch |
| 7 | Coolant Temperature Sensor |
| 8 | Vehicle Speed Sensor{8} |
| 1984–95 2.2L & 2.5L/L4 |  |
| 11 | Fault In Distributor Circuit{4} |
|  | Engine Not Cranked Since Battery Was Disconnected{1} |
|  | No Distributor Reference During Engine Cranking{2} |
| 12 | Memory Standby Power Lost{3} |
| 13 | MAP Sensor Pneumatic Circuit{3} |
| 13 | No Or Little Change In MAP Sensor Signal{5} |
| 14 | MAP Sensor Electrical Circuit |
| 15 | Vehicle Speed/Distance Sensor |
| 16 | Loss Of Battery Voltage Sensor{6} |
| 17 | Engine Running Too Cool{7} |
| 21 | Oxygen Sensor Circuit |
| 22 | Coolant Temperature Sensor Circuit |
| 23 | Charge Temperature Sensor{4} |
|  | Throttle Body Temperature Sensor Circuit{7} |
| 24 | Throttle Position Sensor |
| 25 | AIS Motor Driven Circuit |
| 26 | Peak Injector Current Not Reached |
| 27 | Logic Module Fuel Circuit{8} |
|  | Fuel Injector Control Problem{2} |
| 31 | Purge Solenoid Circuit |
| 32 | Power Loss Lamp Circuit{4} |
|  | EGR Diagnosis{2} |
| 33 | A/C Cutout Relay Circuit |
| 34 | Speed Control Solenoid Driver Circuit |
| 35 | Fan Control Relay Circuit |
| 36 | Spare Driver Circuit{4} |
| 37 | Part Throttle Unlock Solenoid Circuit |
| 41 | Charging System Excess Or No Field Circuit |
| 42 | ASD Relay Driver |
| 43 | Spark Interface Circuit{6} |
| 44 | Battery Temperature Out Of Range{8} |
| 44 | Loss Of FJ2 To Logic Board{9} |
| 46 | On Battery Voltage Too High{7} |
| 47 | Battery Voltage Too Low{7} |
| 51 | Oxygen Feedback System (Lean) |
| 52 | Oxygen Feedback System (Rich) |
| 53 | Logic Module Failure{4} |
|  | Internal Engine Controller Problem{7} |
| 55 | End Of Message |
| 62 | EMR Mileage Unsuccessful Update{10} |
| 63 | Controller Failure EEPROM Write Denied{10} |

{1}—1986–88 models.
{2}—1989–95 models.
{3}—1984–90 models.
{4}—1984–85 models.
{5}—1991–92 models.
{6}—1984–89 models.
{7}—1986–95 models.
{8}—1984–88 models.
{9}—1989 models.
{10}—1988–95 models.

## 85–95 2.2L, 2.5L/L4 Turbo & 
2.5L/L4 Flexible Fuel Engines

| 11 | Distributor Circuit{1} |
| --- | --- |
|  | Engine Has Not Been Cranked Since Battery Was Disconnected{2} |
|  | No Distributor Reference Signal Detected During Engine Cranking{3} |
| 12 | Fault In Stand By Memory Circuit{4} |
|  | Battery Input To Controller Disconnected Within Last 50 Key-Ons{5} |
| 13 | Fault In MAP Sensor Pneumatic System{6} |
|  | Slow Change In Idle MAP Signal Or No Change In MAP From Start To Run{7} |
| 14 | Fault In MAP Sensor Electrical System |
| 15 | Fault In Speed Sensor Circuit |
| 16 | Loss Of Battery Voltage Sense{4} |
|  | Fault In Knock Sensor{8} |
| 17 | Knock Sensor Circuit{9} |
|  | Engine Running Too Cool To Long{10} |
| 21 | Problem In Oxygen Sensor Circuit |
| 22 | Fault In Coolant Temperature Sensor Circuit |
| 23 | Fault In Charge Temperature Sensor Circuit |
| 24 | Fault In Throttle Position Sensor Circuit |
| 25 | Fault In AIS Control Circuit |
| 26 | No. 1 Injector Circuit{1} |
|  | Injector Current Limit Not Achieved{2} |
| 27 | No. 2 Injector Circuit{1} |
|  | Injector Control Circuit{11} |
| 31 | Canister Purge Solenoid Circuit |
| 32 | Power Loss Lamp Circuit{4} |
|  | Open Or Short In EGR Solenoid Circuit{3} |
| 33 | Loss Of FJ2 Logic Board{1} |
|  | A/C Clutch Relay Circuit{11} |
| 34 | EGR Solenoid Circuit{12} |
|  | Speed Control Problem{13} |
| 35 | Fan Relay Circuit |
| 36 | Wastegate Solenoid Circuit{14} |
|  | VNT Solenoid No. 3{15} |
| 37 | Barometric Read Solenoid Circuit{4} |
|  | Open Or Short In Torque Converter Part Throttle Unlock Solenoid Circuit{3} |
| 41 | Fault In Charging System |
| 42 | Fault In ASD Relay Circuit{4} |
|  | No Z1 Voltage Sensed When ASD Relay Is Energized{3} |
| 43 | Interface Problem{1} |
|  | Ignition Coil Control Circuit{16} |
|  | Ignition Coil One & Two Primary Circuit{7} |
| 44 | Battery Out Of Range{1} |
|  | Loss Of FJ2 To Logic Board{2} |
| 45 | Fault In Overboost Shutoff Circuit |
| 46 | Battery Voltage Is Too High |
| 47 | Battery Voltage Is Too Low |
| 51 | Oxygen Feedback System Stays Lean |
| 52 | Oxygen Feedback System Stays Rich{1}{17} |
|  | Battery Temperature Circuit{18} |
| 53 | Internal Problem In Module |
| 54 | Fault In Synchronization Pickup Circuit{19} |
|  | No Camshaft Position Sensor Signal{8} |
| 55 | End Of Message |
| 61 | Open Or Short In BARO Read Circuit |
| 62 | EMR Mileage |
| 63 | Fault Code Error{20} |
|  | Controller Failure EEPROM Write Denied{21} |
| 64 | Open Or Short In VNT No 1 Solenoid Circuit{22} |
| 65 | Open Or Short In VNT No 2 Solenoid Circuit{22} |
| 67 | MAP Reading Above Boost Goal For An Extended Time Period{22} |

| {1}—1985 models. {2}—1986–89 models. {3}—1990–95 models. {4}—1985–89 models {5}—1990 models. {6}—1985–90 models. {7}—1991–95 models. {8}—1993–95 models. {9}—1985–86 models. {10}—1987–95 models. {11}—1986–95 models. | {12}—1985–87 models. {13}—1988–95 models. {14}—Except Turbo III & IV engines. {15}—Turbo III & IV engines. {16}—1986–90 models. {17}—1989–95 models. {18}—1986–88 models. {19}—1985–92 models. {20}—1989–91 models. {21}—1992–95 models. {22}—1990 Turbo IV engine. |
| --- | --- |

## Chrysler Light Trucks & Vans

| 1987–89 2.2L & 2.5L & 1988–89 3.9L & 5.2L |  |
| --- | --- |
| 11 | Distributor Pickup Signal |
| 12 | Battery Feed To ECM{1} |
|  | Fault Codes Erased{2} |
| 13 | MAP Sensor Vacuum |
| 14 | MAP Sensor Electrical |
| 15 | Vehicle Speed Sensor |
| 16 | Battery Voltage Sensing |
| 17 | Engine Cooling System |
| 21 | Oxygen Sensor |
| 22 | Engine Coolant Sensor |
| 23 | Throttle Body Temperature Sensor |
| 24 | Throttle Position Sensor |
| 25 | Automatic Idle Speed Motor |
| 26 | Fuel Injector Driver |
| 27 | Fuel Control |
| 31 | Canister Purge Solenoid |
| 32 | EGR System |
| 33 | A/C Cutout Relay |
| 34 | Speed Control Servo Circuit |
| 35 | Radiator Fan Relay{1} |
|  | Idle Contact Switch Circuit{2} |
| 36 | Air Switching Solenoid Circuit |
| 37 | Transmission Lock-Up Solenoid |
| 41 | Alternator Field Control |
| 42 | Auto Shutdown |
| 43 | Spark Control |
| 44 | ECM Logic Board Fused J2 Voltage |
| 45 | 4-Speed Overdrive Solenoid Circuit |
| 46 | Battery Voltage Sensing |
| 47 | Battery Voltage Sensing |
| 51 | Oxygen Feedback System |
| 52 | Oxygen Feedback System |
| 53 | Single Module Engine Controller |
| 55 | End Of Diagnostic Mode |
| 1990–95 |  |
| 11 | Ignition Reference Signal |
| 12 | Number Key-On Since Fault Stored Or Fault Erased |
| 13 | MAP Signal Circuit |
| 14 | Improper MAP Voltage |
| 15 | Vehicle Speed Signal |
| 16 | Battery Input Sense |
| 17 | Low Engine Temperature |
| 21 | Oxygen Sensor Signal |
| 22 | Coolant Voltage Low Or Oxygen Sensor Shorted High |
| 23 | Improper Throttle Body Temperature Voltage |
| 24 | Improper TP Sensor Voltage |
| 25 | AIS Motor Circuits |
| 26 | Injector 1 Or 2 High Circuit Resistance |
| 27 | Injector 1 Control Circuit |
| 31 | Purge Solenoid Circuit |
| 32 | EGR System Circuit |
| 33 | A/C Clutch Relay Circuit |
| 34 | S/C Servo Solenoids |
| 35 | Idle Switch Circuit |
| 36 | Air Switch Solenoid |
| 37 | PTU Solenoid Circuit |
| 41 | Alternator Field Circuit |
| 42 | Auto Shutdown Relay Circuit |
| 43 | Ignition Control Circuit |
| 44 | Voltage Sensing |
| 45 | Overdrive Solenoid |
| 46 | Battery Voltage High |
| 47 | Charging Output Low |
| 51 | Lean Fuel/Air Condition |
| 52 | Rich Fuel/Air Condition |
| 53 | Internal Self Test |
| 55 | End Of Diagnostic Mode |
| 62 | ECM EMR Miles Not Stored |
| 63 | ECM EEPROM Write Denied |

{1}—2.2L & 2.5L engines.
{2}—3.9L & 5.2L engines.

## MPFI Lt Duty Trucks & Vans

87–95 Chrysler Lt Duty Trucks & Vans, Exc Ram Raider & Ram 50

| 1987–88 |  |
| --- | --- |
| 11 | Distributor Reference Pickup Control |
| 12 | Battery Feed To ECM |
| 13 | MAP Sensor Vacuum |
| 14 | MAP Sensor Circuit |
| 15 | Vehicle Speed Sensor |
| 16 | Battery Voltage Sensing |
| 17 | Engine Cooling System |
| 21 | Oxygen Sensor |
| 22 | Engine Coolant Sensor |
| 23 | Charge Temperature Sensor |
| 24 | Throttle Position Sensor |
| 25 | Automatic Idle Speed Motor |
| 26 | Fuel Injector Driver |
| 27 | Fuel Control |
| 31 | Canister Purge Solenoid |
| 33 | A/C Cutout Relay |
| 32 | EGR System (1988) |
| 34 | Speed Control Servo Circuit |
| 35 | Radiator Fan Relay Circuit |
| 37 | Transmission Lockup Solenoid |
| 41 | Alternator Field Control |
| 42 | Auto Shutdown |
| 43 | Spark Control |
| 44 | ECM Logic Board Fused J2 Voltage |
| 45 | Battery Voltage Sensing (1987) |
| 46 | Battery Voltage Sensing (1988) |
| 47 | Battery Voltage Sensing |
| 51 | Oxygen Feedback System |
| 52 | Oxygen Feedback System |
| 53 | Engine Control Module |
| 54 | Distributor Sync. Pickup |
| 55 | End Of Diagnostic Mode |
| 1989–95 |  |
| 11 | Ignition Reference Signal |
| 12 | Number Of Key On Since Last Fault Or Since Faults Were Erased (1990) |
| 13 | MAP Pneumatic Circuit |
| 14 | Improper MAP Voltage |
| 15 | Vehicle Speed Sensor |
| 16 | Battery Input Sense |
| 17 | Low Engine Temperature |
| 21 | Oxygen Sensor Signal |
| 22 | Improper Coolant Voltage |
| 23 | Improper Charge Temperature Sensor Voltage |
| 24 | Improper TP Sensor Voltage |
| 25 | AIS Motor Circuit |
| 26 | Injector 1, 2 Or 3 Peak Current |
| 27 | Injector 1, 2 Or 3 Control Circuit |
| 31 | Purge Solenoid Circuit |
| 32 | EGR System Circuit |
| 33 | A/C Clutch Relay Circuit |
| 34 | Speed Control Servo Solenoids |
| 35 | Radiator Fan Circuit |
| 36 | Wastegate Solenoid |
| 37 | Torque Converter Unlock Solenoid Circuit |
| 41 | Charging System Circuit |
| 42 | Auto Shutdown Relay Circuit |
| 43 | Ignition Control Circuit |
| 44 | ECM Logic Board Fused J2 Voltage (1989) |
| 44 | Battery Temperature Voltage (1992–95) |
| 45 | Boost Limit Exceeded |
| 46 | Battery Voltage High |
| 47 | Charging Output Low |
| 51 | Lean Air/Fuel Condition |
| 52 | Rich Air/Fuel Condition |
| 53 | Internal Self Test |
| 54 | No Sync. Pickup Signal |
| 55 | End Of Diagnostic Mode |
| 61 | Barometric Read Solenoid |
| 62 | Controller Failure, EMR Miles Not Stored |
| 63 | Controller Failure, EEPROM Write Denied |

### 88 Dodge Colt 1.5L VIN P FI

Displaying, Reading and Clearing Trouble Codes

Trouble code indications are made by reading voltmeter deflections. To determine diagnostic code, proceed as follows:

- Turn ignition switch Off.

- Locate diagnostic connector (See below).

- Connect a suitable voltmeter between MPI-diagnosis terminal and ground.

- Turn ignition switch On and note that an immediate indication of ECU memory contents will begin and voltmeter will deflect accordingly.

- After recording the abnormal items, check and repair each part as directed in Self Diagnostic Test Charts.

- Turn ignition switch Off.

CLEARING ECU MEMORY

After repairs have been completed, disconnect battery ground cable for at least 10 to 15 seconds to erase codes from ECU memory.

## 88–95 3.0L (Except Monaco, Premier & Stealth),
3.3L, 3.5L & 3.8L Engines

| 11 | Engine Has Not Been Cranked Since Battery Was Disconnected{4} |
| --- | --- |
|  | No Crank Sensor Signal During Engine Cranking{3} |
| 12 | Memory Standby Power Recently Lost |
| 13 | MAP Sensor Pneumatic Circuit |
| 14 | MAP Sensor Electrical Circuit |
| 15 | Vehicle/Distance Sensor Circuit |
| 16 | Loss Of Battery Voltage Sense |
| 17 | Engine Running Too Cool |
| 21 | Problem In Oxygen Feedback Circuit |
| 22 | Coolant Temperature Sensor Circuit |
| 23 | Charge Temperature Sensor Circuit |
| 24 | Throttle Position Sensor Circuit |
| 25 | AIS Motor Drive Circuit |
| 26 | Injector Current Limit Not Achieved |
| 27 | Injector Driver Interface Circuit{1} |
|  | Injector Control Circuit{2} |
| 31 | Purge Solenoid Circuit |
| 32 | EGR Diagnostics |
| 33 | A/C Cutout Relay Circuit Or A/C Clutch Relay Circuit |
| 34 | Fault In Speed Control Solenoid Driver Circuit |
| 35 | Fault In Fan Relay Circuit |
| 37 | Part Throttle Unlock Solenoid Driver Circuit |
| 41 | Fault In Charging System Or No Field Current |
| 42 | Fault In ASD Relay Driver Circuit |
| 43 | In Ignition Coil Control Circuits |
| 44 | Lose Of FJ2 To Logic Board |
| 46 | Battery Voltage Is Too High |
| 47 | Battery Voltage Is Too Low |
| 51 | Lean Condition Is Indicated |
| 52 | Rich Condition Is Indicated |
| 53 | Internal Problem In Module |
| 54 | Fault In Distributor High Data Rate Pickup Circuit{1} |
|  | No Sync Pickup Signal{2} |
| 55 | End Of Message |
| 62 | Unsuccessful Attempt To Update EMR Mileage |
| 63 | Fault Error{1} |
|  | Controller Failure EEPROM Write Denied{2} |
| 65 | Manifold Tuning Valve |
| 66 | CCD Bus System |
| 77 | Speed Control Relay Circuit |

{1}—1988–91 models.
{2}—1992–95 models.
{3}—1990–95 models.
{4}—Neon.

## 89–90 3.0L Dodge Monaco & Eagle Premier

| 1000 | Ignition Line Low |
| --- | --- |
| 1001 | Ignition Line High |
| 1002 | Checking Vehicle Configuration |
| 1004 | Battery Voltage Low |
| 1005 | Sensor Ground Line Out Of Limits |
| 1010 | Diagnostic Enable Line Low |
| 1011 | Diagnostic Enable Line High |
| 1012 | MPI Line Low |
| 1013 | MPI Line High |
| 1014 | Fuel Pump Line Low |
| 1015 | Fuel Pump Line High |
| 1016 | Charge Air Temperature Sensor Line Low |
| 1017 | Charge Air Temperature Sensor Line High |
| 1021 | Engine Failed To Start |
| 1022 | Start Line Low |
| 1024 | ECU Does Not See Start Signal |
| 1027 | ECU Sees Wide Open Throttle |
| 1028 | ECU Does Not See Wide Open Throttle |
| 1031 | ECU Sees Closed Throttle |
| 1032 | ECU Does Not See Closed Throttle |
| 1033 | Idle Speed Increase Line Low |
| 1034 | Idle Speed Increase Line High |
| 1035 | Idle Speed Decrease Line Low |
| 1036 | Idle Speed Decrease Line High |
| 1037 | Throttle Position Sensor Reads Low |
| 1038 | Park/Neutral Line High |
| 1040 | Latched B+ Line Low |
| 1041 | Latched B+ Line High |
| 1042 | No Latched B+ 1/2 Volt Drop |
| 1048 | Manual Vehicle Equipped With Automatic ECU |
| 1049 | Automatic Vehicle Equipped With Manual ECU |
| 1050 | Idle RPM Less Than 500 |
| 1051 | Idle RPM Greater Than 2000 |
| 1052 | MAP Sensor Out Of Limits |
| 1053 | Change In MAP Reading (Out Of Limit) |
| 1054 | Coolant Temperature Sensor Line Low |
| 1055 | Coolant Temperature Sensor Line High |
| 1056 | Inactive Coolant Temperature Sensor |
| 1057 | Knock Sensor Shorted |
| 1058 | Knock Sensor Value (Out Of Limits) |
| 1059 | A/C Request Line Low |
| 1060 | A/C Request Line High |
| 1061 | A/C Select Line Low |
| 1062 | A/C Select Line High |
| 1063 | A/C Clutch Line Low |
| 1064 | A/C Clutch Line High |
| 1065 | Oxygen Sensor Reads Rich |
| 1066 | Oxygen Sensor Reads Lean |
| 1067 | Latch Relay Line Low |
| 1068 | Latch Relay Line High |
| 1069 | No Tach |
| 1070 | A/C Cutout Line Low |
| 1071 | A/C Cutout Line High |
| 1073 | ECU Does Not See Speed Sensor Signal |
| 1200 | ECU Defective |
| 1202 | All Injectors Shorted To Ground |
| 1209 | All Injectors Open |
| 1218 | No Voltage At ECU From Power Latch Relay |
| 1220 | No Voltage At ECU From EGR Solenoid |
| 1221 | No Injector Voltage |
| 1222 | MPI Not Grounded |
| 1223 | No ECU Tests Run |

## 89–95 1.5L, 1.6L, 1.8L, 2.4L (VIN G) & 
89–94 2.0L Car

| Code | Circuit Or Condition |
| --- | --- |
| 11 | Oxygen Sensor |
| 12 | Air Flow Sensor |
| 13 | Intake Air Temperature Sensor |
| 14 | Throttle Position Sensor |
| 15 | ISC Motor Position Sensor{1} |
| 21 | ECT Sensor |
| 22 | Crank Angle Sensor |
| 23 | TDC Sensor |
| 24 | Vehicle Speed Sensor |
| 25 | Barometric Pressure Sensor |
| 31 | Detonation Sensor{1} |
| 32 | MAP Sensor{1} |
| 36 | Ignition Timing Adjustment Signal |
| 41 | Injector |
| 42 | Fuel Pump |
| 43 | EGR |
| 44 | Ignition Coil |
| 55 | Servo Valve Position Sensor{2} |
| 59 | Heated Oxygen Sensor (Rear){2} |
| 61 | Transaxle Control Module Wire |

{1}—Plymouth Laser & Eagle Talon.
{2}—Except Plymouth Laser & Eagle Talon.

## 89–95 Jeep

| 1989–90 |  |
| --- | --- |
| 1000 | Ignition Line Low |
| 1001 | Ignition Line High |
| 1002 | Oxygen Heater Line Low |
| 1004 | Battery Voltage Low |
| 1005 | Sensor Ground Line Out Of Limits |
| 1013 | MPA Line High |
| 1014 | Fuel Pump Line Low |
| 1015 | Fuel Pump Line High |
| 1016 | Charge Air Temperature Sensor Line Low |
| 1017 | Charge Air Temperature Sensor Line High |
| 1018 | No Serial Data |
| 1020 | No Sync Pulse Seen |
| 1021 | Engine Failed To Start Due To Mechanical, Fuel Or Ignition Problem |
| 1022 | Start Line Low |
| 1027 | ECU Sees Wide Open Throttle |
| 1028 | ECU Does Not See Wide Open Throttle |
| 1031 | ECU Sees Closed Throttle |
| 1032 | ECU Does Not See Closed Throttle |
| 1037 | Throttle Position Sensor Reads Low |
| 1038 | Park/Neutral Line High |
| 1039 | Park/Neutral Line Low |
| 1040 | Latched B+ Line Low |
| 1041 | Latched B+ Line High |
| 1042 | No Latched B+ 1/2 Volt Drop |
| 1044 | Fault In Shift Light Circuit |
| 1047 | Wrong ECU |
| 1048 | Manual Vehicle Equipped w/Automatic ECU |
| 1049 | Automatic Vehicle Equipped w/Manual ECU |
| 1050 | Idle RPM Less Than 500 |
| 1051 | Idle RPM Greater Than 2000 |
| 1052 | MAP Sensor Out Of Limits |
| 1053 | Change In MAP Reading Out Of Limits |
| 1054 | Coolant Temperature Sensor Line Low |
| 1055 | Coolant Temperature Sensor Line High |
| 1056 | Inactive Coolant Temperature Sensor |
| 1057 | Knock Circuit Shorted |
| 1058 | Knock Value Out Of Limits |
| 1059 | A/C Request Line Low |
| 1060 | A/C Request Line High |
| 1061 | A/C Select Line Low |
| 1062 | A/C Select Line High |
| 1063 | A/C Clutch Line Low |
| 1064 | A/C Clutch Line High |
| 1065 | Oxygen Sensor Reads Rich |
| 1066 | Oxygen Sensor Reads Lean |
| 1067 | Latch Relay Line Low |
| 1068 | Latch Relay Line High |
| 1069 | No Tach |
| 1072 | IAS Motor Cannot Control Idle |
| 1073 | ECU Does Not See Speed Sensor |
| 1200 | Defective ECU Or Shorted To Voltage |
| 1201 | Injector Short To B+ |
| 1202 | All Injectors Shorted To Ground |
| 1203 | Injector No. 1 Shorted To Ground |
| 1204 | Injector No. 2 Shorted To Ground |
| 1205 | Injector No. 3 Shorted To Ground |
| 1206 | Injector No. 4 Shorted To Ground |
| 1207 | Injector No. 5 Shorted To Ground |
| 1208 | Injector No. 6 Shorted To Ground |
| 1209 | All Injectors Open |
| 1210 | Injector No. 1 Circuit Open |
| 1211 | Injector No. 2 Circuit Open |
| 1212 | Injector No. 3 Circuit Open |
| 1213 | Injector No. 4 Circuit Open |
| 1214 | Injector No. 5 Circuit Open |
| 1215 | Injector No. 6 Circuit Open |
| 1216 | No Voltage At ECU From A/C Relay |
| 1217 | No Voltage At ECU From Oxygen Heater Relay |
| 1218 | No Voltage At ECU From Power Latch Relay |
| 1219 | No Voltage At ECU From Shift Light |
| 1220 | No Voltage At ECU From EGR Solenoid |
| 1222 | MPA Not Grounded |
| 1223 | Open Injector Feed Circuits |
| 1991–95 |  |
| 11 | Ignition Reference Signal |
| 12 | Battery Disconnected |
| 13 | MAP Signal Fault |
| 14 | MAP Voltage Fault |
| 15 | No Vehicle Speed Signal |
| 17 | Engine Is Cold Too Long |
| 21 | Oxygen Signal Circuit |
| 22 | Improper Coolant Sensor Voltage |
| 23 | Improper Charge Temperature Sensor Voltage |
| 24 | Improper TP Sensor Voltage |
| 25 | Auto. Idle Speed Motor Circuit |
| 27 | Injector Control Circuit |
| 31 | EVAP Purge Solenoid Circuit |
| 32 | EGR System Failure |
| 33 | A/C Clutch Relay Circuit |
| 34 | Speed Control Solenoid Circuits |
| 35 | Radiator Fan Relay |
| 37 | Torque Converter Clutch Solenoid Circuit |
| 41 | Alternator Field Switching Improperly |
| 42 | ASD Relay Circuit |
| 44 | Improper Battery Temperature Sensor Voltage |
| 45 | Alternator Field Switching Improperly |
| 46 | Charging System Voltage Too High |
| 47 | Charging System Voltage Too Low |
| 51 | Oxygen Sensor Signal Lean |
| 52 | Oxygen Sensor Signal Rich |
| 53 | Internal Controller Failure |
| 54 | No Sync Pickup Signal |
| 55 | Completion Of Fault Code Display |
| 62 | Controller Failure, EMR Miles Not Stored |
| 63 | Controller Failure, EEPROM Write Denied |
| 76 | Fuel Pump Resistor Bypass Relay Circuit |

## 89–95 1.5L, 1.6L, 1.8L, 2.4L (VIN G) & 
89–94 2.0L Car

| Code | Circuit Or Condition |
| --- | --- |
| 11 | Oxygen Sensor |
| 12 | Air Flow Sensor |
| 13 | Intake Air Temperature Sensor |
| 14 | Throttle Position Sensor |
| 15 | ISC Motor Position Sensor{1} |
| 21 | ECT Sensor |
| 22 | Crank Angle Sensor |
| 23 | TDC Sensor |
| 24 | Vehicle Speed Sensor |
| 25 | Barometric Pressure Sensor |
| 31 | Detonation Sensor{1} |
| 32 | MAP Sensor{1} |
| 36 | Ignition Timing Adjustment Signal |
| 41 | Injector |
| 42 | Fuel Pump |
| 43 | EGR |
| 44 | Ignition Coil |
| 55 | Servo Valve Position Sensor{2} |
| 59 | Heated Oxygen Sensor (Rear){2} |
| 61 | Transaxle Control Module Wire |

{1}—Plymouth Laser & Eagle Talon.
{2}—Except Plymouth Laser & Eagle Talon.

## 91–92 3.0L Dodge Monaco & Eagle Premier

| 11 | No Reference Signal During Cranking |
| --- | --- |
| 13 | MAP Signal |
| 14 | MAP Voltage Too Low Or Too High |
| 15 | Vehicle Speed Signal |
| 17 | Engine Cold Too Long |
| 21 | Oxygen Sensor |
| 22 | Coolant Sensor |
| 23 | Charge Temperature Sensor |
| 24 | Throttle Position Sensor |
| 25 | Automatic Idle Speed (AIS) Motor |
| 26 | Injector |
| 27 | Injector Control Circuit |
| 32 | EGR System Failure |
| 33 | A/C Clutch Relay Circuit |
| 34 | Speed Control Solenoid Circuit |
| 35 | Radiator Fan Relay Circuit |
| 42 | ASD Relay And/Or Control Circuit |
| 43 | Ignition Coil Primary Circuit |
| 51 | Oxygen Sensor (Lean Indication) |
| 52 | Oxygen Sensor (Rich Indication) |
| 53 | Internal Controller Failure |
| 54 | No Sync Pickup Signal |
| 55 | End Of Message |
| 63 | Controller Failure EEPROM Write Denied |
| 77 | Speed Control Power Relay Circuit |

## 89-93 Plymouth Colt 1.5L code retrieval

READING DIAGNOSTIC CODES

- Connect an analog voltmeter to the ground and self diagnosis output pin of the diagnostic connector.

- Set the ignition switch to "ON".

- Note the pattern of pulses displayed on the voltmeter. If the voltmeter indicates a steady 12 VDC the ECU has failed.

- Refer to the diagnostic codes chart to determine which device is causing the malfunction code.

- Test and repair the offending components as necessary.

- Clear the malfunction codes by disconnecting the vehicle battery negative terminal for ten seconds or longer.

- Road test the vehicle and recheck for newly stored diagnostic codes.

- After the cause of a failure code has been detected and corrected the diagnostic code memory of the ECU should be cleared by disconnecting the vehicle battery for 10 seconds or more.

DIAGNOSTIC CODE INTERPRETATION 

The Engine Control Unit (ECU) monitors operation of the emission control system. Some components are monitored continuously, others only during specified operating conditions. When the ECU detects a malfunction lasting longer than a specified period of time the ECU will generate an error code that can be detected at the diagnostic connector.

There are sixteen diagnostic items (including normal operation) that will generate diagnostic codes. The malfunction codes are erased when the vehicle battery is disconnected for more than ten seconds.
NOTE: If the oxygen sensor is disconnected during engine operation for testing purposes a code will be stored and must be cleared after the test is complete.
To test for oxygen sensor codes drive the vehicle a significant distance and check for diagnostic codes before turning the ignition off. Do not however disconnect the battery with the ignition on or the engine running as permanent damage to the vehicle electronic components may result.
If multiple malfunctions occur the diagnostic codes will be sent sequentially beginning with the lowest number code in ascending order with a 2 second pause between each code.
The voltmeter needle will deflect for a particular duration and a certain number of times.

|  | FOR EXAMPLE: Code 13 |
| --- | --- |

The needle will deflect one time for approximately 0.5 second. This will represent one unit of ten. There will be approximately a one second delay and then the needle will deflect three times for approximately 0.2 seconds with 0.1 second delay between deflections. This will represent three units of one.

Code 23

The needle will deflect two times for approximately 0.5 second with approximately 0.3 second delay between deflections. This will represent two units of ten. There will be approximately a one second delay and then the needle will deflect three times for approximately 0.2 seconds with 0.1 second delay between deflections. This will represent three units of one.

## 91–95 3.0L Dodge Stealth

| 11 | Oxygen Sensor |
| --- | --- |
| 12 | Air Flow Sensor |
| 13 | Intake Air Temperature Sensor |
| 14 | Throttle Position Sensor |
| 21 | ECT Sensor |
| 22 | Crank Angle Sensor |
| 23 | Top Dead Center Sensor |
| 24 | Vehicle Speed Sensor Reed Switch |
| 25 | Barometric Pressure Sensor |
| 31 | Detonation Sensor |
| 36 | Ignition Timing Adjustment Signal |
| 39 | Front Oxygen Sensor |
| 41 | Injector |
| 43 | EGR |
| 44 | Ignition Coil, Power Transistor Unit, No. 1 & 4 Cylinders |
| 45 | Ignition Signal No. 2 |
| 46 | Ignition Signal No. 3 |
| 52 | Ignition Coil, Power Transistor Unit, No. 2 & 5 Cylinders |
| 53 | Ignition Coil, Power Transistor Unit, No. 3 & 6 Cylinders |
| 61 | Torque Decrease Demand Signal Line |
| 62 | Warm Up Control Valve Position Sensor |

## 95 2.4L (VIN X)

| 11 | No Crank Reference At PCM |
| --- | --- |
| 12 | Battery Disconnected |
| 13 | No Change In MAP From Start To Run |
| 14 | MAP Sensor Voltage Too Low, Too High Or No 5 Volts |
| 15 | No VSS Signal |
| 16 | Knock Sensor Circuit |
| 17 | Engine Too Cold Too Long |
| 21 | Upstream O2S Shorted To Voltage Or Stays At Center |
| 22 | ECT Sensor Voltage Too Low Or Too High |
| 23 | IAT Sensor Voltage Too Low Or Too High |
| 24 | TP Sensor Voltage Too Low, Too High Or No 5 Volts |
| 25 | IAC Motor Circuits |
| 27 | Injector No.1, 2, 3 Or 4 Control Circuit |
| 31 | EVAP Solenoid Circuit |
| 32 | EGR System Failure Or EGR Solenoid Circuit |
| 33 | A/C Clutch relay Circuit Or A/c Pressure Sensor Volts Too Low Or Too High |
| 34 | Speed Control Solenoid Circuit Or Speed Control Switch Always Low Or High |
| 35 | High Or Low Speed Fan Control Relay Circuit |
| 41 | Alternator Field Not Switching Properly |
| 42 | Fuel Pump relay Control Circuit Or Auto Shutdown Relay Control Circuit Or No ASD Relay Output voltage At PCM |
| 43 | Ignition Coil No. 1 Or No. 2 Primary Circuit |
| 44 | Ambient Temperature Sensor |
| 46 | Charging Voltage Too High |
| 47 | Charging Voltage Too Low |
| 51 | Upstream O2S Stays Below (Lean) Center |
| 52 | Upstream O2S Stays Above (Rich) Center |
| 53 | Internal Controller Failure Or PCM Failure SPI Communications |
| 54 | No Cam Signal From PCM |
| 55 | Completion Of Fault Code Display |
| 62 | PCM Failure SRI Mile Not Stored |
| 63 | PCM Failure EEPROM Write Denied |
| 66 | No CCD Message From TCM Or Body Controller |
| 71 | 5 Volt Output Low |
| 77 | Speed Control Power Circuit |

## 95 2.5L w/SFI

Avenger & Sebring

| DRB II | MIL |  |
| --- | --- | --- |
| 01 | 54 | No CAM Signal |
| 02 | 53 | Intermittent Controller Failure |
| 03 | 52 | Front Bank O2S Stays Above Center (Rich) |
| 04 | 51 | Front Bank O2S Stays Below Center (Lean) |
| 05 | 47 | Charging System Voltage Too Low |
| 06 | 46 | Charging System Voltage Too High |
| 08 | 52 | Rear Bank O2S Stays Above Center (Rich) |
| 09 | 51 | Rear Bank O2S Stays Below Center (Lean) |
| 10 | 42 | MFI Relay (ASD Relay) Control Circuit |
| 11 | 41 | Alternator Field Not Switching Properly |
| 16 | 33 | A/C Clutch Relay Circuit |
| 17 | 32 | EGR Solenoid Circuit |
| 18 | 31 | EVAP Purge Solenoid Circuit |
| 19 | 27 | Injector No. 3 Control Circuit |
| 20 | 27 | Injector No. 2 Control Circuit |
| 21 | 27 | Injector No. 1 Control Circuit |
| 25 | 25 | Idle Air Control Motor Circuit |
| 26 | 24 | TP Sensor Voltage Low |
| 27 | 24 | TP Sensor Voltage High |
| 30 | 22 | ECT Sensor Voltage Low |
| 31 | 22 | ECT Sensor Voltage High |
| 32 | 21 | Front Bank O2S Stays At Center |
| 33 | 17 | Engine Is Cold Too Long |
| 35 | 15 | No VSS Signal |
| 36 | 14 | MAP Sensor Voltage Too Low |
| 37 | 14 | MAP Sensor Voltage Too High |
| 39 | 13 | Slow Change In Idle MAP Sensor Signal |
| 40 | 11 | No Crankshaft Reference Signal At PCM |
| 44 | 42 | No MFI Relay (ASD Relay) Output Voltage At PCM |
| 46 | 32 | EGR System Failure |
| 48 | 62 | PCM Failure SRI Mile Not Stored |
| 49 | 63 | PCM Failure EEPROM Write Denied |
| 57 | 23 | IAT Sensor Voltage Low |
| 58 | 23 | IAT Sensor Voltage High |
| 61 | 27 | Injector No. 4 Control Circuit |
| 62 | 21 | Front Bank O2S Shorted To Voltage |
| 65 | 21 | Rear Bank O2S Stays At Center |
| 66 | 21 | Rear Bank O2S Shorted To Voltage |
| 68 | 53 | PCM Failure SPI Communications |
| 69 | 27 | Injector No. 5 Control Circuit |
| 70 | 27 | Injector No. 6 Control Circuit |
| 71 | 44 | Battery Temperature Sensor Volts Out Of Limits |
| 92 | 35 | Low Speed fan Control Relay Circuit |
| 93 | 35 | High Speed Condenser Fan Control Relay Circuit |
| 96 | 66 | No CCD Messages From TCM |
| 101 | 42 | Fuel Pump Relay Control Circuit |
| 135 | 14 | No 5 Volts At MAP Sensor |
| 136 | 24 | No 5 Volts At TP Sensor |
| 139 | 35 | High Speed Fan & High Fan Ground Control Relay Circuit |
| 146 | 71 | 5 Volt supply Output Low |
| 157 | 11 | Intermittent Loss Of Camshaft Position Or Crankshaft Position |

## 95–96 1.5L, 1.8L, 2.4L (VIN B) & 3.0L OBD II 
& 95–98 2.0L Turbo

| P0100 | Volume Air Flow Circuit Malfunction |
| --- | --- |
| P0105 | Barometric Pressure Circuit Malfunction |
| P0107 | MAP Sensor Voltage Too Low |
| P0108 | MAP Sensor Voltage Too High |
| P0110 | Intake Air Temperature Circuit Malfunction |
| P0112 | Intake Air Temperature Sensor Voltage Low |
| P0113 | Intake Air Temperature Sensor Voltage High |
| P0115 | Engine Coolant Temperature Circuit Malfunction |
| P0117 | ECT Sensor Voltage Too Low |
| P0118 | ECT Sensor Voltage Too High |
| P0120 | Throttle Position Circuit Malfunction |
| P0121 | TP Sensor Voltage Does Not Agree With MAP |
| P0122 | TP Sensor Voltage Low |
| P0123 | TP Sensor Voltage High |
| P0125 | Excessive Time To Enter Closed Loop Fuel Control |
| P0130 | Bank 1 Heated Oxygen Sensor 1 Circuit Malfunction |
| P0131 | Upstream O2S Voltage Shorted To Ground |
| P0132 | Upstream O2S Shorted To Voltage |
| PO133 | Upstream O2S Slow Response |
| P0134 | Upstream O2S Stays At Center |
| P0135 | Bank 1 Heated Oxygen Sensor 1 Heater Circuit Malfunction |
| P0136 | Bank 1 Heated Oxygen Sensor 2 Circuit Malfunction |
| P0137 | Downstream O2S Voltage Shorted To Ground |
| P0138 | Downstream O2S Shorted To Voltage |
| P0139 | Downstream O2S Unable To Switch Rich/Lean |
| P0140 | Downstream O2S Stays At Center |
| P0141 | Bank 1 Heated Oxygen Sensor 2 Heater Circuit Malfunction |
| P0151 | RH Upstream O2S Voltage Shorted To Ground |
| P0152 | RH Upstream O2S Voltage Shorted To Voltage |
| P0154 | RH Upstream O2S Stays At Center |
| P0155 | RH Upstream O2S Heater Failure |
| P0157 | RH Downstream O2S Shorted To Ground |
| P0158 | RH Downstream O2S Shorted To Voltage |
| P0160 | RH Downstream O2S Stays At Center |
| P0161 | RH Downstream O2S Heater Failure |
| P0170 | Fuel Trim Malfunction, Bank 1 |
| P0171 | Fuel System Lean |
| P0172 | Fuel System Rich |
| P0174 | RH Downstream Fuel System Lean |
| P0175 | LH Downstream Fuel System Lean |
| P0201–P0208 | Injector Circuit Malfunction, Nos. 1–8 |
| P0300 | Random Misfire Detected |
| P0301–P0310 | Misfire Detected, Cylinder Nos. 1–10 |
| P0325 | Knock Sensor 1 Circuit Malfunction |
| P0335 | Crankshaft Position Sensor Circuit Malfunction |
| P0340 | Camshaft Position Sensor Circuit Malfunction |
| P0351 | Ignition Coil No. 1 Primary Circuit |
| P0352 | Ignition Coil No. 2 Primary Circuit |
| P0353 | Ignition Coil No. 3 Primary Circuit |
| P0400 | Exhaust Gas Recirculation (EGR) Flow Malfunction |
| P0401 | EGR System Failure |
| P0403 | EGR Solenoid Malfunction |
| P0420 | Catalytic Efficiency Below Threshold, Bank 1 |
| P0421 | Warm Up Catalyst Efficiency Below Threshold (Bank 1) (California) |
| P0432 | Catalytic Converter Efficiency Failure |
| P0440 | Evaporative Emission Control System Malfunction |
| P0441 | EVAP Purge Flow Monitor Failure |
| P0442 | EVAP System Small Leak |
| P0443 | Evaporative Emission Control System Purge Control Valve Circuit Malfunction |
| P0446 | EVAP Vent Control Malfunction |
| P0450 | EVAP Pressure Sensor Malfunction |
| P0455 | EVAP System Large Leak |
| P0500 | Vehicle Speed Sensor Malfunction |
| P0505 | Idle Control System Malfunction |
| P0510 | Closed TP Switch Malfunction |
| P0551 | Power Steering Switch Failure |
| P0600 | PCM Internal Fault SPI Communications |
| P0601 | PCM Internal Fault Internal Controller Failure |
| P0700 | Transaxle Or Transmission Control System Malfunction |
| P0705 | Transmission Range Sensor Circuit Malfunction (PRNDL Input) |
| P0710 | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P1103 | Turbocharger Wastegate Actuator Malfunction |
| P1104 | Turbocharger Wastegate Solenoid Malfunction |
| P1105 | Fuel Pressure Solenoid Malfunction |
| P1294 | Target Idle Not Reached |
| P1296 | No 5 Volt To MAP Sensor |
| P1297 | No Change In MAP From Start To Run |
| P1299 | Vacuum Leak Found (IAC Fully Seated) |
| P1300 | Ignition Timing Adjustment Circuit Malfunction |
| P1390 | Timing Belt Skipped One Or More Notches |
| P1391 | Camshaft Or Crankshaft Position Sensor Intermittent Loss |
| P1398 | Crankshaft Position Sensor Target Windows Excessive Variation |
| P1400 | Manifold Differential Pressure Sensor Circuit Malfunction |
| P1486 | EVAP Leak Monitor Hose Pinched |
| P1489 | High Speed Cooling Fan Relay Circuit Open Or Short |
| P1490 | Low Speed Cooling Fan Relay Circuit Open Or Short |
| P1492 | Battery Temperature Sensor Voltage Too High |
| P1493 | Battery Temperature Sensor Voltage Too Low |
| P1494 | Leak Detection Pump Pressure Switch |
| P1495 | Leak Detection Pump Solenoid Circuit |
| P1496 | 5 Volt Output Low |
| P1500 | Alternator FR Terminal Circuit Malfunction |
| P1600 | Serial Communication Link Malfunction |
| P1696 | PCM Failure EEPROM Write Denied |
| P1697 | PCM Failure SRI Mile Not Stored |
| P1715 | PG Assembly Malfunction |
| P1750 | Solenoid Assembly Malfunction |
| P1791 | ECT Level Input Circuit |
| P1899 | Park/Neutral Switch Failure |

## 95–96 1.5L, 1.8L, 2.4L (VIN B) & 3.0L OBD II 
& 95–98 2.0L Turbo

| P0100 | Volume Air Flow Circuit Malfunction |
| --- | --- |
| P0105 | Barometric Pressure Circuit Malfunction |
| P0107 | MAP Sensor Voltage Too Low |
| P0108 | MAP Sensor Voltage Too High |
| P0110 | Intake Air Temperature Circuit Malfunction |
| P0112 | Intake Air Temperature Sensor Voltage Low |
| P0113 | Intake Air Temperature Sensor Voltage High |
| P0115 | Engine Coolant Temperature Circuit Malfunction |
| P0117 | ECT Sensor Voltage Too Low |
| P0118 | ECT Sensor Voltage Too High |
| P0120 | Throttle Position Circuit Malfunction |
| P0121 | TP Sensor Voltage Does Not Agree With MAP |
| P0122 | TP Sensor Voltage Low |
| P0123 | TP Sensor Voltage High |
| P0125 | Excessive Time To Enter Closed Loop Fuel Control |
| P0130 | Bank 1 Heated Oxygen Sensor 1 Circuit Malfunction |
| P0131 | Upstream O2S Voltage Shorted To Ground |
| P0132 | Upstream O2S Shorted To Voltage |
| PO133 | Upstream O2S Slow Response |
| P0134 | Upstream O2S Stays At Center |
| P0135 | Bank 1 Heated Oxygen Sensor 1 Heater Circuit Malfunction |
| P0136 | Bank 1 Heated Oxygen Sensor 2 Circuit Malfunction |
| P0137 | Downstream O2S Voltage Shorted To Ground |
| P0138 | Downstream O2S Shorted To Voltage |
| P0139 | Downstream O2S Unable To Switch Rich/Lean |
| P0140 | Downstream O2S Stays At Center |
| P0141 | Bank 1 Heated Oxygen Sensor 2 Heater Circuit Malfunction |
| P0151 | RH Upstream O2S Voltage Shorted To Ground |
| P0152 | RH Upstream O2S Voltage Shorted To Voltage |
| P0154 | RH Upstream O2S Stays At Center |
| P0155 | RH Upstream O2S Heater Failure |
| P0157 | RH Downstream O2S Shorted To Ground |
| P0158 | RH Downstream O2S Shorted To Voltage |
| P0160 | RH Downstream O2S Stays At Center |
| P0161 | RH Downstream O2S Heater Failure |
| P0170 | Fuel Trim Malfunction, Bank 1 |
| P0171 | Fuel System Lean |
| P0172 | Fuel System Rich |
| P0174 | RH Downstream Fuel System Lean |
| P0175 | LH Downstream Fuel System Lean |
| P0201–P0208 | Injector Circuit Malfunction, Nos. 1–8 |
| P0300 | Random Misfire Detected |
| P0301–P0310 | Misfire Detected, Cylinder Nos. 1–10 |
| P0325 | Knock Sensor 1 Circuit Malfunction |
| P0335 | Crankshaft Position Sensor Circuit Malfunction |
| P0340 | Camshaft Position Sensor Circuit Malfunction |
| P0351 | Ignition Coil No. 1 Primary Circuit |
| P0352 | Ignition Coil No. 2 Primary Circuit |
| P0353 | Ignition Coil No. 3 Primary Circuit |
| P0400 | Exhaust Gas Recirculation (EGR) Flow Malfunction |
| P0401 | EGR System Failure |
| P0403 | EGR Solenoid Malfunction |
| P0420 | Catalytic Efficiency Below Threshold, Bank 1 |
| P0421 | Warm Up Catalyst Efficiency Below Threshold (Bank 1) (California) |
| P0432 | Catalytic Converter Efficiency Failure |
| P0440 | Evaporative Emission Control System Malfunction |
| P0441 | EVAP Purge Flow Monitor Failure |
| P0442 | EVAP System Small Leak |
| P0443 | Evaporative Emission Control System Purge Control Valve Circuit Malfunction |
| P0446 | EVAP Vent Control Malfunction |
| P0450 | EVAP Pressure Sensor Malfunction |
| P0455 | EVAP System Large Leak |
| P0500 | Vehicle Speed Sensor Malfunction |
| P0505 | Idle Control System Malfunction |
| P0510 | Closed TP Switch Malfunction |
| P0551 | Power Steering Switch Failure |
| P0600 | PCM Internal Fault SPI Communications |
| P0601 | PCM Internal Fault Internal Controller Failure |
| P0700 | Transaxle Or Transmission Control System Malfunction |
| P0705 | Transmission Range Sensor Circuit Malfunction (PRNDL Input) |
| P0710 | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P1103 | Turbocharger Wastegate Actuator Malfunction |
| P1104 | Turbocharger Wastegate Solenoid Malfunction |
| P1105 | Fuel Pressure Solenoid Malfunction |
| P1294 | Target Idle Not Reached |
| P1296 | No 5 Volt To MAP Sensor |
| P1297 | No Change In MAP From Start To Run |
| P1299 | Vacuum Leak Found (IAC Fully Seated) |
| P1300 | Ignition Timing Adjustment Circuit Malfunction |
| P1390 | Timing Belt Skipped One Or More Notches |
| P1391 | Camshaft Or Crankshaft Position Sensor Intermittent Loss |
| P1398 | Crankshaft Position Sensor Target Windows Excessive Variation |
| P1400 | Manifold Differential Pressure Sensor Circuit Malfunction |
| P1486 | EVAP Leak Monitor Hose Pinched |
| P1489 | High Speed Cooling Fan Relay Circuit Open Or Short |
| P1490 | Low Speed Cooling Fan Relay Circuit Open Or Short |
| P1492 | Battery Temperature Sensor Voltage Too High |
| P1493 | Battery Temperature Sensor Voltage Too Low |
| P1494 | Leak Detection Pump Pressure Switch |
| P1495 | Leak Detection Pump Solenoid Circuit |
| P1496 | 5 Volt Output Low |
| P1500 | Alternator FR Terminal Circuit Malfunction |
| P1600 | Serial Communication Link Malfunction |
| P1696 | PCM Failure EEPROM Write Denied |
| P1697 | PCM Failure SRI Mile Not Stored |
| P1715 | PG Assembly Malfunction |
| P1750 | Solenoid Assembly Malfunction |
| P1791 | ECT Level Input Circuit |
| P1899 | Park/Neutral Switch Failure |

## 95–98 2.0L Non-Turbo, 2.4L (VIN X), 
2.5L/V6, 2.7L, 3.2L, 3.3L & 3.5L w/OBD II

| MIL 95 | OBD-II | Hex |  |
| --- | --- | --- | --- |
| 11 | — | — | Crank Sensor |
| 13 | P1297 | — | MAP Sensor Fault |
| 14 | P0107 | — | MAP Sensor Voltage Low |
|  | P0108 | — | MAP Sensor Voltage High |
|  | P1296 | — | No 5 Volts To MAP Sensor{2} |
| 15 | P0500 | — | No Vehicle Speed Sensor Signal |
| 16 | — | — | Knock Sensor No. 1 Circuit |
| 17 | P0125 | — | Closed Loop Temperature Not Reached |
| 21 | P0132 | — | Upstream O2 Sensor Shorted To Voltage |
|  | P0134 | — | Upstream O2 Sensor Stays At Center |
|  | P0135 | — | Upstream O2 Sensor Heater Failure |
|  | P0133 | — | Upstream O2 Sensor Slow Response |
|  | P0131 | — | Upstream O2 Sensor Voltage Shorted To Ground{2} |
|  | P0138 | — | Downstream O2 Sensor Shorted To Voltage |
|  | P0140 | — | Downstream O2 Sensor Stays On At Center{2} |
|  | P0141 | — | Downstream O2 Sensor Heater Failure |
|  | P0139 | — | Downstream O2 Sensor Slow Response |
|  | P0137 | — | Downstream O2 Sensor Voltage Shorted To Ground{2} |
| 22 | P0117 | — | ECT Sensor Voltage Low |
|  | P0118 | — | ECT Sensor Voltage High |
| 23 | P0112 | — | IAT Sensor Low |
|  | P0113 | — | IAT Sensor High |
| 24 | P0122 | — | TP Sensor Voltage Low |
|  | P0123 | — | TP Sensor Voltage High |
|  | P0121 | — | TP Sensor Voltage Does Not Agree With MAP |
|  | P1295 | — | No 5 Volts To TP Sensor{2} |
| 25 | P0505 | — | IAC Motor Circuits |
| 27 | P0201 | — | Injector No. 1 Control Circuit |
|  | P0202 | — | Injector No. 2 Control Circuit |
|  | P0203 | — | Injector No. 3 Control Circuit |
|  | P0204 | — | Injector No. 4 Control Circuit |
| 31 | P0443 | — | EVAP Solenoid Circuit |
|  | P0441 | — | EVAP Purge Flow Monitor Failure |
| 32 | P0403 | — | EGR Solenoid Circuit |
|  | P0403 | — | EGR System Failure |
| 33 | — | — | A/C Clutch Relay Circuit |
| 34 | — | — | Speed Control Solenoid Circuit |
| 35 | P1491 | — | Fan Control Relay Circuit{1} |
|  | P1489 | — | High Speed Fan Control Relay Circuit |
|  | P1490 | — | Low Speed Fan Control Relay Circuit |
| 36 | P0412 | — | Air Switching Solenoid Circuit{3} |
|  | — | — | Too Little Or To Much Secondary Air{3} |
| 37 | P1899 | — | PNP Switch Failure |
|  | P0740 | — | Torque Converter Clutch, No RPM Drop At Lockup{1} |
|  | P0743 | — | Torque Converter Clutch Solenoid Circuit{1} |
| 41 | — | — | Alternator Field Not Switching Properly |
| 42 | — | — | Fuel Pump Relay Control Circuit |
| 42 | P0352 | — | Ignition Coil No. 2 Primary Circuit |
| 43 | P0351 | — | Ignition Coil No. 1 Primary Circuit |
| 43 | P0352 | — | Ignition Coil No. 2 Primary Circuit |
| 44 | — | — | Battery Temperature Sensor Volts Out Of Limit |
| 46 | — | — | Charging System Voltage Too High |
| 47 | — | — | Charging System Voltage Too Low |
| 51 | P0171 | — | Fuel System Lean |
| 52 | P0172 | — | Fuel System Rich |
| 53 | P0601 | — | Internal Controller Failure |
| 54 | P0340 | — | No Cam Signal At PCM |
| 62 | — | — | PCM Failure SRI Mile Not Stored |
| 63 | — | — | PCM Failure EEPROM Write Denied |
| 64 | P0422 | — | Catalytic Converter Efficiency Failure |
| 65 | P0551 | — | Power Steering Switch Failure |
|  | P1391 | — | Brake Switch Circuit |
| 66 | P1698 | — | No CCD Message From TCM |
|  | — | — | No CCD Message From Body Control Module{2} |
| 71 | P1496 | — | 5 Volt Supply Output Too Low |
| 77 | — | — | Speed Control Power Circuit |
| 1996–98 |  |  |  |
| 11 | P0320 | 28 | No Crankshaft Reference Signal At PCM |
|  | P0335 | — | No Crankshaft Reference Signal At PCM |
|  | P1390 | 85 | Timing Belt Skipped Tooth |
|  | P1391 | 9D | No Crank Reference Signal/Intermittent Loss Of CMP Or CKP |
|  | P1398 | BA | Misfire Adaptive Numerator At Limit |
| 12 | — | — | Battery Disconnected |
| 13 | P1297 | 27 | No Change In MAP From Start To Run |
| 14 | P0107 | 24 | MAP Sensor Voltage Too Low |
|  | P0108 | 25 | MAP Sensor Voltage Too High |
|  | P1296 | 87 | No 5 Volts To MAP Sensor |
|  | P1496 | 92 | 5 Volt Supply Output Too Low |
| 15 | P0500 | 23 | No VSS Signal |
| 16 | P0325 | 3B | Knock Sensor No. 1 Circuit |
|  | — | 53 | Knock Sensor No. 2 Circuit |
| 17 | P1281 | 21 | Engine Cold Too Long |
|  | P0125 | 80 | Closed Loop Temperature Not Reached |
| 21 | P0131 | 9B | Upstream O2S Shorted To Ground |
|  | P0132 | 3E | Upstream O2S Shorted To Power |
|  | P0133 | 66, CO | Upstream O2S Slow Response |
|  | P0134 | 20 | O2S Stays At Center |
|  | P0135 | 67 | Upstream O2S Heater Failure |
|  | P0137 | 9C | Downstream O2S Shorted To Ground |
|  | P0138 | 7E | Downstream O2S Shorted To Power |
|  | P0139 | — | Downstream O2S Stays At Center |
|  | P0140 | 81 | Downstream O2S Stays At Center |
|  | P0141 | 69 | Downstream O2S Heater Failure |
|  | P0151 | B5 | Front Bank Upstream HO2S Shorted To Ground |
|  | P0152 | 42 | Front Bank Upstream HO2S Shorted To Voltage |
|  | P0153 | 7A, C1 | Front Bank Upstream HO2S Slow Response |
|  | P0154 | 41 | Front Bank Upstream HO2S Stays At Center |
|  | P0155 | 7C | Front Bank Upstream HO2S Heater Failure |
|  | P0157 | B6 | Front Bank Downstream HO2S Shorted To Ground |
|  | P0158 | 7F | Front Bank Downstream HO2S Shorted To Voltage |
|  | P0160 | 82 | Front Bank Downstream HO2S Stays At Center |
|  | P0161 | 7D | Front Bank Downstream HO2S Heater Failure |
|  | P0175 | 7C | RH Bank Upstream HO2S Heater Failure |
| 22 | P0117 | 1E | ECT Sensor Voltage Too Low |
|  | P0118 | 1F | ECT Sensor Voltage Too High |
| 23 | P0112 | 39 | IAT Sensor Voltage Low |
|  | P0113 | 3A | IAT Sensor Voltage High |
| 24 | P0121 | 84 | TP Sensor Signal Does Not Agree With MAP |
|  | P0122 | 1A | TP Sensor Voltage Low |
|  | P0123 | 1B | TP Sensor Voltage High |
|  | P1295 | 88 | No 5 Volts To TP Sensor |
| 25 | P0505 | 19 | IAC Motor Circuits |
|  | P1294 | 8A | Target Idle Not Reached (+/-200 RPM) |
|  | P1299 | 91 | Vacuum Leak Found (IAC Fully Seated) |
| 27 | P0201–P0206 | 15, 14, 13, 3D, 45 & 46 | Injector Circuit No. 1–6 |
|  | P0208 | 80 | Closed Loop Temperature Not Reached |
| 31 | P0441 | 71 | EVAP Purge Flow Monitor Failure, Improper Purge Flow |
|  | P0442 | A0 | EVAP Monitor Small Leak |
|  | P0443 | 12 | EVAP Purge Solenoid Circuit |
|  | P0455 | A1 | EVAP Monitor Large Leak |
|  | P1486 | BB | EVAP Monitor Pinched Hose |
|  | P1494 | B8 | EVAP Emission Vent Solenoid Switch Or Mechanical Fault |
|  | P1495 | B7 | EVAP Emission Vent Or Leak Detection Pump Solenoid Circuit |
|  | P1498 | C5 | High Speed Radiator Fan Ground Control Relay Circuit |
| 32 | P0401 | 2E | EGR System Failure |
|  | P0403 | 11 | EGR Solenoid Circuit |
| 33 | P0645 | 10 | A/C Clutch Relay & Sensor Circuit |
| 34 | P1595 | 0F | Speed Control Vacuum Or Vent Solenoid Circuit |
|  | P1596 | 56 | Speed Control Switch Always High |
|  | P1597 | 57 | Speed Control Switch Always Low |
| 35 | P1487 | 8B | High Speed Radiator Fan Control Relay Circuit |
|  | P1489 | 5D | High Speed Condenser Fan Control Relay Circuit |
|  | P1490 | 5C | Low Speed Fan Control Relay Circuit |
|  | P1491 | 0E | Cooling Fan Relay Circuit |
| 37 | P0740 | 94 | TCC No RPM Drop At Lockup |
|  | P0743 | 0C | TCC Solenoid Circuits |
|  | P1899 | 72 | PNP Or Transaxle Switch Failure |
| 41 | P0622 | 0B | Alternator Field Improper Switching |
| 42 | P0220 | 65 | Fuel Pump Relay Control Circuit |
|  | P1389 | 2C | No MFI (ASD) Relay Output Voltage At PCM |
|  | P1388 | 0A | MFI (ASD) Relay Control Circuit |
|  | P0462 | 95 | Fuel Level Sending Unit Voltage Out Of Limits, Too Low |
|  | P0463 | 96 | Fuel Level Sending Unit Voltage Out Of Limits, Too High |
|  | P0460 | 97 | Fuel Level Sending Unit & Circuit, No Change Over Miles |
| 43 | P0300 | 6A | Multiple Cylinder Misfire |
|  | P0301–P0304 | 6B, C, D & E | Misfire Cylinder No. 1–4 |
|  | P0305 | AE | Misfire Cylinder No. 5 |
|  | P0306 | AF | Misfire Cylinder No. 6 |
|  | P0351 | 2B | Ignition Coil No. 1 Primary Circuit |
|  | P0352 | 2A | Ignition Coil No. 2 Primary Circuit |
|  | P0353 | 29 | Ignition Coil No. 3 Primary Circuit |
| 44 | P1478 | 47 | Battery Temperature Voltage Or Internal Temperature Sensor Input Voltage Out Of Limits |
|  | P1492 | 9A | Ambient/Battery Temperature Sensor Voltage High |
|  | P1493 | 99 | Ambient/Battery Temperature Sensor Voltage Low |
| 45 | P0700 | 89 | EATX Controller DTC Present |
| 46 | P1594 | 06 | Charging System Voltage Too High |
| 47 | P1682 | 05 | Charging System Voltage Too Low |
| 51 | P0171 | 77 | Rear Bank Fuel System Lean/Rich |
|  | P0174 | 79 | Front Bank Fuel System Lean/Rich |
| 52 | P0172 | 76 | Rear Bank Fuel System Lean/Rich |
|  | P0175 | 78 | Front Bank Fuel System Lean/Rich |
| 53 | P0600 | 44 | PCM Internal Fault Condition |
|  | P0601 | 02 | Internal Controller Failure |
|  | P0605 | — | Internal Controller Failure |
| 54 | P0340 | 01 | No Cam Signal At PCM |
| 55 | — | — | MIL Fault Display Complete |
| 61 | P0106 | 3C | BARO Out Of Range |
| 62 | P1697 | 30 | PCM Failure SRI Mile Not Stored |
| 63 | P1696 | 31 | PCM Failure EEPROM Write Denied |
| 64 | P0420 | 70 | Catalyst Efficiency Below Required Level |
|  | P0422 | 70 | Catalytic Converter Efficiency Failure |
| 65 | P0551 | 73 | PS Pressure Switch Failure Or Performance |
|  | P0703 | 98 | Brake Switch Performance Circuit |
|  | P1289 | 58 | Manifold Tune Valve Control Circuit |
| 66 | P1695 | 61 | No CCD Bus Messages From Body Controller Or BCM |
|  | P1698 | 60 | No CCD Messages From Body Or TCM |
| 71 | P1496 | 92 | 5 Volts Output From Regulator Low |
| 72 | P0420 | 70 | Catalyst Efficiency Below Required Level |
|  | P0432 | BA | Front Bank Catalytic Converter Efficiency Failure |
|  | P0432 | B4 | Catalytic Converter 2/1 Efficiency Failure |
| 77 | P1683 | 52 | S/C Power Relay Circuit |
| — | P0354 | 4C | Ignition Coil No. 4 Circuit |
| — | P0355 | 4D | Ignition Coil No. 5 Circuit |
| — | P0356 | D8 | Ignition Coil No. 6 Circuit |
| — | P0404 | D7 | EGR Position Sensor Performance |
| — | P0405 | D4 | EGR Position Sensor Voltage Low |
| — | P0406 | D5 | EGR Position Sensor Voltage High |
| — | P1195 | CO | Cat Mon Slow O2 RH Upstream |
| — | P1196 | C1 | Cat Mon Slow O2 LH Upstream |
| — | P1282 | 65 | Fuel Pump Relay Control Circuit |
| — | P1288 | D9 | Short Run Valve Tuning Circuit |
| — | P1598 | 5A | A/C Pressure Sensor Voltage Too High |
| — | P1599 | 5B | A/C Pressure Sensor Voltage Too Low |
| — | P1685 | E8 | ECM Received Invalid SKIM Key |
| — | P1686 | E2 | No SKIM Bus Message At PCI |
| — | P1687 | E1 | No MIC Bus Message At PCI |

## 95–98 Lt Duty Truck, Van & Jeep w/OBD II

| OBDII | Hex |  |
| --- | --- | --- |
| — | 00 | DTC Error |
| — | 5A | A/C Pressure Voltage Too High |
| — | 5B | A/C Pressure Voltage Too Low |
| — | 61 | No CCD Message From BCM |
| P0100 | — | Volume Air Flow Circuit Malfunction |
| P0105 | — | Barometric Pressure Circuit Malfunction |
| P0106 | 3C | MAP Sensor Reading Below Acceptable Value |
| P0107 | 24 | MAP Sensor Voltage Too Low |
| P0108 | 25 | MAP Sensor Voltage Too High |
| P0110 | — | Intake Air Temperature Circuit Malfunction |
| P0112 | 39 | Intake Air Temperature Sensor Voltage Low |
| P0113 | 3A | Intake Air Temperature Sensor Voltage High |
| P0115 | — | Engine Coolant Temperature Circuit Malfunction |
| P0117 | 1E | ECT Sensor Voltage Too Low |
| P0118 | 1F | ECT Sensor Voltage Too High |
| P0120 | — | Throttle Position Circuit Malfunction |
| P0121 | 84 | TP Sensor Voltage Does Not Agree With MAP |
| P0122 | 1A | TP Sensor Voltage Low |
| P0123 | 1B | TP Sensor Voltage High |
| P0125 | 80 | Excessive Time To Enter Closed Loop Fuel Control{1} |
| P0128 | 80 | Closed Loop Temp Not Reached{2} |
| P0129 | C2 | Bank 1 HO2S 2 Slow Switching During Catalyst Monitor Test |
| P0130 | — | Bank 1 Heated Oxygen Sensor 1 Circuit Malfunction |
| P0131 | 9B | Upstream O2S Voltage Shorted To Ground |
| P0132 | 3E | Upstream O2S Shorted To Voltage |
| P0133 | 66, CO | Upstream O2S Slow Response |
| P0134 | 20 | Upstream O2S Stays At Center |
| P0135 | 67 | Bank 1 HO2S 1 Heater Circuit Malfunction |
| P0136 | — | Bank 1 HO2S 2 Circuit Malfunction |
| P0137 | 9C | Downstream O2S Voltage Shorted To Ground |
| P0138 | — | Downstream O2S Shorted To Voltage |
| P0139 | 68 | Downstream O2S Unable To Switch Rich/Lean |
| P0140 | 81 | Downstream O2S Stays At Center |
| P0141 | 69 | Bank 1 HO2S 2 Heater Circuit Malfunction |
| P0147 | BD | HO2S Heater Element Malfunction |
| P0151 | B5 | RH Upstream O2S Voltage Shorted To Ground |
| P0152 | 42 | RH Upstream O2S Voltage Shorted To B+ |
|  | 66 | HO2S Slow Switching Frequency Response |
| P0153 | C1, 7A | Bank 2 HO2S 1 Slow Switching |
| P0154 | 41 | RH Upstream O2S Stays At Center |
| P0155 | 7C | RH Upstream O2S Heater Failure |
| P0157 | B6 | RH Downstream O2S Shorted To Ground |
| P0158 | 7F | RH Downstream O2S Shorted To Voltage |
| P0159 | 7B | HO2S Bank 1, Sensor 2 Slow Switching Frequency Response |
| P0160 | — | RH Downstream O2S Stays At Center |
| P0161 | 7D | RH Downstream O2S Heater Failure |
| P0162 | 05 | Charging System Voltage Too Low |
| P0170 | — | Fuel Trim Malfunction, Bank 1 |
| P0171 | 77 | Fuel System Lean |
| P0172 | 76 | Fuel System Rich |
| P0174 | 79 | RH Downstream Fuel System Lean |
| P0175 | 78 | LH Downstream Fuel System Rich |
| P0201–P0208 | 15, 14 & 13 | Injector Circuit Malfunction, Nos. 1–8 |
| P0204 | 3D | Injector No. 4 Control Circuit |
| P0205 | 45 | Injector No. 5 Control Circuit |
| P0206 | 46 | Injector No. 6 Control Circuit |
| P0207 | 4F | Injector No. 7 Control Circuit |
| P0208 | 50 | Injector No. 8 Control Circuit |
| P0300 | 6A | Random Misfire Detected |
| P0301–P0304 | 6B, C, D & E | Misfire Detected, Cylinder Nos. 1–4 |
| P0305 | AE | Misfire Detected, Cylinder No. 5 |
| P0306 | AF | Misfire Detected, Cylinder No. 6 |
| P0307 | B0 | Misfire Detected, Cylinder No. 7 |
| P0308 | B1 | Misfire Detected, Cylinder No. 8 |
| P0309 | B2 | Misfire Detected, Cylinder No. 9 |
| P0310 | B3 | Misfire Detected, Cylinder No. 10 |
| P0320 | 28 | No Crank Reference Signal At PCM |
| P0325 | — | Knock Sensor 1 Circuit Malfunction |
| P0335 | — | Crankshaft Position Sensor Circuit Malfunction |
| P0340 | 01 | Camshaft Position Sensor Circuit Malfunction |
| P0351 | 2B | Ignition Coil No. 1 Primary Circuit |
| P0352 | 2A | Ignition Coil No. 2 Primary Circuit |
| P0353 | 29 | Ignition Coil No. 3 Primary Circuit |
| P0354 | 4C | Ignition Coil No. 4 Primary Circuit |
| P0355 | 4D | Ignition Coil No. 5 Primary Circuit |
| P0400 | 8C | Exhaust Gas Recirculation (EGR) Flow Malfunction |
| P0401 | 2E | EGR System Failure |
| P0403 | — | EGR Solenoid Malfunction |
| P0420 | 70 | Catalytic Efficiency Below Threshold, Bank 1 |
| P0421 | — | Warm Up Catalyst Efficiency Below Threshold (Bank 1) (California) |
| P0432 | B4 | Catalytic Converter Efficiency Failure |
| P0440 | — | Evaporative Emission Control System Malfunction |
| P0441 | 71 | EVAP Purge Flow Monitor Failure |
| P0442 | A0 | EVAP System Small Leak |
| P0443 | 12 | Evaporative Emission Control System Purge Control Valve Circuit Malfunction |
| P0455 | A1 | EVAP System Large Leak |
| P0460 | 97 | No Fuel Lever Sender Movement Detected |
| P0462 | 95 | Fuel Level Sender Voltage Too Low |
| P0463 | 96 | Fuel Level Sender Voltage Too High |
| P0500 | 23 | Vehicle Speed Sensor Malfunction |
| P0505 | 19 | Idle Control System Malfunction |
| P0510 | — | Closed TP Switch Malfunction |
| P0551 | 73 | Power Steering Switch Failure |
| P0600 | 44 | PCM Internal Fault SPI Communications |
| P0601 | 02 | PCM Internal Fault Internal Controller Failure |
| P0622 | 0B | Alternator Field Improper Switching |
| P0645 | 10 | A/C Clutch Relay Circuit |
| P0700 | 89 | Transaxle Or Transmission Control System Malfunction |
| P0703 | 98 | Brake Switch Stuck, Pressed Or Released |
| P0705 | — | Transmission Range Sensor Circuit Malfunction (PRNDL Input) |
| P0710 | — | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P0711 | A4 | No Trans Temp Sensor Rise After Start |
| P0712 | 4A | Trans Temp Sensor Voltage Too Low |
| P0713 | 4B | Trans Temp Sensor Voltage Too High |
| P0720 | A6 | Low Output Speed Sensor RPM Above 15 M.P.H. |
| P0740 | 94 | TCC, No RPM Drop At Lockup |
| P0743 | 0C | TCC Solenoid/Trans Relay Circuits (3 Speed Auto RH Trans Only) |
| P0748 | AB | Governor Pressure Sol/Control Trans Relay Circuits |
| P0751 | BC | OD Off Switch Input Too Low For More Than 5 Minutes |
| P0753 | 32 | Trans 3–4 Shift Sol/Trans Relay Circuits |
| P0783 | A5 | 3–4 Shift Solenoid, No RPM Drop At 3–4 Shift |
| P1103 | — | Turbocharger Wastegate Actuator Malfunction |
| P1104 | — | Turbocharger Wastegate Solenoid Malfunction |
| P1105 | — | Fuel Pressure Solenoid Malfunction |
| P1195 | CO | Bank 1 HO2S 1 Slow Switching During Catalyst Monitor Test |
| P1196 | C1 | Bank 2 HO2S 1 Slow Switching During Catalyst Monitor Test |
| P1197 | C2 | Bank 1 HO2S 2 Slow Switching During Catalyst Monitor Test |
| P1281 | 21 | Engine Cold Too Long |
| P1282 | 65 | Fuel Pump Relay Control Circuit |
| P1290 | BE | CNG System Pressure Too High |
| P1294 | 8A | Target Idle Not Reached |
| P1296 | 87 | No 5 Volt To MAP Sensor |
| P1297 | 27 | No Change In MAP From Start To Run |
| P1299 | — | Vacuum Leak Found (IAC Fully Seated) |
| P1300 | — | Ignition Timing Adjustment Circuit Malfunction |
| P1388 | 0A | Auto Shutdown (ASD) Relay Circuit |
| P1389 | 2C | No Auto Shutdown (ASD) Relay Output Voltage At PCM |
| P1390 | — | Timing Belt Skipped One Or More Notches |
| P1391 | 9D | Camshaft Or Crankshaft Position Sensor Intermittent Loss |
| P1398 | BA | Crankshaft Position Sensor Target Windows Excessive Variation |
| P1400 | — | Manifold Differential Pressure Sensor Circuit Malfunction |
| P1486 | BB | EVAP Leak Monitor Hose Pinched |
| P1489 | 56 | High Speed Cooling Fan Relay Circuit Open Or Short |
| P1490 | 57 | Low Speed Cooling Fan Relay Circuit Open Or Short |
| P1491 | 0E | Radiator Fan Control Relay Circuit (2.5L Only) |
| P1492 | 9A | Battery Temperature Sensor Voltage Too High |
| P1493 | 99 | Battery Temperature Sensor Voltage Too Low |
| P1494 | B8 | Leak Detection Pump Pressure Switch |
| P1495 | B7 | Leak Detection Pump Solenoid Circuit |
| P1496 | 92 | 5 Volt Output Low |
| P1500 | — | Alternator FR Terminal Circuit Malfunction |
| P1594 | 06 | Charging System Voltage Too High |
| P1595 | 0F | Speed Control Vacuum Or Vent Solenoid Circuits |
| P1596 | 56 | Speed Control Input Voltage Too High |
| P1597 | 57 | Speed Control Input Voltage Too Low |
| P1683 | 52 | Speed Control Power Relay Feed Circuits |
| P1693 | DE | No Engine Bus Msgs;. |
| P1694 | DF | Fault In Comp Module |
| P1696 | 31 | PCM Failure EEPROM Write Denied{1} |
| P1697 | 30 | PCM Failure SRI Mile Not Stored |
| P1698 | 31 | PCM Failure EEPROM Write Denied{2} |
| P1698 | 60 | No CCD Messages From TCM{1} |
| P1715 | — | PG Assembly Malfunction |
| P1750 | — | Solenoid Assembly Malfunction |
| P1756 | 8D | Governor Sensor Input Not At 10–25 PSI On Request |
| P1757 | 8E | Governor Pressure Above 3 PSI When Request Is 0 PSI |
| P1762 | A9 | Governor Pressure Sensor Offset Improper Voltage |
| P1763 | A8 | Governor Pressure Sensor Voltage Too High |
| P1764 | A7 | Governor Pressure Sensor Voltage Too Low |
| P1765 | AD | Trans 12 Volt Supply Relay Control Circuit |
| P1899 | 72 | PNP Switch Failure |

{1}—Except 1998 Caravan, Town & Country, Voyager & Wrangler.
{2}—1998 Caravan, Town & Country, Voyager & Wrangler.

## 99–01 Exc. Avenger & Sebring Coupe

| P0030 | 1/1 O2S Heater Relay Circuit{1} |
| --- | --- |
| P0036 | 1/2 O2S Heater Relay Circuit{1} |
| P0106 | Barometric Pressure Out Of Range |
| P0107 | MAP Sensor Voltage Too Low |
| P0108 | MAP Sensor Voltage Too High |
| P0112 | IAT Sensor Voltage Low |
| P0113 | IAT Sensor Voltage High |
| P0116 | ECT Sensor Malfunction |
| P0117 | ECT Sensor Voltage Too Low |
| P0118 | ECT Sensor Voltage Too High |
| P0121 | TP Sensor Voltage Does Not Agree With MAP |
| P0122 | TP Sensor Voltage Low |
| P0123 | TP Sensor Voltage High |
| P0125 | Closed Loop Temperature Not Reached |
| P0130 | 1/1 O2S Sensor Heater relay Circuit |
| P0131 | 1/1 O2S Shorted To Ground |
| P0132 | 1/1 O2S Shorted To Voltage |
| PO133 | 1/1 O2S Slow Response |
| P0134 | 1/1 O2S Stays At Center |
| P0135 | 1/1 O2S Heater Failure |
| P0136 | 1/2 O2S Circuit Malfunction |
| P0137 | 1/2 O2S Shorted To Ground |
| P0138 | 1/2 O2S Shorted To Voltage |
| P0139 | 1/2 O2S Slow Response |
| P0140 | 1/2 O2S Stays At Center |
| P0141 | 1/2 O2S Heater Failure |
| P0143 | 1/3 O2S Shorted To Ground |
| P0144 | 1/3 O2S Shorted To Voltage |
| P0145 | 1/3 O2S Slow Response |
| P0146 | 1/3 O2S Stays At Center |
| P0147 | 1/3 O2S Heater Failure |
| P0151 | 2/1 O2S Shorted To Ground |
| P0152 | 2/1 O2S Voltage Shorted To Voltage |
| P0153 | 2/1 O2S Slow Response |
| P0154 | 2/1 O2S Stays At Center |
| P0155 | 2/1 O2S Heater Failure |
| P0157 | 2/2 O2S Shorted To Ground |
| P0158 | 2/2 O2S Shorted To Voltage |
| P0159 | 2/2 O2S Slow Response |
| P0160 | 2/2 O2S Stays At Center |
| P0161 | 2/2 O2S Heater Failure |
| P0165 | Starter Relay Control Circuit |
| P0171 | 1/1 Fuel System Lean |
| P0172 | 1/1 Fuel System Rich |
| P0174 | 2/1 Fuel System Lean |
| P0175 | 2/1 Fuel System Rich |
| P0178 | Water In Fuel Sensor Voltage Too High |
| P0179 | Flex Fuel Sensor Volts Too High |
| P0181{1} | Fuel Injection Pump Failure |
| P0182 | CNG Temperature Sensor Voltage Too Low |
| P0183 | CNG Temperature Sensor Voltage Too High |
| P0201 | Injector Circuit Malfunction, No. 1 |
| P0202 | Injector Circuit Malfunction, No. 2 |
| P0203 | Injector Circuit Malfunction, No. 3 |
| P0204 | Injector Circuit Malfunction, No. 4 |
| P0205 | Injector Circuit Malfunction, No. 5 |
| P0206 | Injector Circuit Malfunction, No. 6 |
| P0207 | Injector Circuit Malfunction, No. 7 |
| P0208 | Injector Circuit Malfunction, No. 8 |
| P0209 | Injector Circuit Malfunction, No. 9 |
| P0210 | Injector Circuit Malfunction, No. 10 |
| P0300 | Random Misfire Detected |
| P0301 | Misfire Detected, Cylinder No. 1 |
| P0302 | Misfire Detected, Cylinder No. 2 |
| P0303 | Misfire Detected, Cylinder No. 3 |
| P0304 | Misfire Detected, Cylinder No. 4 |
| P0305 | Misfire Detected, Cylinder No. 5 |
| P0306 | Misfire Detected, Cylinder No. 6 |
| P0307 | Misfire Detected, Cylinder No. 7 |
| P0308 | Misfire Detected, Cylinder No. 8 |
| P0309 | Misfire Detected, Cylinder No. 9 |
| P0310 | Misfire Detected, Cylinder No. 10 |
| P0320 | No Crank Reference Signal At PCM |
| P0325 | KS 1 Circuit Malfunction |
| P0330 | KS 2 Circuit Malfunction |
| P0335 | CP Sensor Circuit Malfunction |
| P0340 | No CAM Signal At PCM |
| P0350 | Ignition Coil Draws Too Much Current |
| P0351 | Ignition Coil No. 1 Primary Circuit |
| P0352 | Ignition Coil No. 2 Primary Circuit |
| P0353 | Ignition Coil No. 3 Primary Circuit |
| P0354 | Ignition Coil No. 4 Primary Circuit |
| P0355 | Ignition Coil No. 5 Primary Circuit |
| P0356 | Ignition Coil No. 6 Primary Circuit |
| P0357 | Ignition Coil No. 7 Primary Circuit |
| P0358 | Ignition Coil No. 8 Primary Circuit |
| P0401 | EGR System Failure |
| P0403 | EGR Solenoid Circuit |
| P0404 | EGR Position Sensor Rationality |
| P0405 | EGR Position Sensor Volts Too Low |
| P0406 | EGR Position Sensor Volts Too High |
| P0412 | Secondary Air Solenoid Circuit |
| P0420 | 1/1 Catalytic Converter Efficiency |
| P0432 | 1/2 Catalytic Converter Efficiency |
| P0441 | EVAP Purge Flow Monitor Failure |
| P0442 | EVAP Leak Monitor Medium Leak Detected |
| P0443 | EVAP System Purge Solenoid Circuit |
| P0455 | EVAP Leak Monitor Large Leak Detected |
| P0456 | EVAP Leak Monitor Small Leak Detected |
| P0460 | Fuel Level Unit No Change Over Miles |
| P0461 | Fuel Level Unit No Change Over Time |
| P0462 | Fuel Level Unit Volts Too Low |
| P0463 | Fuel Level Unit Volts Too High |
| P0500 | No VSS Signal |
| P0505 | IAC Motor Circuits |
| P0522 | Oil Pressure Sens Low |
| P0523 | Oil Pressure Sens High |
| P0551 | Power Steering Switch Failure |
| P0600 | PCM Internal Fault SPI Communications |
| P0601 | PCM Internal Fault Internal Controller Failure |
| P0604 | TCM RAM Self Test Fault Detected |
| P0605 | TCM ROM Self Test Fault Detected |
| P0622 | Alternator Field No Switching Properly |
| P0645 | A/C Clutch Relay Circuit |
| P0700 | Transaxle Or Transmission Control System Malfunction |
| P0703 | Brake Switch Stuck |
| P0711 | Transmission Temperature Sensor Circuit No Temperature Rise After Start |
| P0712 | Transmission Temperature Sensor Voltage Too Low |
| P0713 | Transmission Temperature Sensor Voltage Too High |
| P0720 | Low Output SPD Sensor RPM, Above 15 MPH |
| P0740 | TCC, No RPM Drop At Lockup |
| P0743 | TCC Solenoid/Trans. Relay Circuits |
| P0748 | Governor Pressure Solenoid Control/Trans. Relay Circuits |
| P0751 | O/D Switch Pressure (Lo) More Than 5 Minutes |
| P0753 | Trans. 3-4 Shift Solenoid/Trans. Relay Circuits |
| P0756 | AW4 Shift Solenoid b (2-3) Functional Failure |
| P0783 | 3-4 Shift Solenoid No RPM Drop At Lockup |
| P0801 | Reverse Gear Lockout Circuit Open Or Short Circuit |
| P1192 | IAT Circuit Low |
| P1193 | IAT Circuit High |
| P1195 | 1/1 O2S Slow During Catalyst Monitor |
| P1196 | 2/1 O2S Slow During Catalyst Monitor |
| P1197 | 1/2 O2S Slow During Catalyst Monitor |
| P1198 | Radiator Temperature Sensor Volts Too High |
| P1199 | Radiator Temperature Sensor Volts Too Low |
| P1281 | Engine Cold Too Long |
| P1282 | Fuel Pump Relay Control Circuit |
| P1288 | Intake Manifold Short Runner Solenoid Circuit |
| P1289 | Manifold Tune Valve Solenoid Circuit |
| P1290 | CNG Fuel System Pressure Too High |
| P1291 | No Temperature Rise Seen From Intake Heaters |
| P1292 | CNG Pressure Sensor Voltage Too High |
| P1293 | CNG Pressure Sensor Voltage Too Low |
| P1294 | Target Idle Not Reached |
| P1295 | No 5 Volts To TP Sensor |
| P1296 | No 5 Volt To MAP Sensor |
| P1297 | No Change In MAP From Start To Run |
| P1298 | Lean Operation At Wide Open Throttle |
| P1299 | Vacuum Leak Found (IAC Fully Seated) |
| P1388 | Auto Shutdown Relay Control Circuit |
| P1389 | No ASD Relay Output Voltage At PCM |
| P1390 | Timing Belt Skipped One Or More Notches |
| P1391 | Camshaft Or Crankshaft Position Sensor Intermittent Loss |
| P1398 | Misfire Adaptive Numerator At Limit |
| P1399 | Wait To Start Lamp Circuit |
| P1403 | Loss Of 5 Volts Feed To EGR Position Sensor |
| P1476 | Too Little Secondary Air |
| P1477 | Too Much Secondary Air |
| P1478 | Battery Temperature Sensor Volts Out Of Limit |
| P1479 | Transmission Fan Relay Circuit |
| P1480 | PCV Solenoid Circuit |
| P1481 | EATX RPM Pulse Generator Signal For Misfire Detection Does Not Correlate With Expected Value |
| P1482 | Catalyst Temperature Sensor Circuit Shorted Low |
| P1483 | Catalyst Temperature Sensor Circuit Shorted High |
| P1484 | Catalytic Converter Overheat Detected |
| P1485 | Air injection Solenoid Circuit |
| P1486 | EVAP Leak Monitor Hose Pinched |
| P1487 | High Speed Fan Control Relay Circuit |
| P1488 | Auxiliary 5 Volt Supply Output Too Low |
| P1489 | High Speed Cooling Fan Relay Circuit |
| P1490 | Low Speed Cooling Fan Relay Circuit |
| P1491 | Radiator Fan Control Relay Circuit |
| P1492 | Battery Temperature Sensor Voltage Too High |
| P1493 | Battery Temperature Sensor Voltage Too Low |
| P1494 | Leak Detection Pump Pressure Switch Or Mechanical Failure |
| P1495 | Leak Detection Pump Solenoid Circuit |
| P1496 | 5 Volt Output Too Low |
| P1498 | High Speed Radiator Fan Ground Control Relay Circuit |
| P1594 | Charging System Voltage Too High |
| P1595 | Speed Control Solenoid Circuits |
| P1596 | Speed Control Switch Always High |
| P1597 | Speed Control Switch Always Low |
| P1598 | A/C Pressure Sensor Volts Too High |
| P1599 | A/C Pressure Sensor Volts Too Low |
| P1680 | Clutch Released Switch Circuit |
| P1681 | No Instrument Panel Cluster CCD/J1850 Messages Received |
| P1682 | Charging System Voltage Too Low |
| P1683 | Speed Control Power relay Or Speed Control 12 Volt Driver Circuit |
| P1684 | Battery Has Been Disconnected Within Last 50 Starts |
| P1685 | SKIM Invalid Key |
| P1686 | No SKIM BUS Messages Received |
| P1687 | No MIC BUS Messages |
| P1693 | DTC Detected In Companion Module |
| P1694 | Fault In Companion Module |
| P1695 | No CDD/J1850 Message From Body Control Module |
| P1696 | PCM Failure EEPROM Write Denied |
| P1697 | PCM Failure SRI Mile Not Stored |
| P1698 | No CCD/J1850 Messages From TCM |
| P1719 | Skip Shift Solenoid |
| P1740{1} | A Rationality Error Has Been Detected In Either TCC Solenoid Or O/D Solenoid Systems |
| P1756 | GOV Pressure Not Equal To Target At 15-20 Psi. |
| P1757 | GOV Pressure Not Equal To Target At 15-20 Psi. |
| P1762 | GOV Pressure Sensor Offset Volts Too Low Or High |
| P1763 | GOV Pressure Sensor Volts Too High |
| P1764 | GOV Pressure Sensor Volts Too Low |
| P1765 | Transmission 12 Volt Supply Relay Control Circuit |
| P1899 | Park/Neutral Switch Failure |

{1}—1999–2001 models only

## 00 Avenger & Sebring Coupe

| DRB III | General Scan Tool | MIL |  |
| --- | --- | --- | --- |
| 01 (01) | P0340 | 54 | No Cam Signal At PCM |
| 02 (02) | P0601 | 53 | Internal Controller Failure |
| 11 (17) | P0403 | 32 | EGR Solenoid Circuit |
| 12 (18) | P0443 | 31 | EVAP Solenoid Circuit |
| 13 (19) | P0203 | 27 | Injector No. 3 Control Circuit |
| 14 (20) | P0202 | 27 | Injector No. 2 Control Circuit |
| 15 (21) | P0201 | 27 | Injector No. 1 Control Circuit |
| 19 (25) | P0505 | 25 | IAC Motor Circuits |
| 1A (26) | P0122 | 24 | TP Sensor Voltage Low |
| 1B (27) | P0123 | 24 | TP Sensor Voltage High |
| 1E (30) | P0117 | 22 | ECT Sensor Voltage Low |
| 1F (31) | P0118 | 22 | ECT Sensor Voltage High |
| 20 (32) | P0134 | 21 | Upstream HO2S Stays At Center |
| 23 (35) | P0500 | 15 | No VSS Signal |
| 24 (36) | P0107 | 14 | MAP Sensor Voltage Too Low |
| 25 (37) | P0108 | 14 | MAP Sensor Voltage Too High |
| 27 (39) | P1297 | 13 | No Change In MAP From Start To Run |
| 2B (43) | P0351 | 43 | Ignition Coil No. 1 Primary Circuit |
| 2E (46) | P0401 | 32 | EGR System Failure |
| 30 (48) | P1697 | — | PCM Failure, SRI Mile Not Stored |
| 31 (49) | P1696 | — | PCM Failure, EEPROM Write Denied |
| 39 (57) | P0112 | 23 | IAT Sensor Voltage Low |
| 3A (58) | P0113 | 23 | IAT Sensor Voltage High |
| 3C (60) | P0106 | 61 | BARO Pressure Out Of Range |
| 3D (61) | P0204 | 27 | Injector No. 4 Control Circuit |
| 3E (62) | P0132 | 21 | Left Bank Upstream HO2S Shorted To B+ (V6) |
| 41 (65) | P0154 | 21 | Right Bank Upstream HO2S Stays At Center |
| 42 (66) | P0152 | 21 | Right Bank Upstream HO2S Shorted To Voltage |
| 44 (68) | P0600 | 53 | PCM Failure, SPI Communications |
| 45 (69) | P0205 | 27 | Injector No. 5 Control Circuit |
| 46 (70) | P0206 | 27 | Injector No. 6 Control Circuit |
| 47 (71) | — | 44 | Battery Temperature Volts Out Of Limit |
| 5C (92) | P1490 | 35 | Low Speed Fan Control Relay Circuit |
| 5D (93) | P1489 | 35 | High Speed Fan Control Relay Circuit |
| 60 (96) | P1698 | — | No CCD Messages From TCM, A/T |
| 66 (102) | P0133 | 21 | Left Bank Upstream HO2S Response (V6) |
| 67 (103) | P0135 | 21 | Left Bank Upstream HO2S Heater Failure (V6) |
| 69 (105) | P0141 | 21 | Left Bank Downstream HO2S Heater Failure (V6) |
| 6A (106) | P0300 | 43 | Multiple Cylinder Misfire |
| 6B (107) | P0301 | 43 | Cylinder No. 1 Misfire |
| 6C (108) | P0302 | 43 | Cylinder No. 2 Misfire |
| 6D (109) | P0303 | 43 | Cylinder No. 3 Misfire |
| 6E (110) | P0304 | 43 | Cylinder No. 4 Misfire |
| 70 (112) | P0420 | 64 | Left Bank Catalytic Converter Efficiency Failure (V6) |
| 71 (113) | P0441 | 31 | EVAP Purge Flow Monitor Failure |
| 72 (114) | P1899 | 37 | PNP Switch Failure |
| 73 (115) | P0551 | — | PSP Switch Failure |
| 76 (118) | P0172 | 52 | Left Bank Fuel System Rich |
| 77 (119) | P0171 | 51 | Left Bank Fuel System Lean |
| 78 (120) | P0175 | 52 | Right Bank Fuel System Rich |
| 79 (121) | P0174 | 51 | Right Bank Fuel System Lean |
| 7A (122) | P0153 | 21 | Right Bank Upstream HO2S Slow Response |
| 7C (124) | P0155 | 21 | Right Bank Upstream HO2S Heater Failure |
| 7D (125) | P0161 | 21 | Right Bank Downstream HO2S Heater Failure |
| 7E (126) | P0138 | 21 | Left Bank Downstream HO2S Shorted To Voltage (V6) |
| 7F (127) | P0158 | 21 | Right Bank Downstream HO2S Shorted To Voltage |
| 80 (128) | P0125 | 17 | Closed Loop Temperature Not Reached |
| 81 (129) | P0140 | 21 | Left Bank Downstream HO2S Stays At Center |
| 82 (130) | P0160 | 21 | Right Bank Downstream HO2S Stays At Center |
| 84 (132) | P0121 | 24 | TP Sensor Voltage Disagrees With MAP |
| 87 (135) | P1296 | 14 | No 5 Volts To MAP Sensor |
| 88 (136) | P1295 | 24 | No 5 Volts To TP Sensor |
| 89 (137) | P0700 | 45 | EATX Controller DTC Present |
| 8A (138) | P1294 | 25 | Target Idle Not Reached (±200 RPM) |
| 8B (139) | P1487 | 35 | High Speed Radiator Fan Relay Circuit |
| 92 (146) | P1496 | 14 | 5 Volt Supply Output Too Low |
| 98 (152) | P0703 | 65 | Stop Lamp Switch Circuit |
| 99 (153) | P1493 | 44 | Battery Temperature Sensor Voltage Too Low |
| 9A (154) | P1492 | 44 | Battery Temperature Sensor Voltage Too High |
| 9B (155) | P0131 | 21 | Left Bank Upstream HO2S Voltage Grounded |
| 9C (156) | P0137 | 21 | Left Bank Downstream HO2S Voltage Grounded |
| 9D (157) | P1391 | 11 | Intermittent Loss Of CMP Or CKP |
| A0 (160) | P0442 | 31 | EVAP System Small Leak |
| A1 (161) | P0455 | 31 | EVAP System Large Leak |
| AE (174) | P0305 | 43 | Cylinder No. 5 Misfire |
| AF (175) | P0306 | 43 | Cylinder No. 6 Misfire |
| B4 (180) | P0432 | 72 | Right Bank Catalytic Converter Failure |
| B5 (181) | P0151 | 21 | Right Bank Upstream HO2S Shorted To Ground |
| B6 (182) | P0157 | 21 | Right Bank Downstream HO2S Shorted To Ground |
| B7 (183) | P1495 | 31 | LDT Solenoid Electrical Malfunction |
| B8 (184) | P1494 | 31 | LDP Switch/Mechanical Fault |
| BA (186) | P1398 | 11 | Misfire Adaptive Numerator At Limit |
| BB (187) | P1486 | 31 | EVAP Leak Monitor Pinched Hose Found |
| C0 (192) | P1195 | 21 | Slow Upstream HO2S Circuit During Catalyst Monitor |
| C1 (193) | P1196 | 21 | Slow Right Bank Upstream HO2S Circuit During Catalyst Monitor |
| C5 (197) | P1498 | 31 | High Speed Radiator Fan Ground Control Relay Circuit |

## 99–01 Light Duty Truck, Van & Jeep w/OBD II

| P0030 | 1/1 O2S Heater Relay Circuit |
| --- | --- |
| P0036 | 1/2 O2S Heater Relay Circuit |
| P0106 | Barometric Pressure Out Of Range |
| P0107 | MAP Sensor Voltage Too Low |
| P0108 | MAP Sensor Voltage Too High |
| P0112 | IAT Sensor Voltage Low |
| P0113 | IAT Sensor Voltage High |
| P0116 | ECT Sensor Malfunction |
| P0117 | ECT Sensor Voltage Too Low |
| P0118 | ECT Sensor Voltage Too High |
| P0121 | TP Sensor Voltage Does Not Agree With MAP |
|  | Accelerator Position Sensor Signal Voltage Too Low |
| P0122 | TP Sensor Voltage Low |
| P0123 | TP Sensor Voltage High |
|  | Accelerator Position Sensor Signal Voltage Too High |
| P0125 | Closed Loop Temperature Not Reached Or Engine Does Not Reach Operating Temperature |
| P0131 | 1/1 O2S Shorted To Ground |
| P0132 | 1/1 O2S Shorted To Voltage |
| P0133 | 1/1 O2S Slow Response |
| P0134 | 1/1 O2S Stays At Center |
| P0135 | 1/1 O2S Heater Failure |
| P0137 | 1/2 O2S Shorted To Ground |
| P0138 | 1/2 O2S Shorted To Voltage |
| P0139 | 1/2 O2S Slow Response |
| P0140 | 1/2 O2S Stays At Center |
| P0141 | 1/2 O2S Heater Failure |
| P0143 | 1/3 O2S Shorted To Ground |
| P0144 | 1/3 O2S Shorted To Voltage |
| P0145 | 1/3 O2S Slow Response |
| P0146 | 1/3 O2S Stays At Center |
| P0147 | 1/3 O2S Heater Failure |
| P0151 | 2/1 O2S Shorted To Ground |
| P0152 | 2/1 O2S Voltage Shorted To Voltage |
| P0153 | 2/1 O2S Slow Response |
| P0154 | 2/1 O2S Stays At Center |
| P0155 | 2/1 O2S Heater Failure |
| P0157 | 2/2 O2S Shorted To Ground |
| P0158 | 2/2 O2S Shorted To Voltage |
| P0159 | 2/2 O2S Slow Response |
| P0160 | 2/2 O2S Stays At Center |
| P0161 | 2/2 O2S Heater Failure |
| P0165 | Starter Relay Control Circuit |
| P0168 | Decreased Engine Performance Due To High Injection Pump Fuel Temperature |
| P0171 | 1/1 Fuel System Lean |
| P0172 | 1/1 Fuel System Rich |
| P0174 | 2/1 Fuel System Lean |
| P0175 | 2/1 Fuel System Rich |
| P0176 | Loss Of Flex Fuel Calibration Signal |
| P0177 | Water In Fuel |
| P0178 | Water In Fuel Sensor Voltage Too High |
|  | Flex Fuel Sensor Voltage Too Low |
| P0179 | Flex Fuel Sensor Volts Too High |
| P0181 | Fuel Injection Pump Failure |
| P0182 | CNG Temperature Sensor Voltage Too Low |
| P0183 | CNG Temperature Sensor Voltage Too High |
| P0201 | Injector Circuit Malfunction, No. 1 |
| P0202 | Injector Circuit Malfunction, No. 2 |
| P0203 | Injector Circuit Malfunction, No. 3 |
| P0204 | Injector Circuit Malfunction, No. 4 |
| P0205 | Injector Circuit Malfunction, No. 5 |
| P0206 | Injector Circuit Malfunction, No. 6 |
| P0207 | Injector Circuit Malfunction, No. 7 |
| P0208 | Injector Circuit Malfunction, No. 8 |
| P0209 | Injector Circuit Malfunction, No. 9 |
| P0210 | Injector Circuit Malfunction, No. 10 |
| P0215 | Fuel Injection Pump Control Circuit |
| P0216 | Fuel Injection Pump Timing Failure |
| P0217 | Decreased Engine Performance Due To Engine Overheat Condition |
| P0219 | CKP Sensor Overspeed Condition |
| P0222 | Idle Validation Signals Both Low |
| P0223 | Idle Validation Signals Both High (Above 5 Volts) |
| P0230 | Transfer Pump (Lift Pump) Circuit Out Of Range |
| P0232 | Fuel Shut-Off Signal Voltage Too High |
| P0234 | Turbo Boost Limit Exceeded |
| P0236 | MAP Sensor Too High Too Long |
| P0237 | MAP Sensor Voltage Too Low |
| P0238 | MAP Sensor Voltage Too High |
| P0251 | Fuel Injection Pump Mechanical Failure Fuel valve Feedback Circuit |
| P0253 | Fuel Injection Pump Fuel Valve Open Circuit |
| P0254 | Fuel Injection Pump Fuel Valve Current Too High |
| P0300 | Random Misfire Detected |
| P0301 | Misfire Detected, Cylinder No. 1 |
| P0302 | Misfire Detected, Cylinder No. 2 |
| P0303 | Misfire Detected, Cylinder No. 3 |
| P0304 | Misfire Detected, Cylinder No. 4 |
| P0305 | Misfire Detected, Cylinder No. 5 |
| P0306 | Misfire Detected, Cylinder No. 6 |
| P0307 | Misfire Detected, Cylinder No. 7 |
| P0308 | Misfire Detected, Cylinder No. 8 |
| P0309 | Misfire Detected, Cylinder No. 9 |
| P0310 | Misfire Detected, Cylinder No. 10 |
| P0320 | No Crank Reference Signal At PCM |
|  | No RPM Signal To PCM |
| P0325 | Knock Sensor 1 Circuit Malfunction |
| P0330 | Knock Sensor 2 Circuit Malfunction |
| P0336 | CKP Sensor Signal |
| P0340 | No CAM Signal At PCM |
| P0341 | CMP Sensor Signal |
| P0350 | Ignition Coil Draws Too Much Current |
| P0351 | Ignition Coil No. 1 Primary Circuit |
| P0352 | Ignition Coil No. 2 Primary Circuit |
| P0353 | Ignition Coil No. 3 Primary Circuit |
| P0354 | Ignition Coil No. 4 Primary Circuit |
| P0355 | Ignition Coil No. 5 Primary Circuit |
| P0356 | Ignition Coil No. 6 Primary Circuit |
| P0357 | Ignition Coil No. 7 Primary Circuit |
| P0358 | Ignition Coil No. 8 Primary Circuit |
| P0370 | Fuel Injection Pump Speed/Position Sensor Signal Lost |
| P0380 | Intake Air Heater Relay No. 1 Control Circuit |
| P0381 | Wait To Start Lamp Inoperative |
| P0382 | Intake Air Heater Relay No. 2 Control Circuit |
| P0387 | CKP Sensor Supply Voltage Too Low |
| P0388 | CKP Sensor Supply Voltage Too High |
| P0401 | EGR System Failure |
| P0403 | EGR Solenoid Circuit |
| P0404 | EGR Position Sensor Rationality |
| P0405 | EGR Position Sensor Volts Too Low |
| P0406 | EGR Position Sensor Volts Too High |
| P0412 | Secondary Air Solenoid Circuit |
| P0420 | 1/1 Catalytic Converter Efficiency |
| P0432 | 1/2 Catalytic Converter Efficiency |
| P0441 | EVAP Purge Flow Monitor Failure |
| P0442 | EVAP Leak Monitor Medium Leak Detected |
| P0443 | EVAP System Purge Solenoid Circuit |
| P0455 | EVAP Leak Monitor Large Leak Detected |
| P0456 | EVAP Leak Monitor Small Leak Detected |
| P0460 | Fuel Level Unit No Change Over Miles |
| P0461 | Fuel Level unit No Changeover Time |
| P0462 | Fuel Level Unit Volts Too Low |
| P0463 | Fuel Level Unit Volts Too High |
| P0500 | No VSS Signal |
| P0505 | IAC Motor Circuits |
| P0522 | Oil Pressure Sens Low |
| P0523 | Oil Pressure Sens High |
| P0524 | Oil Pressure Too Low |
| P0545 | A/C Clutch Relay Circuit |
| P0551 | Power Steering Switch Failure |
| P0562 | Charging System Voltage Too Low |
| P0563 | Charging System Voltage Too High |
| P0600 | PCM Internal Fault SPI Communications |
| P0601 | PCM Internal Fault Internal Controller Failure |
| P0602 | ECM Fueling Calibration Error |
| P0604 | TCM RAM Self Test Fault Detected |
| P0605 | TCM ROM Self Test Fault Detected |
| P0606 | ECM Failure |
| P0615 | Starter Relay Control Circuit |
| P0622 | Alternator Field No Switching Properly |
| P0645 | A/C Clutch Relay Circuit |
| P0700 | Transaxle Or Transmission Control System Malfunction |
| P0703 | Brake Switch Stuck |
| P0711 | Transmission Temperature Sensor Circuit No Temperature Rise After Start |
| P0712 | Transmission Temperature Sensor Voltage Too Low |
| P0713 | Transmission Temperature Sensor Voltage Too High |
| P0720 | Low Output SPD Sensor RPM, Above 15 MPH |
| P0740 | TCC, No RPM Drop At Lockup |
| P0743 | TCC Solenoid/Trans. Relay Circuits |
| P0748 | Governor Pressure Solenoid Control/Trans. Relay Circuits |
| P0751 | O/D Switch Pressure (Lo) More Than 5 Minutes |
| P0753 | Trans. 3-4 Shift Solenoid/Trans. Relay Circuits |
| P0756 | AW4 Shift Solenoid B (2-3) Functional Failure |
| P0783 | 3-4 Shift Solenoid No RPM Drop At Lockup |
| P0801 | Reverse Gear Lockout Circuit Open Or Short Circuit |
| P0830 | Clutch Depressed Switch Circuit |
| P0833 | Clutch Depressed Switch Circuit |
| P1110 | Decreased Engine Performance Due To High Intake Air Temperature |
| P1180 | Decreased Engine Performance Due To High Injection Pump Fuel Temperature |
| P1192 | IAT Circuit Low |
| P1193 | IAT Circuit High |
| P1195 | 1/1 O2S Slow During Catalyst Monitor |
| P1196 | 2/1 O2S Slow During Catalyst Monitor |
| P1197 | 1/2 O2S Slow During Catalyst Monitor |
| P1198 | Radiator Temperature Sensor Volts Too High |
| P1199 | Radiator Temperature Sensor Volts Too Low |
| P1281 | Engine Cold Too Long |
| P1282 | Fuel Pump Relay Control Circuit |
| P1283 | Idle Select Signal Invalid |
| P1284 | Fuel Injection Pump Battery Voltage Out Of Range |
| P1285 | Fuel Injection Pump Controller Always On |
| P1286 | Accelerator Position Sensor Supply Voltage Too High |
| P1287 | Fuel Injection Pump Controller Supply Voltage Low |
| P1288 | Intake Manifold Short Runner Solenoid Circuit |
| P1289 | Manifold Tune Valve Solenoid Circuit |
| P1290 | CNG Fuel System Pressure Too High |
| P1291 | No Temperature Rise Seen From Intake Heaters |
| P1292 | CNG Pressure Sensor Voltage Too High |
| P1293 | CNG Pressure Sensor Voltage Too Low |
| P1294 | Target Idle Not Reached |
| P1295 | No 5 Volts To TP Sensor |
|  | Accelerator Position Sensor Supply Voltage Too Low |
| P1296 | No 5 Volt To MAP Sensor |
| P1297 | No Change In MAP From Start To Run |
| P1298 | Lean Operation At Wide Open Throttle |
| P1299 | Vacuum Leak Found (IAC Fully Seated) |
| P1388 | Auto Shutdown Relay Control Circuit |
| P1389 | No ASD Relay Output Voltage At PCM |
| P1390 | Timing Belt Skipped One Or More Notches |
| P1391 | Camshaft Or Crankshaft Position Sensor Intermittent Loss |
| P1398 | Misfire Adaptive Numerator At Limit |
| P1399 | Wait To Start Lamp Circuit |
| P1403 | Loss Of 5 Volts Feed To EGR Position Sensor |
| P1475 | Auxiliary 5 Volt Supply Voltage High |
| P1476 | Too Little Secondary Air |
| P1477 | Too Much Secondary Air |
| P1478 | Battery Temperature Sensor Volts Out Of Limit |
| P1479 | Transmission Fan Relay Circuit |
| P1480 | PCV Solenoid Circuit |
| P1481 | EATX RPM Pulse Generator Signal For Misfire Detection Does Not Correlate With Expected Value |
| P1482 | Catalyst Temperature Sensor Circuit Shorted Low |
| P1483 | Catalyst Temperature Sensor Circuit Shorted High |
| P1484 | Catalytic Converter Overheat Detected |
| P1485 | Air injection Solenoid Circuit |
| P1486 | EVAP Leak Monitor Hose Pinched |
| P1487 | High Speed Fan Control Relay Circuit |
| P1488 | Auxiliary 5 Volt Supply Output Too Low |
|  | 5 Volt Supply Voltage Low |
| P1489 | High Speed Cooling Fan Relay Circuit |
| P1490 | Low Speed Cooling Fan Relay Circuit |
| P1491 | Radiator Fan Control Relay Circuit |
| P1492 | Battery Temperature Sensor Voltage Too High |
| P1493 | Battery Temperature Sensor Voltage Too Low |
| P1494 | Leak Detection Pump Pressure Switch Or Mechanical Failure |
| P1495 | Leak Detection Pump Solenoid Circuit |
| P1496 | 5 Volt Output Too Low |
| P1498 | High Speed Radiator Fan Ground Control Relay Circuit |
| P1594 | Charging System Voltage Too High |
| P1595 | Speed Control Solenoid Circuits |
| P1596 | Speed Control Switch Always High |
| P1597 | Speed Control Switch Always Low |
| P1598 | A/C Pressure Sensor Volts Too High |
| P1599 | A/C Pressure Sensor Volts Too Low |
| P1680 | Clutch Released Switch Circuit |
| P1681 | No Instrument Panel Cluster CCD/J1850 Messages Received |
| P1682 | Charging System Voltage Too Low |
| P1683 | Speed Control Power relay Or Speed Control 12 Volt Driver Circuit |
| P1684 | Battery Has Been Disconnected Within Last 50 Starts |
| P1685 | SKIM Invalid Key |
| P1686 | No SKIM BUS Messages Received |
| P1687 | No MIC BUS Messages |
| P1688 | Internal Fuel Injection Pump Controller Failure |
| P1689 | No Communication Between ECM & Injection Pump Module |
| P1690 | Fuel Injection Pump CKP Sensor Does Not Agree With ECM CKP Sensor |
| P1691 | Fuel Injection Pump Controller Calibration Error |
| P1692 | DTC Set In ECM |
| P1693 | DTC Detected In Companion Module |
|  | DTC Detected In PCM/ECM |
| P1694 | Fault In Companion Module |
|  | No CCD Messages Received From ECM |
| P1695 | No CDD/J1850 Message From Body Control Module |
| P1696 | PCM Failure EEPROM Write Denied |
| P1697 | PCM Failure SRI Mile Not Stored |
| P1698 | No CCD/J1850 Messages From TCM |
|  | No CCD Messages Received From PCM |
| P1719 | SKIP Shift Solenoid |
| P1740 | A Rationality Error Has Been Detected In Either TCC Solenoid Or O/D Solenoid Systems |
|  | TCC Or O/D Solenoid Performance |
| P1756 | GOV Pressure Not Equal To Target At 15-20 Psi. |
| P1757 | GOV Pressure Not Equal To Target At 15-20 Psi. |
| P1762 | GOV Pressure Sensor Offset Volts Too Low Or High |
| P1763 | GOV Pressure Sensor Volts Too High |
| P1764 | GOV Pressure Sensor Volts Too Low |
| P1765 | Transmission 12 Volt Supply Relay Control Circuit |
| P1899 | Park/Neutral Switch Failure |

## Jeep

| 1000 | Ignition Circuit Low |
| --- | --- |
| 1001 | Ignition Circuit High |
| 1004 | ECU Battery Feed & Power Grounds |
| 1005 | System Ground Circuit |
| 1006 | EGR/EVAP Solenoid Circuit Low |
| 1007 | EGR/EVAP Solenoid Circuit High |
| 1008 | Power Steering Circuit Low |
| 1009 | Power Steering Circuit High |
| 1012 | MPA Circuit Low |
| 1013 | MPA Circuit High |
| 1014 | Fuel Pump Circuit Low |
| 1015 | Fuel Pump Circuit High |
| 1016 | Charge Air Temperature Circuit Low |
| 1017 | Charge Air Temperature Circuit High |
| 1018 | Serial Data Circuit |
| 1019 | Power Latch Not Set |
| 1021 | Engine Failed To Start Due To Mechanical, Fuel Or Ignition Conditions |
| 1022 | Starter Relay Circuit Low |
| 1024 | ECU Start Circuit Low |
| 1025 | WOT Circuit Low |
| 1026 | WOT Circuit High |
| 1027 | ECU Sees Wide Open Throttle |
| 1028 | ECU Does Not See Wide Open Throttle |
| 1029 | ISA Closed Throttle Circuit Low |
| 1030 | ISA Closed Throttle Circuit High |
| 1031 | ECU Sees Closed Throttle |
| 1032 | ECU Does Not See Closed Throttle |
| 1033–36 | ISA Circuits |
| 1037 | TP Sensor Circuit Reads Low |
| 1038 | Park/Neutral Line High |
| 1039 | Park/Neutral Line Low |
| 1040 | Latched B+ Line Low |
| 1041 | Latched B+ Line High |
| 1042 | No Latched B+ 1/2 Volt Drop |
| 1043 | Shift Lamp Circuit Grounded |
| 1044 | D2–1 Circuit Low (A/T){1} |
|  | Upshift Lamp Circuit (M/T){1} |
|  | Shift Lamp Circuit High{2} |
| 1045 | Shift Lamp Circuit Open{1} |
| 1047 | Wrong ECU |
| 1048 | M/T Vehicle Configuration |
| 1049 | A/T Vehicle Configuration |
| 1050 | Idle RPM Low |
| 1051 | Idle RPM High |
| 1052 | MAP Sensor Out Of Limits |
| 1053 | Change In MAP Reading Out Of Limits |
| 1054 | Coolant Sensor & 5 Volt Supply For TP Sensor/MAP Circuits Low |
| 1055 | Coolant Sensor Circuit High |
| 1056 | Inactive Coolant Temperature Sensor |
| 1059 | A/C Request Circuit Low |
| 1060 | A/C Request Circuit High |
| 1061 | A/C Select Circuit Low |
| 1062 | A/C Select Circuit High |
| 1063 | A/C Clutch Circuit Low |
| 1064 | A/C Clutch Circuit High & Power Steering Input |
| 1065 | Rich Oxygen Sensor Input |
| 1066 | Lean Oxygen Sensor Input |
| 1067 | Latch Relay Circuit Low |
| 1068 | Latch Relay Circuit High |
| 1069 | No Tach |
| 1074 | ECU Does Not See Speed Sensor |

{1}—Cherokee & Wagoneer.
{2}—Wrangler.

## Ram Raider & Ram 50

| 2.4L & 3.0L |  |
| --- | --- |
| 11 | Oxygen Sensor |
| 12 | Air Flow Sensor |
| 13 | Intake Air Temperature Sensor |
| 14 | Throttle Position Sensor |
| 15 | Motor Position Sensor |
| 21 | ECT Sensor |
| 22 | Crank Angle (Crankshaft Position) Sensor |
| 23 | Top Dead Center (Camshaft Position) Sensor |
| 24 | Vehicle Speed Sensor (Reed Switch) |
| 25 | Barometric Pressure Sensor |
| 36 | Ignition Timing Adjustment Signal |
| 41 | Injector |
| 42 | Fuel Pump |
| 43 | EGR (California) |
| 55 | Idle Air Control Valve Position Switch (California) |
| 59 | Heated Rear Oxygen Sensor (California) |