## FORD

## Accessing Trouble Codes

### Using Analog Voltmeter

Exc. EEC-V, DI 7.3L Turbo Diesel & Villager

Connect analog voltmeter to data link connector located in engine compartment. When a diagnostic trouble code is reported on the analog voltmeter for a functional test, it will represent itself as a pulsing or sweeping movement of the voltmeter’s needle across the dial face.

Accessing diagnostic trouble codes. Use the Analog voltmeter or Test light method, not both.

## Using Check Engine/ MIL

Exc DI 7.3L Turbo Diesel, Merkur (2.9L), Villager & 85–88 2.3L Engine

The MIL lamp is intended to alert the driver of certain malfunctions in the engine control system. If such a fault occurs, the EEC-IV processor will substitute a value or values and continue operating. In some cases this action may result in a slight change in vehicle driveability.

The Check Engine lamp can be used to read DTCs. The instrument panel Check Engine lamp will remain on when a hard fault is present. During the Self test sequence, a DTC is reported by the Check Engine lamp. It will represent itself as a flash on the Check Engine lamp display on the instrument panel. A single digit number of three will be reported by three flashes. However, as previously stated, a DTC is represented by a two digit number, such as 2–3. As a result, DTC 2–3 will appear on the Check Engine lamp display as two flashes, then after a two second pause, the lamp will flash three times.

DTC interpretation. Check engine lamp

Continuous Memory DTCs are separated from Key On Engine Off DTCs by a six second delay, a single half second flash, then another six second delay.

Villager

- Disconnect Diagnostic Test Connector (DTC) located on LH side of engine compartment, on transaxle behind coolant reservoir.

- Using suitable jumper wire, jump BL/W and GY/BL wires.

Accessing DTCs. MIL

  - Wait about two seconds.

  - Remove jumper wire and reconnect DTC.

  - Record DTCs by reading MIL flashes.

## Using Message Center

Continental

- Using electronic instrument cluster, hold in all three buttons (select, checkout and reset buttons) at same time.

- To conduct Key-ON/Engine-OFF (KOEO) Self test portion, hold in all three buttons and turn ignition On, then release three buttons.

- To conduct Key-ON/Engine-Running (KOER) Self test portion, hold in all three buttons, turn ignition On and release three buttons.

- On 87–90 models, press select button three times

- On 91–95 models, depress gauge select button three times until ‘‘DEALER 4’’ is displayed.

- On all models, to initiate Self test, using a jumper wire, jump self test connectors STI to SIG RTN.

Accessing DTCs. Message center See Image above

  - On 87–90 models, a base readout of 4255 indicates that KOEO self test has been entered.

  - On 87–90 models, a base readout of 4030 indicates engine running self test has been entered.

  - On 87–90 models, DTC output will be the RH three digits displayed. Service pass code is 4011.

  - On 91–97 models, DTCs will be displayed on message center.

  - On all models, to exit Self test, turn ignition Off.

## Using Overdrive Cancel/Transmission MIL (OCIL/TMIL)

On 89–98 models equipped with 7.3L diesel engine and E4OD transmission, perform KOEO Self Test if the OCIL/TMIL lamp flashes. The lamp stays off when OSC is toggled once to indicate vehicle can attain overdrive gear position. The lamp stays on when OSC is toggled to prevent vehicle from shifting into overdrive position. DTC 99 indicates a fault in the E4OD transmission Electronic Pressure Control (EPC) circuit. Under this condition, the lamp serves as the Transmission Malfunction Indicator Lamp (TMIL).

# Clearing Codes

Except MCU System

EEC-V & DI 7.3L Turbo Diesel

The PCM can be reset using the New Generation Star (NGS) Tester or by disconnecting the battery ground cable for five minutes. If the battery ground cable was disconnected to clear DTCs, the vehicle may operate rough for the first few miles until adaptive values are relearned.

EEC-IV Continuous Memory

1. Perform KOEO self test.
2. When DTCs begin to display, deactivate self test as follows:

a. STAR tester, position center button in up position. b. Analog Volt/Ohmmeter, remove jumper wire from self test input connector and signal return pin of self test connector. c. NGS tester, depress STOP button. d. Check engine lamp, remove jumper wire from self test input connector and signal return pin of self test connector. e. Message center, remove jumper wire from self test input connector and signal return pin of self test connector. f. OCIL, remove jumper wire from self test connector and signal return pin of self test connector.

3. Continuous DTCs should now be erased.

Keep Alive Memory (KAM)

To clear keep alive memory, disconnect and isolate negative battery cable for a minimum of five minutes. After clearing memory, it is necessary to drive vehicle a minimum of 10 miles to allow processor time to relearn value.

Merkur 2.9L & 85–88 2.3L Engine

1. Initiate KOEO self test. 2. Remove jumper wire from self test input terminal as soon as first diagnostic trouble code is received (even if an 11 is the first DTC). 3. Repeat self test with jumper wire to verify DTCs have been erased.

Aspire, Capri (1.6L), Escort & Tracer (1.8L), Probe (2.0L w/4EAT, 2.2L & 2.5L) & 88–93 Festiva

1. Initiate KOEO self test. 
2. Unlatch push button on Super Star II Tester as soon as first diagnostic trouble code is received. 
3. Disconnect and isolate battery ground cable, then depress brake pedal for 5–10 seconds. 
4. Conduct self test to ensure DTCs have been erased.

Villager

Except Malfunction Indicator Lamp

DTCs may be erased using one of two methods. Disconnect battery ground cable, DTCs will be erased from back-up memory after 24 hours. When using NGS tester, select Diagnostic Test Mode Results, then depress CLEAR.

Malfunction Indicator Lamp

1. Activate diagnostic test mode.
2. Disconnect DTC.
3. Using suitable jumper wire, jump BL/W and GY/BL wires.
4. Wait about two seconds.
5. Remove jumper wire and reconnect DTC.
6. MIL will stay on, and DTCs will be erased.

MCU System

To clear DTCs, turn ignition Off. MCU system does not have KAM to retain service DTCs

Also see this page EEC IV Diagnostic page for more Ford information.

Ford Trouble Codes

| 88-95 | 1.3L EFI, 1.6L, 1.8L, 2.0L w/4EAT & 2.5L |
| --- | --- |
| 93-95 | probe |
| 93-95 | EEC Villager |
| 96-97 | 1.3L EFI, 1.8L & 2.5L |
| 96-01 | OBD2/MIL |
| EEC | 1.3L FBC Engine exc. Villager |
| EEC-IV | EEC-IV/OBD2 |
| EEC-V | EEC-V/OBD2 |
| MCU | MCU |
|  | 7.3L Turbo Diesel |

(R) = Engine Running test (M) = Memory code  (O)=Key On Engine Off test

|  |  | 2 Digit Codes |
| --- | --- | --- |
| 11 |  | System OK |
| 12 |  | Idle Speed Control motor or Air Bypass not controlling idle properly (generally idle too low) - ISC |
| 13 | (O) | ISC did not respond properly (extends to touch throttle then retracts for KOEO) - ISC |
|  | (R) | Idle Speed Control motor or Air Bypass not controlling idle properly (generally idle too high) |
|  | (M) | ISC sticking, open ITS circuit or TP sticking |
| 14 |  | Ignition pickup (PIP) was erratic - Ignition Systems |
|  |  | E4OD Transmission diesel RPM sensor - Diesel RPM sensor |
| 15 | (O) | No Keep Alive Memory power to PCM pin 1 or bad PCM (Memory Test Failure) |
|  | (M) | KAM (pin 1) was interrupted (was battery disconnected ?) |
| 16 |  | 1 9L & 2 5L - Throttle stop set too high - IDLE or Idle Set Procedures |
|  |  | 2 3L - RPM's too low - IDLE |
|  | (O) | Electronic ignition - IDM circuit fault - Ignition Systems |
| 17 |  | 1 9L & 2 5L - Throttle stop set too low - IDLE |
| 18 | (R) | Check base timing & advance function - Timing Tests |
|  | (M) | Ignition TACH signal erratic - Ignition Systems |
|  |  | Spark Angle Word (SAW) circuit failure (1 9L SFI) |
| 19 | (O) | No Vehicle Power (pins 37 + 57) or bad PCM VPWR Diagnosis |
|  | (R) | Erratic idle during test (reset throttle & retest) - Idle Set Procedures |
|  |  | Electronic ignition Cylinder ID sensor/circuit problem - Ignition Systems |
| 21 |  | Engine Coolant Temperature (ECT) sensor out of range - ECT |
| 22 |  | MAP (vacuum) or BARO signal out of range - MAP |
| 23 |  | Throttle sensor out of range or throttle set too high - TPS |
| 24 |  | Intake Air Temperature (IAT) or Vane Air Temperature (VAT) sensor out of range - IAT VAT |
| 25 |  | Knock sensor not tested (ignore if not pinging) - KS |
| 26 |  | Mass Air Flow (MAF) or Vane Air Flow (VAF) out of range - MAF VAF |
|  |  | Transmission Oil Temperature (TOT) sensor out of range, Transmissions |
| 27 |  | Vehicle Speed Sensor problem - VSS |
| 28 |  | Vane Air Temperature (VAT) sensor out of range - VAT |
|  |  | 2 3L w/Electronic Ignition - Cyl ID, IDM low or right coil pack failure - Ignition Systems |
| 29 |  | Vehicle Speed Sensor problem - VSS |
| 31 | (O,R,M) | EVP - EVP signal is/was out of range - EVP |
|  | (O,R,M) | EVR - EVP signal is/was low - EVR |
|  | (O,R,M) | PFE - PFE signal is/was low - PFE |
| 32 | (R) | EVP - EGR not responding properly during test - EVP |
|  | (O,R,M) | EVR - EVP signal is/was low - EVR |
|  | (R,M) | PFE - PFE shows low pressure, EGR not seating or memory, not seating intermittently - PFE |
| 33 | (O,M) | ALL - EGR did not open/ respond during test or if memory code, did not open intermittently - EVP EVR PFE |
| 34 | (R) | EVP - EGR did not respond properly during test - EVP |
|  | (O,R,M) | EVR - EVP sensor is/was high - EVR |
|  | (O,R,M) | PFE - PFE sensor is/was out of range - PFE |
| 35 | (R) | EVP - Engine RPM's too low to test EGR system - EVP |
|  | (O,R,M) | EVR - EVP sensor signal is/was high - EVR |
|  | (O,R,M) | PFE - PFE sensor signal is/was high - PFE |
| 38 |  | Idle Tracking Switch signal was intermittent - ISC |
| 39 |  | Transmission Torque Converter clutch not engaging - Transmissions |
| 41 | (R) | System lean - Fuel control |
|  | (M) | System was lean for 15 seconds or more (no HO2S switching) - Fuel control |
| 42 | (R) | System rich - Fuel control |
|  | (M) | System was rich for 15 seconds or more (no HO2S switching) - Fuel control |
| 43 | (R) | HO2S sensor not reading (run at 2000 rpm's for 2 minutes and retest - check for HO2S switching) |
|  | (M) | Was lean at WOT for 3 seconds or more - Fuel control |
| 44 |  | AIR system inoperative - Air Injection |
| 45 |  | AIR not Diverting (AIRD) - Air Injection |
|  |  | Electronic Ignition - coil primary circuit failure - Ignition Systems |
| 46 |  | AIR Bypass (AIRB) not working - Air Injection |
|  |  | Electronic Ignition - primary circuit failure coil 2 - Ignition Systems |
| 47 |  | Low flow unmetered air (check for small vacuum leaks, injector o'rings, gaskets etc ) |
|  |  | E4OD transmission 4x4 switch/circuit problem - Transmissions |
| 48 |  | High flow unmetered air (check for large vacuum leak, inlet hoses etc ) |
|  |  | Electronic Ignition - coil primary circuit failure - Ignition Systems |
| 49 |  | Electronic Ignition - spout signal circuit problem - Ignition Systems |
|  |  | Transmission 1/2 shift problem - Transmissions |
| 51 |  | Engine Coolant Temperature (ECT) sensor signal is/was too high - ECT |
| 52 |  | Power Steering Pressure Switch/circuit open - PSP |
|  | (R) | Did you turn wheel during test ? |
| 53 |  | Throttle Position sensor too high - TPS |
| 54 |  | Intake Air Temperature (IAT) or Vane Air Temperature (VAT) signal high - IAT VAT |
| 55 |  | No or low (under 7 5 V) Key Power to PCM pin 5 |
| 56 |  | Vane Air Flow (VAF) or Mass Air Flow (MAF) sensor high - VAF MAF |
|  |  | Transmission Oil Temperature sensor too high - Transmissions |
| 57 |  | Intermittent in Park/Neutral/ Switch or Neutral Pressure switch circuit - PNP or Transmissions |
| 58 |  | Idle Tracking Switch (ITS) signal problem ISC |
|  |  | Vane Air Temperature (VAT) sensor out of range or open - VAT |
| 59 |  | AXOD 4/3 circuit fault - Transmissions |
|  |  | 3 0L SHO - Low speed fuel pump circuit problem - Power / Fuel Pump Circuits |
|  |  | Transmission 2/3 shift problem - Transmissions |
| 61 |  | Engine Coolant Temperature (ECT) sensor is or was too low - ECT |
| 62 |  | AXOD (KOEO only) 3/2 circuit short to ground - Transmissions |
|  |  | AXOD (KOEO AND KOER) 4/3 circuit failure - Transmissions |
|  |  | E4OD excessive converter clutch slippage - Transmissions |
| 63 |  | Throttle Position Sensor (TPS) signal too low TPS |
| 64 |  | Intake Air Temperature (IAT) or Vane Air Temperature (VAT) signal low or grounded - IAT VAT |
| 65 |  | Check intermittent HO2S (signal or ground) - Fuel Control |
|  | (R) | E4OD truck - cycle OD cancel switch after engine ID is received - Transmissions |
|  |  | 1984 3 8L ONLY - O, M Battery voltage high (check for electrical system overcharging) |
| 66 |  | Vane Air Flow (VAF) or Mass Air Flow (MAF) signal low - VAF MAF |
|  |  | Transmission Oil Temperature (TOT) signal low (possibly grounded) - Transmissions |
| 67 |  | Park/Neutral circuit fault - PNP |
|  |  | Transmission Manual Lever Position (MLP) sensor circuit - Transmissions |
|  | (M) | Intermittent Park Neutral Position (PNP) sensor fault - PNP |
| 68 |  | Idle Tracking Switch (ITS) circuit (possibly grounded) - ISC |
|  |  | Vane Air Temperature (VAT) sensor out of range or grounded - VAT |
|  |  | 3 8L AXOD -Transmission Temperature Switch (TTS) open - Transmissions |
|  |  | Electronic Transmission - Transmission Oil Temperature (TOT) sensor was overheated -Transmissions |
| 69 |  | AXOD transmission (O) 3/2 switch closed (possible short circuit) - Transmissions |
|  |  | AXOD (M) 3/2 switch open (poss short to power) - Transmissions |
|  |  | E4OD 3/4 shift problem - Transmissions |
| 70 | (M) | 3 8L AXOD - Data link to instrument cluster fault Service any other EEC codes, erase memory and retest If code is still present refer to instrument cluster diagnosis manual |
| 71 | (M) | 1 9L TBI, 2 3L TBI, 2 5L TBI - ITS signal was grounded when throttle should have been opening ITS-ISC ISC motor problem or Idle Tracking Switch (ITS) signal wire shorted to ground - ISC |
|  | (M) | 1 9L MFI - PCM re-initialized Possible electrical noise, case ground or intermittent VPWR problem - VPWR Diagnosis |
|  | (M) | 3 8L AXOD - Data link to instrument cluster fault - See code 70 |
| 72 | (R) | No MAP or MAF change in "goose" test - retest, check for frequency or voltage change - MAP MAF |
|  | (M) | 1 9L MFI - VPWR circuit to PCM was intermittent - VPWR Diagnosis |
|  | (M) | 2 3L T/C - PCM re-initialized Possible electrical noise, case ground or intermittent VPWR problem - VPWR Diagnosis |
|  | (M) | 3 8L AXOD - Message center data link circuit fault - See code 70 |
| 73 | (O) | Rerun test, if 73 is still output replace TPS |
|  | (R) | No Throttle Position Sensor (TPS) change in "goose" test Must get at least 25% throttle rotation - TPS |
| 74 |  | Was brake depressed after engine ID was received ? |
|  |  | Brake On Off (BOO) signal open or short to ground - BOO |
| 75 |  | Brake On Off (BOO) signal shorted to power - BOO |
| 76 |  | Vane Air Flow (VAF) did not respond to "goose" test - VAF |
| 77 |  | System did not receive "goose" test - see TESTS |
| 78 | (M) | VPWR circuit to PCM was intermittent or the PCM is bad VPWR Diagnosis |
| 79 |  | A/C is on or pin 10 is shorted to power |
| 81 |  | Boost control solenoid - Solenoids |
|  |  | AIRD solenoid - Solenoids and Air Injection |
|  |  | 3 0L SHO - Inlet Air Solenoid - Solenoids |
| 82 |  | 2 3L TC - Fan Control wire shorted to ground - A/C and Fan Circuits |
|  |  | AIRB solenoid - Solenoids and Air Injection |
|  |  | 3 8L SC - Super Charger Bypass Solenoid - Solenoids |
| 83 |  | High Electro Drive Fan circuit fault - A/C and Fan Circuits |
|  |  | EGR Control solenoid - Solenoids |
|  |  | 3 0L SHO - Low Speed Fuel Pump Relay circuit - Power / Fuel Pump Circuits |
| 84 |  | EGR Vacuum Regulator - Solenoids |
|  |  | EGR cutoff solenoid - Solenoids |
|  |  | EGR Vent solenoid - Solenoids |
| 85 |  | 2 3L T/C Automatic - 3/4-4/3 Shift solenoid - Transmissions |
|  |  | CANP solenoid (ALL 1989) - Solenoids |
|  | (M) | 1 9L MFI - System has corrected rich condition - Fuel control |
| 86 |  | 2 3L or 2 9L Truck - A4LD 3/4 shift solenoid - Transmissions |
|  |  | (M) 1 9L MFI - System has corrected lean condition - Fuel control |
| 87 | (O) | Fuel pump circuit fault (check inertia switch) - Power / Fuel Pump Circuits |
|  |  | Vehicles with 2BBL carb - Temperature Compensated Accelerator Pump Solenoid - Solenoids (M) intermittent in fuel pump primary circuit - Power / Fuel Pump Circuits NOTE: On some Escorts with automatic seat belts this code is normal IN MEMORY due to the wiring |
| 88 |  | Throttle Kicker Solenoid - Solenoids |
|  |  | Variable Voltage Choke relay circuit fault - VVC |
|  |  | Fan Control circuit fault - A/C and Fan Circuits |
|  |  | A4LD - Converter Clutch Override solenoid - Transmissions |
|  |  | Electronic Ignition - IDM, DPI or spout circuit fault - Ignition Systems |
| 89 |  | A4LD - Converter Clutch Override solenoid - Transmissions |
|  |  | AXOD Torque Converter Control solenoid circuit - Transmissions |
|  |  | Exhaust Heat Control (heat riser) solenoid circuit - Solenoids |
| 91 | (R, M) | System running lean - Fuel control |
|  |  | Transmission SS 1 circuit/solenoid problem - Transmissions |
| 92 | (R) | System running rich - Fuel control |
|  |  | Transmission SS 2 circuit/solenoid problem - Transmissions |
| 93 | (O,R) | Throttle linkage binding or bad ISC motor ISC HO2S not reading Fuel control |
|  |  | Transmission TCC circuit/solenoid problem - Transmissions |
| 94 |  | AIR system inoperative - Air Injection |
|  |  | Transmission TCC circuit/solenoid problem - Transmissions |
| 95 | (O) | Fuel pump: open, bad ground or always on - Power / Fuel Pump Circuits |
|  | (R) | AIR not Diverting (AIRD) - Air Injection |
|  | (M) | Possible bad fuel pump ground or open between fuel pump and pin 8 at PCM (Fuel Pump Monitor signal) - Power / Fuel Pump Circuits |
| 96 | (O) | Fuel pump monitor circuit shows no power - Power / Fuel Pump Circuits |
|  | (R) | AIR Bypass (AIRB) not working - Air Injection |
|  | (M) | (Service 87 code first if present) Fuel pump relay or battery power feed was open - Power / Fuel Pump Circuits |
| 97 |  | E4OD OD cancel light circuit failure - Transmissions |
| 98 | (R) | Did not pass KOEO yet (Get 11 in KOEO first) |
|  |  | Transmission EPC circuit/solenoid failure - Transmissions |
| 99 | (R) | ISC needs to learn (Let idle for 2 minutes, Erase memory and retest) |
|  |  | Transmission EPC circuit/solenoid failure - Transmissions |

(R) = Engine Running test (M) = Memory code  (O)=Key On Engine Off test

|  |  | 3 Digit codes |
| --- | --- | --- |
| 111 |  | System checks OK |
| 112 | (O,M) | Intake Air Temperature (IAT) sensor is/was low or grounded - IAT |
| 113 | (O,M) | IAT sensor is/was high or open - IAT |
| 114 | (O,R) | IAT sensor out of range - IAT |
| 116 | (O,R) | Engine Coolant (ECT) sensor out of range - ECT |
| 117 | (O,M) | ECT sensor is/was low or grounded - ECT |
| 118 | (O,M) | ECT sensor is/was high or open - ECT |
| 121 | (O,R,M) | Throttle Position (TP) sensor out of range - TPS |
| 122 | (O,M) | TP low (possibly grounded or open circuit) - TPS |
| 123 | (O,M) | TP is/was high or short to power - TPS |
| 124 | (M) | TP voltage was higher than expected - Fuel control |
| 125 | (M) | TP voltage was lower than expected - Fuel control |
| 126 | (O,R,M) | MAP or BARO sensor out of range - ">MAP |
| 128 | (M) | MAP vacuum has not been changing - check vacuum lines - ">MAP |
| 129 | (R) | No MAP or Mass Air Flow sensor change during "goose" test - MAP MAF |
| 136 | (R) | Oxygen sensor not switching/system lean Left or Front HO2S - Fuel control |
| 137 | (R) | Oxygen sensor not switching/system rich Left or Front HO2S - Fuel control |
| 138 | (R) | Fault in Cold Start Injector circuit - Fuel control |
| 139 | (M) | Oxygen sensor not switching Left or Front HO2S - Fuel control |
| 144 | (M) | Oxygen sensor not switching Single, Right or Rear HO2S - Fuel control |
| 157 | (R,M) | Mass Air Flow signal is/was low or grounded - MAF |
| 158 | (O,R,M) | MAF sensor is/was high or short to power - MAF |
| 159 | (O,R) | MAF sensor is/was out of range - MAF |
| 167 | (R) | No Throttle Position sensor change in "goose" test (must get at least 25% rotation) - TPS |
| 171 | (M) | Oxygen sensor not switching - system was at adaptive limits - Single, Right or Rear HO2S - Fuel control |
| 172 | (R,M) | Oxygen sensor not switching - system is or was lean - Single, Right or Rear HO2S - Fuel control |
| 173 | (R,M) | Oxygen sensor not switching - system is or was rich - Single, Right or Rear HO2S - Fuel control |
| 174 | (M) | Oxygen sensor was slow in switching Single, Right or Rear HO2S - Fuel control |
| 175 | (M) | Oxygen sensor not switching - system was at adaptive limits - Left or Front HO2S - Fuel control |
| 176 | (M) | Oxygen sensor not switching - system is or was lean Left or Front HO2S - Fuel control |
| 177 | (M) | Oxygen sensor not switching - system was rich Left or Front HO2S - Fuel control |
| 178 | (M) | Oxygen sensor was slow in switching Left or Front HO2S - Fuel control |
| 179 | (M) | Fuel system was rich at part throttle Single, Right or Rear HO2S - Fuel control |
| 181 | (M) | Fuel system was lean at part throttle Single, Right or Rear HO2S - Fuel control |
| 182 | (M) | Fuel system was rich at idle Single, Right or Rear HO2S - Fuel control |
| 183 | (M) | Fuel system was lean at idle Single, Right or Rear HO2S - Fuel control |
| 184 | (M) | Mass Air (MAF) output higher than expected - Fuel control |
| 185 | (M) | Mass Air (MAF) output lower than expected - Fuel control |
| 186 | (M) | Injector pulse width longer than expected or Mass Air Flow (MAF) lower than expected - Fuel control |
| 187 |  | Injector pulse width shorter than expected or Mass Air Flow (MAF) higher than expected - Fuel control |
| 188 | (M) | Fuel system was rich at part throttle - Left or Front HO2S - Fuel control |
| 189 | (M) | Fuel system was lean at part throttle - Left or Front HO2S - Fuel control |
| 191 | (M) | Fuel system was rich at idle - Left or Front HO2S - Fuel control |
| 192 | (M) | Fuel system was lean at idle - Left or Front HO2S - Fuel control |
| 193 |  | Failure in Flexible Fuel (FF) sensor circuit - Fuel control |
| 194 | (M) | Perform cylinder balance test to check for inoperative injectors |
| 195 | (M) | Perform cylinder balance test to check for inoperative injectors |
| 211 | (M) | Ignition PIP signal was erratic or missing - Ignition Systems |
| 212 | (M) | Ignition TACH signal was erratic (module/wiring) or SPOUT circuit fault - Ignition Systems |
| 213 | (R) | Ignition SPOUT or SAW circuit open or shorted - Ignition Systems |
| 214 | (M) | Error in Cylinder ID (CID) circuit or signal - Ignition Systems |
| 215 | (M) | Primary circuit failure - ignition coil 1 - Ignition Systems |
| 216 | (M) | Primary circuit failure - ignition coil 2 - Ignition Systems |
| 217 | (M) | Primary circuit failure - ignition coil 3 - Ignition Systems |
| 218 | (M) | IDM signal open or high or left coil pack failure - Ignition Systems |
| 219 | (M) | SPOUT circuit failure, timing defaulted to 10 degrees - follow code 213 diagnosis |
| 222 | (M) | IDM open or high or right coil pack failure - Ignition Systems |
| 223 | (M) | Dual Plug (DPI), SPOUT or IDM circuit fault - Ignition Systems |
| 224 | (M) | Failure in ignition coil primary circuit - Ignition Systems |
| 225 | (R) | Knock sensor not tested (ignore if not pinging) - KS |
| 226 | (O) | Ignition Diagnostic Monitor (IDM) signal fault - Ignition Systems |
| 232 | (M) | EI primary coil circuit failure - Ignition Systems |
| 238 | (M) | EI primary circuit failure - ignition coil 4 - Ignition Systems |
| 311 | (R) | AIR system not working - Single, Right or Rear HO2S - Air Injection |
| 312 | (R) | AIR not diverting - Air Injection |
| 313 | (R) | AIR not bypassing - Air Injection |
| 314 | (R) | AIR inoperative, Left or Front HO2S - Air Injection |
| 326 | (R,M) | Pressure Feedback EGR shows low pressure EGR not seating or not seating intermittantly - PFE |
| 327 | (O,R,M) | EGR feedback signal is/was low - EVR or PFE |
| 328 | (O,R,M) | EGR Valve Position (EVP) is/was low - EVR |
| 332 | (R,M) | EGR did not open/respond during test or if memory code, did not open intermittantly - EVR or PFE |
| 334 | (O,R,M) | EVP sensor is/was high - EVR |
| 335 | (O) | EGR feedback signal is/was out of range - EVR or PFE |
| 336 | (O,R,M) | PFE sensor signal is/was was high - ">PFE |
| 337 | (O,R,M) | EGR feedback signal is/was was high - EVR |
| 338 | (M) | Cooling system did not heat up (check cooling system / thermostat operation) |
| 339 | (M) | Cooling system overheated (check cooling system / thermostat operation) |
| 341 | (O) | Octane jumper installed (information only code to notify you if it is installed) |
| 411 | (R) | Idle speed system not controlling idle properly (generally idle too high) - ISC |
| 412 | (R) | Idle speed system not controlling idle properly (generally idle too low) - ISC |
| 452 | (M) | Vehicle Speed Sensor (VSS) problem |
| 511 | (O) | No power to PCM pin 1 or bad PCM (processor) |
| 512 | (M) | Memory power (PCM pin 1) was interrupted - Was battery disconnected ? |
| 513 | (O) | Replace processor (PCM) (internal failure) |
| 519 | (O) | PSP switch/circuit open - PSP h Pedal Position (CPP) circuit fault - PNP |
| 528 | (M) | System shows voltage at pin 10 (is A/C on ?) or pin 30 (PNP, CPP switch) - PNP |
| 529 | (M) | Data Communications Link to processor failure Service any EEC codes, erase memory and retest If code is still present refer to instrument cluster diagnosis manual |
| 533 | (M) | Data Communications Link to instrument cluster failure - see 529 |
| 536 | (O,R,M) | Brake On Off open or shorted to ground - BOO |
| 538 | (R) | System did not receive "goose" test - TESTS |
| 539 | (O) | System shows voltage at PCM pin 10 Is A/C on ? |
| 542 | (O,M) | Fuel pump open, bad ground or always on - - Power / Fuel Pump Circuits |
| 543 | (O) | Fuel pump monitor circuit shows no power - Power / Fuel Pump Circuits |
|  | (M) | (Service 556 code first if present) Fuel pump relay or battery power feed was open - Power / Fuel Pump Circuits |
| 551 |  | Problem in Intake Manifold Runner Control (IMRC) solenoid/circuit - Solenoids |
| 552 | (O) | AIRB solenoid/circuit failure - Solenoids |
| 553 | (O) | AIRD solenoid/circuit failure - Solenoids |
| 554 | (O) | Fuel Press Regulator Control solenoid/circuit fault - Power / Fuel Pump Circuits |
| 556 | (O,M) | Fuel pump relay primary circuit fault - Power / Fuel Pump Circuits |
| 557 | (O,M) | Low speed pump relay primary circuit fault - Power / Fuel Pump Circuits |
| 558 | (O) | EGR vacuum regulator solenoid/circuit failure - EVR or PFE or Solenoids |
| 559 | (O) | A/C relay primary circuit fault - A/C and Fan Circuits |
| 563 | (O) | High Fan Control (HFC) circuit failure - A/C and Fan Circuits |
| 564 | (O) | Fan Control (FC) circuit failure - A/C and Fan Circuits |
| 565 | (O) | Canister Purge 1 solenoid/circuit failure - Solenoids |
| 566 | (O) | transmission 3/4 shift solenoid/circuit - Transmissions |
| 569 | (O) | Canister Purge 2 solenoid/circuit failure - Solenoids |
| 578 | (M) | A/C pressure sensor VREF short to ground - A/C and Fan Circuits |
| 579 | (M) | ACP sensor did not change with A/C on - A/C and Fan Circuits |
| 581 | (M) | Cooling fan current was excessive - A/C and Fan Circuits |
| 582 | (O) | Open cooling fan circuit - A/C and Fan Circuits |
| 583 | (M) | Fuel pump current was excessive - Power / Fuel Pump Circuits |
| 584 | (M) | Open power ground circuit - Power / Fuel Pump Circuits |
| 585 | (M) | A/C clutch current was excessive - A/C and Fan Circuits |
| 586 | (M) | Open circuit in A/C clutch - A/C and Fan Circuits |
| 587 | (O, M) | Communication problem between PCM and Variable Control Relay Module (VCRM) - Power / Fuel Pump Circuits |
| 617 | (M) | Transmission shift failure (1/2 shift) - Transmissions |
| 618 | (M) | Transmission shift failure (2/3 shift) - Transmissions |
| 619 | (M) | Transmission shift failure (3/4 shift) - Transmissions |
| 621 | (O) | Solenoid/circuit failure - shift solenoid 1 - Transmissions |
| 622 | (O) | Solenoid/circuit failure - shift solenoid 2 - Transmissions |
| 624 | (O,M) | Solenoid/circuit failure -Electronic Pressure Control (EPC) current is high - Transmissions |
| 625 | (O,M) | Solenoid/circuit failure - Electronic Pressure Control (EPC) current is low - Transmissions |
| 626 | (O) | Transmission Coast Clutch (CCS) Solenoid/circuit fault - Transmissions |
| 627 | (O) | Torque Converter Clutch circuit fault - Transmissions |
| 628 | (M) | Excessive converter clutch slippage - Transmissions |
| 629 | (O,M) | Torque Converter Clutch circuit fault - Transmissions |
| 631 | (O) | Overdrive Cancel Light circuit problem - Transmissions |
| 632 | (R) | E4OD - Transmission Control Switch (TCS) should be cycled once between engine ID and Goose test |
| 633 | (O) | 4x4L switch should be in 4x2 or 4x4 high for the test |
| 634 | (O,M) | Park/Neutral Position (PNP) or Clutch Pedal Position (CPP) circuit fault Electronic shift transmission - Manual Lever Position (MLP) sensor out of range in Park-Transmissions |
| 636 | (O,R) | Transmission Oil Temperature (TOT) sensor out of range - Transmissions |
| 637 | (O,M) | TOT sensor is/was high or open - Transmissions |
| 638 | (O,M) | TOT sensor is/was low or grounded - Transmissions |
| 639 | (R,M) | Transmission Speed sensor (TSS) circuit fault - Transmissions |
| 641 | (O) | Transmission solenoid/circuit failure Shift Solenoid 3 - Transmissions |
| 643 | (O,M) | Torque Converter Clutch (TCC) circuit - Transmissions |
| 645 | (M) | Transmission 1st gear failure - Transmissions |
| 646 | (M) | Transmission 2nd gear failure - Transmissions |
| 647 | (M) | Transmission 3rd gear failure - Transmissions |
| 648 | (M) | Transmission 4th gear failure - Transmissions |
| 649 | (M) | Transmission EPC system failure - Transmissions |
| 651 | (M) | Transmission EPC solenoid/circuit fault - Transmissions |
| 652 | (O) | Torque Converter Clutch (TCC) circuit fault - Transmissions |
| 654 | (O) | Transmission selector not in PARK - Transmissions |
| 656 | (M) | Torque Converter Clutch (TCC) slip - Transmissions |
| 657 | (M) | Transmission temperature was excessive - Transmissions |

## 88–95 1.3L EFI, 1.6L, 1.8L, 2.0L w/4EAT & 2.5L

| 01 | IDM |
| --- | --- |
|  | PIP Circuit{3} |
| 02 | IDM{1} |
|  | CP Sensor{2} |
| 03 | CID Sensor |
| 04 | CID Sensor |
| 05 | Knock Signal |
| 06 | VSS |
| 08 | VAF Signal |
| 09 | ECT Sensor |
| 10 | VAT Sensor |
| 11 | IAT Sensor |
| 12 | TP Sensor |
| 14 | BP Sensor |
| 15 | Oxygen Sensor Voltage |
| 16 | EGRT/EVP |
| 17 | Oxygen Sensor Voltage |
| 25 | FPRC Solenoid |
| 26 | CANP Solenoid |
| 28 | EVR/EGRC Solenoid |
| 29 | ERGV Solenoid |
| 34 | ISC Solenoid |
| 41 | HSIA Control Solenoid |
| 42 | Turbo Boost Control Solenoid |
| 55 | PSG |
|  | TCS Sensor |
| 56 | TOT Sensor |
| 60 | SS1 |
| 61 | SS2 |
| 62 | SS3 |
| 63 | TCCC Solenoid |
| 64 | DSS |
| 65 | TCC Solenoid |
| 66 | Line Pressure Solenoid |
| 67 | LFAN Relay |
| 68 | HFAN Relay |
| 69 | ECTF Sensor |

{1}—1990 1.3L engine.
{2}—1.6L, 2.0L w/4EAT & 1991–95 1.3L engine.
{3}—Turbo.

## Probe - Accessing codes

With Analog Voltmeter

Images used with permission Copyright © 2001 ALLDATA LLC.

NOTE: Correct results of the Quick tests are dependent on the proper operation of related non-EEC components. It may be necessary to disconnect or disassemble harness connector assemblies to do some of the inspections. Pin locations should be noted prior to disassembly.

- Perform the following safety precautions to start and run vehicle tests.

  - Apply parking brake.

  - Place gear select lever into PARK (A/T) position (NEUTRAL on M/T).

  - Block drive wheels.

- Inspect the air cleaner and inlet ducting.

- Check all engine vacuum hoses for damage, leaks, cracks, blockage, proper routing, etc.

- Check EEC system wiring harness for proper connections, bent or broken pins, corrosion, loose wires, proper routing, etc.

- Check the Electronic Control Assembly (ECA), sensors, and actuators for physical damage.

- Check the engine coolant for proper level.

- Check the transmission fluid level and quality.

- Make all necessary repairs prior to continuing with test preparation.

- Turn OFF all electrical loads (radio, lights, A/C-heater blower fans, etc.)

- Turn the ignition switch OFF.

- Connect analog voltmeter positive (+) lead to the EEC STO terminal of the diagnostic connector and the negative (-) lead to engine ground.

- Jumper EEC STI of the diagnostic connector to engine ground.

- Set the analog voltmeter on a DC voltage range to read from 0 to 20 volts.

### Key on Engine Off test

Using Analog Voltmeter or Check Engine Light

- START engine and bring up to operating temperature.

- Turn the key OFF, wait ten seconds.

- Turn analog voltmeter ON. Turn ignition key ON.

- Observe analog voltmeter needle sweeps for any code indications.

- Record all codes received.

- Turn the ignition key OFF. IF NO CODES ARE STORED AND ENGINE STARTS, proceed with Key On Engine Running Self Test.

### Key on Engine running test

Using Analog Voltmeter or Check Engine Light

- Run engine at 2000 rpm for two minutes.

- Turn engine OFF, and wait 10 seconds.

- START engine.

- Run engine at idle.

- Jumper STI on diagnostic connector to ground.

- Record service codes.

NOTE: If codes received in Key ON Engine Off (KOEO) self test from the first time cannot be duplicated it may be necessary to tap suspected sensors, shake and wiggle component harness, or drive the vehicle in order to induce a failure.

| Probe codes |  |
| --- | --- |
| 01 | Ignition Diagnostic Monitor IDM |
| 08 | Vane Air Flow Meter VAF |
| 09 | Engine Coolant Temperature ECT |
| 10 | Vane Air Temperature VAT |
| 12 | Throttle Position Sensor TP |
| 14 | Barometric Pressure Sensor BP |
| 15 | Exhaust Gas Oxygen EGO |
| 16 | EGR Valve Position EVP |
| 17 | Exhaust Gas Oxygen EGO |
| 25 | Solenoid Controlled By Ground SCG |
| 26 | Solenoid Controlled By Ground SCG |
| 28 | Solenoid Controlled By Ground SCG |
| 34 | Solenoid Controlled By Ground SCG |

## EEC, Villager

93–95

| 11 | Camshaft Position Sensor |
| --- | --- |
| 12 | Mass Air Flow Sensor |
| 13 | ECT Sensor |
| 14 | Vehicle Speed Sensor |
| 21 | Ignition Signal |
| 31 | Powertrain Control Module |
| 32 | EGR Control Solenoid |
| 33 | Heated Oxygen Sensor |
| 34 | Knock Sensor |
| 35 | EGR Temperature Sensor |
| 43 | Throttle Position Sensor |
| 45 | Injector Leak |
| 51 | Injector Signal |
| 55 | Pass Code |

## 96–97 1.3L EFI, 1.8L & 2.5L

| P0100 | Mass Air Flow Sensor Or Measuring Core Volume Air Flow Sensor Circuit |
| --- | --- |
| P0110 | Intake Air Temperature Sensor Circuit |
| P0115 | Engine Coolant Temperature Circuit |
| P0120 | Throttle Position Sensor Circuit |
| P0125 | Insufficient Coolant Temperature For Closed Loop Control |
| P0130 | O2S Or HO2S Circuit (Bank 1 Sensor 1) |
| P0134 | O2S Or HO2S Circuit No Activity Sensed (Bank 1 Sensor 1) |
| P0135 | HO2S Heater Circuit |
| P0136 | O2S Or HO2S Circuit Malfunction (Bank 1 Sensor 2) |
| P0140 | O2S Or HO2S Circuit No Activity Sensed (Bank 1 Sensor 2) |
| P0141 | O2S Or HO2S Circuit Malfunction (Bank 1 Sensor 2) |
| P0146 | O2S Or HO2S Circuit Malfunction (Bank 1 Sensor 3) |
| P0147 | O2S Or HO2S Circuit Malfunction (Bank 1 Sensor 3) |
| P0150 | HO2S Circuit (Bank 2 Sensor 1) |
| P0154 | O2S Or HO2S Circuit No Activity Sensed (Bank 2 Sensor 2) |
| P0155 | HO2S Heater Circuit (Bank 2 Sensor 1) |
| P0160 | HO2S Circuit (Bank 2 Sensor 2) |
| P0170 | Fuel Trim Malfunction (Bank 1) |
| P0173 | Fuel Trim Malfunction (Bank 2) |
| P0300 | Random Or Multiple Cylinder Misfire Detected |
| P0301–P0306 | Misfire Detected Cylinders 1–6 |
| P0325 | Knock Sensor Circuit |
| P0335 | Camshaft Position Sensor Circuit |
| P0340 | Camshaft Position Sensor Circuit |
| P0400 | EGR Flow Malfunction |
| P0404 | EGR Circuit Range/Performance Malfunction |
| P0420 | Catalyst Efficiency Below Threshold (Bank 1) |
| P0430 | Catalyst Efficiency Below Threshold (Bank 2) |
| P0440 | Evaporative Emission Control System |
| P0441 | Evaporative Emission Control System Malfunction |
| P0443 | Evaporative Emission Control System Purge Control Valve Malfunction |
| P0470 | EGR Boost Sensor (1.3L) |
| P0500 | Vehicle Speed Sensor Circuit |
| P0505 | Idle Control System Malfunction |
| P0510 | Closed Throttle Position Switch Malfunction |
| P0703 | Brake On/Off Switch |
| P0705 | Transmission Range Sensor Circuit Malfunction Short |
| P0706 | Transmission Range Sensor Circuit Malfunction Open |
| P0710 | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P0711 | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P0715 | Turbine Speed Sensor Circuit |
| P0731 | Improper Gear 1 Ratio |
| P0732 | Improper Gear 2 Ratio |
| P0733 | Improper Gear 3 Ratio |
| P0734 | Improper Gear 4 Ratio |
| P0740 | Torque Converter Clutch Circuit |
| P0745 | Pressure Control Solenoid Malfunction |
| P0750 | Shift Solenoid No. 1 |
| P0755 | Shift Solenoid No. 2 |
| P0760 | Shift Solenoid No. 3 |
| P1000 | OBD II Readiness Function Code |
| P1169 | Feedback A/F Mixture Control (H02S12) |
| P1170 | HO2S Voltage Fixed |
| P1173 | HO2S Inversion (Front LH) |
| P1173 | HO2S Voltage Fixed |
| P1195 | Barometric Pressure Sensor Circuit (Signal From EGR Boost Sensor) |
| P1196 | Starter Switch Circuit |
| P1250 | Fuel Pressure Regulator Control Solenoid |
| P1345 | Cylinder Discrimination Signal (Signal From Camshaft Position Sensor) |
| P1402 | EGR Valve Position Sensor |
| P1485 | EGR Control (EGRC) Solenoid |
| P1486 | EGR Control (EGRV) Solenoid |
| P1487 | EGR Check Solenoid |
| P1521 | Variable Resonance Induction System Solenoid No. 1 Malfunction |
| P1522 | Variable Resonance Induction System Solenoid No. 2 Malfunction |
| P1523 | High Speed Inlet Air Solenoid Malfunction |
| P1608 | PCM Internal Circuit Malfunction |
| P1609 | PCM Internal Circuit Malfunction |
| P1709 | Throttle Position Sensor malfunction |
| P1720 | Vehicle Speed Sensor Malfunction |
| P1743 | Torque Converter Clutch Control Solenoid Malfunction |
| P1744 | Torque Converter Clutch Solenoid Malfunction |
| P1765 | 3–2 Timing Solenoid Malfunction |
| P1792 | Idle Switch Malfunction |
| P1794 | Loss Of Battery Voltage |
| P1795 | EGR Boost Sensor Malfunction |
| P1797 | Clutch Pedal Position Switch Or Neutral Switch Malfunction |

## 96–01

| OBD2 | MIL |  |
| --- | --- | --- |
| P0000 | 0505 | No Failures |
| P0100 | 0102 | MAF Sensor Circuit Malfunction |
| P0105 | 0803 | MAP/BARO Sensor Circuit Malfunction |
| P0110 | 0401 | IAT Sensor Circuit |
| P0115 | 0103 | ECT Sensor Circuit Malfunction |
| P0120 | 0403 | TP Sensor Circuit Malfunction |
| P0125 | 0908 | Insufficient Coolant Temperature For Closed Loop Control |
| P0130 | 0303 | HO2S Circuit Malfunction Bank 1 Sensor 1 |
| P0131 | 0411 | HO2S Bank 1 Sensor 1 Lean |
| P0132 | 0410 | HO2S Bank 1 Sensor 1 Rich |
| P0133 | 0409 | HO2S Bank 1 Sensor 1 |
| P0134 | 0412 | HO2S Bank 1 Sensor 1 |
| P0135 | 0901 | HO2S Heater Circuit Malfunction Bank 1 Sensor 1 |
| P0136 | 0707 | HO2S Circuit Malfunction Bank 1 Sensor 2 |
| P0137 | 0511 | HO2S Bank 1 Sensor 2 Lean |
| P0138 | 0510 | HO2S Bank 1 Sensor 2 Rich |
| P0139 | 0707 | HO2S Bank 1 Sensor 2 Response |
| P0140 | 0512 | HO2S Bank 1 Sensor 2 |
| P0141 | 0902 | HO2S Heater Circuit Malfunction Bank 1 Sensor 2 |
| P0171 | 0115 | Fuel System Too Lean |
| P0172 | 0114 | Fuel System Too Rich |
| P0180 | 0402 | Fuel Temperature Sensor Circuit Malfunction |
| P0300 | 0701 | Random/Multiple Cylinder Misfire Detected |
| P0301 | 0608 | Cylinder No. 1 Misfire Detected |
| P0302 | 0607 | Cylinder No. 2 Misfire Detected |
| P0303 | 0606 | Cylinder No. 3 Misfire Detected |
| P0304 | 0605 | Cylinder No. 4 Misfire Detected |
| P0305 | 0604 | Cylinder No. 5 Misfire Detected |
| P0306 | 0603 | Cylinder No. 6 Misfire Detected |
| P0325 | 0304 | Knock Sensor Circuit Malfunction |
| P0335 | 0802 | CKP Sensor Circuit Malfunction |
| P0340 | 0101 | CMP Sensor Circuit Malfunction |
| P0400 | 0302 | EGR Flow Malfunction |
| P0402 | 0306 | EGR Flow Excessive |
| P0420 | 0702 | Catalyst System Efficiency Below Threshold Bank 1 |
| P0440 | 0705 | EVAP System Leak Detected, Small Leak |
| P0443 | 1008 | EVAP System Purge Volume Control Valve Circuit Malfunction |
| P0446 | 0903 | EVAP System Vent Control Circuit Malfunction |
| P0450 | 0704 | EVAP System Pressure Sensor Malfunction |
| P0500 | 0104 | VSS Malfunction |
| P0505 | 0205 | Idle Control System Malfunction |
| P0510 | 0203 | Closed TP Sensor Malfunction |
| P6000 | 0504 | Serial Communication Link Malfunction |
| P0605 | 0301 | PCM ROM Error |
| P0705 | 1101 | Transmission Range Selector Circuit Malfunction |
| P0710 | 1208 | Transmission Fluid Temperature Circuit Malfunction |
| P0720 | 1102 | Output Speed Sensor Circuit Malfunction |
| P0725 | 1207 | Engine Speed Input Circuit Malfunction |
| P0731 | 1103 | Gear 1 Improper Ratio |
| P0732 | 1104 | Gear 2 Improper Ratio |
| P0733 | 1105 | Gear 3 Improper Ratio |
| P0734 | 1106 | Gear 4 Improper Ratio |
| P0740 | 1204 | Torque Converter Clutch Circuit Malfunction |
| P0744 | 1107 | Torque Converter Clutch Solenoid Malfunction |
| P0745 | 1205 | Pressure Control Solenoid Malfunction |
| P0750 | 1108 | Shift Solenoid No. 1 Malfunction |
| P0755 | 1201 | Shift Solenoid No. 2 Malfunction |
| P1105 | 1302 | MAP/BARO Solenoid Circuit Malfunction |
| P1148 | 0307 | Closed Loop Control Malfunction |
| P1320 | 0201 | Distributor Signal Interrupt |
| P1336 | 0905 | CKP Sensor |
| P1400 | 1005 | DPFE EGR Sensor Circuit Low Voltage Detected |
| P1401 | 0305 | EGR Temperature Sensor |
| P1402 | 0514 | EGR System High Flow Rate |
| P1440 | 0213 | EVAP System Leak Detected, Small Leak |
| P1444 | 0214 | EVAP System Volume Control Valve |
| P1446 | 0215 | EVAP System Vent Control Closed |
| P1447 | 0111 | EVAP System Purge Flow Monitor |
| P1448 | 0309 | EVAP System Vent Control Open |
| P1490 | 0801 | EVAP System Vacuum Cut Bypass Solenoid Circuit Malfunction |
| P1491 | 0311 | EVAP System Vacuum Cut Bypass Solenoid Malfunction |
| P1605 | 0804 | A/T Diagnostic Communication Link |
| P1705 | 1206 | TP Sensor (A/T) |
| P1706 | 1003 | Park/Neutral Switch Not Switching |
| P1760 | 1203 | Overrun Clutch Solenoid Valve |
| P1900 | 1308 | Cooling Fan |

# EEC, Except Villager

1.3L FBC Engine

| 01 | IDM |
| --- | --- |
| 09 | ECT Sensor |
| 13 | MAP Sensor |
| 15 | EGO Sensor |
| 16 | EVP Sensor |
| 17 | EGO Sensor Feedback |
| 18 | Feedback Carburetor Actuator |
| 19 | Feedback Carburetor Actuator |
| 20 | Feedback Carburetor Actuator |
| 21 | Feedback Carburetor Actuator |
| 28 | EGR Control Solenoid |
| 29 | EGR Vent Solenoid |
| 34 | Electric Load Control Solenoid |
| 35 | A/C Load Control Solenoid |
| 38 | Early Fuel Evaporation Heater |
| 70 | WOT Vacuum Switch |

## EEC-IV

C—Continuous memory code.
O—Key On, engine Off self test code.
R—Engine running self test code.

| MIL |  | Type |
| --- | --- | --- |
| 11 | System Pass | C/O/R |
| 12 | RPM Not Within Self Test Upper Limit | R |
| 13 | RPM Not Within Self Test Lower Limit | R |
|  | DC Motor Did Not Move | C/O/R |
| 14 | Profile Ignition Pickup Circuit Failure | C |
|  | Engine RPM Sensor Circuit Fault (Diesel) | C |
| 15 | Readout Memory Test Failed | O |
|  | Keep Alive Memory Test Failed | C |
| 16 | Ignition Diagnostic Module (2.9L, 3.0L & 4.0L) | O |
|  | Idle RPM High With ISC Retracted (2.5L) | R |
|  | RPM Above Self Test Limit With ISC Off (1986–90 1.9L EFI) | R |
|  | RPM Too Low To Perform HO2S Test (Others) | R |
| 17 | Idle RPM Low With ISC Retracted (1.9L & 2.5L CFI) | R |
|  | RPM Below Self Test Limit With ISC Off (1986–90 1.9L EFI) | R |
| 18 | Spark Out (SPOUT) Circuit Open | R |
|  | Loss Of Ignition Diagnostic Module Input To PCM/SPOUT Circuit Grounded | C |
|  | Spark Angle Word (SAW) Circuit Failure (1.9L SFI) | C/R |
| 19 | Failure In PCM Internal Voltage | R |
|  | Erratic RPM During Hard Idle Self Test | R |
|  | RPM Drop Too Low During ISC Off Test (1.9L EFI) | R |
|  | Power Processor Check (2.3L HSC EFI) | O |
|  | Cylinder Identification Sensor Input Failure (3.0L SHO) | C |
| 21 | Engine Coolant Temperature (ECT) Sensor Out Of Self Test Range | O/R |
| 22 | Manifold Absolute Pressure/Barometric Pressure Sensor Out Of Self Test Range | C/O/R |
| 23 | Throttle Position Sensor Out Of Self Test Range | C/O/R |
|  | Fuel Injector Pump Lever Sensor Input Is Out Of Self Test Range (Diesel) | O/R |
| 24 | Intake Air Temperature/Air Charge Temperature Sensor Out Of Self Test Range | O/R |
| 25 | Knock Not Sensed During Dynamic Response Test | R |
| 26 | Mass Air Flow/Vane Air Flow Sensor Out Of Self Test Range | O/R |
|  | Transmission Fluid Temperature Sensor Input Out Of Self Test Range (5.0L, 5.8L, 7.3L & 7.5L Truck) | O/R |
| 27 | Vehicle Speed Sensor (2.3L Turbo) | C |
| 28 | Loss Of Ignition Diagnostic Module RH Side | C |
|  | Vane Air Temperature Sensor Input Out Of Self Test Range (1.9L EFI) | O/R |
| 29 | Insufficient Input From Programmable Speedometer/Odometer Module (From 1993) | C |
|  | Insufficient Input From Vehicle Speed Sensor (To 1992) | C |
| 31 | EGR Valve Position/Pressure Feedback EGR Circuit Below Minimum Voltage | C/O/R |
| 32 | EGR Not Controlling (2.3L OHC & 3.8L CFI) | R |
|  | EGR Valve Not Seated (1.9L CFI, 2.9L & 3.0L SHO) | C/R |
|  | EGR Valve Position/Pressure Feedback EGR Voltage Below Closed Voltage (Others) | C/O/R |
| 33 | EGR Not Closing Properly (2.3L OHC & 3.8L CFI) | C/R |
|  | Throttle Position Sensor Noisy/Harsh On Line (Diesel) | C |
|  | EGR Valve Opening Not Detected (Others) | C/R |
| 34 | EGR Valve Opening Not Detected (2.3L OHC & 3.8L CFI) | R |
|  | Defective EGR Pressure Transducer Sensor (1.9L CFI, 2.3L HSC EFI & 3.0L) | C/O/R |
|  | EGR On/Off Control (2.3L Turbo) | R |
|  | EGR Valve Position/Pressure Feedback EGR Voltage Above Closed Limit (Others) | C/O/R |
| 35 | RPM Too Low To Perform EGR Test (2.3L OHC & 3.8L CFI) | R |
|  | EGR Valve Position/EGR Pressure Feedback EGR Circuit Above Maximum Voltage (Others) | C/O/R |
| 36 | System Indicates Lean At Idle | R |
| 37 | System Indicates Rich At Idle | R |
| 38 | DC Motor Idle Speed Control/Idle Tracking Throttle Position Sensor Open Circuit (2.3L CFI & 2.5L HSC) | C |
| 39 | AXOD Converter Bypass Clutch Not Applying Properly | C |
| 41 | System Indicates Lean | R |
|  | No HO2S Switching Detected | C |
| 42 | System Indicates Rich | R |
|  | No O2S Switching Detected | C |
| 43 | HO2S Lean At Wide Open Throttle (1986–90 1.9L MFI) | C |
|  | Throttle Position Sensor Below Idle Sped (Diesel) | C |
| 44 | Secondary Air System Inoperative | R |
| 45 | Secondary Air Upstream During Self Test | R |
|  | Coil 1 Primary Circuit Failure (1990–91 1.9L EFI/SFI, ) | C |
|  | Distributorless Ignition System Coil Pack 3 Circuit Failure (3.0L SHO 3.8L SC) | C |
| 46 | Secondary Air Not Bypassed During Self Test | R |
|  | Coil 2 Primary Circuit Failure (1990–91 1.9L MFI/SFI) | C |
|  | Distributorless Ignition System Coil Pack 1 Circuit Failure (3.0L SHO 3.8L SC) | C |
| 47 | 4 X 4 Switch Is Closed | O |
|  | Measured Air Flow Low At Base Idle | R |
| 48 | Loss Of Ignition Diagnostic Monitor LH Side (2.3L) | C |
|  | Measured Air Flow High At Base Idle (1986–90 1.9L EFI) | R |
|  | Distributorless Ignition System Coil Pack 2 Circuit Failure (3.0L SHO 3.8L SC) | C |
| 49 | Spark Output (SPOUT) Signal Default To 10°. BDTC (3.0L SHO & 3.8L SC) | C |
|  | 1–2 Shift Error (Others) | C |
| 51 | Engine Coolant Temperature Circuit Open | O/C |
| 52 | Power Steering Pressure Switch Circuit Open | O |
|  | Power Steering Pressure Switch Circuit Did Not Change States | R |
| 53 | Throttle Position Sensor Circuit Above Maximum Voltage | C/O |
|  | Fuel Injector Pump Lever Sensor Input Is Greater Than Self Test (Diesel) | C/O |
| 54 | Intake Air Temperature/Air Charge Temperature Circuit Open | C/O/R |
| 55 | Key Power Check | R |
| 56 | Transmission Fluid Temperature Circuit Open | C/O |
|  | Mass Air Flow/Vane Air Flow Sensor Circuit Above Maximum Voltage | C/O/R |
| 57 | Transaxle Neutral Position Switch Circuit Failed Open | C |
| 58 | Idle Tracking Switch Circuit Open Or Grounded (CFI) | O/R |
|  | Vane Air Temperature Sensor Input Greater Than Self Test Maximum (EFI) | C/O |
| 59 | AXOD Neutral Position Switch Circuit Failed Open (3.0L & 3.8L) | C |
|  | 2–3 Shift Error (Others) | C |
|  | Low Speed Fuel Pump Circuit Failure, Battery To PCM (3.0L SHO) | C/O |
|  | AXOD 4/3 Pressure Switch Circuit Failed Closed | O |
|  | AXOD 4/3 Pressure Switch Circuit Failed Open | C |
| 61 | ECT Sensor Circuit Grounded | C/O/R |
| 62 | Torque Converter Clutch Error | C |
|  | AXOD 4/3 Or 3/2 Pressure Switch Circuit Grounded | O |
| 63 | Throttle Position Sensor Circuit Below Minimum Voltage | C/O/R |
|  | Fuel Injection Pump Lever Sensor Input Is Less Than Self Test Min. (Diesel) | C/O |
| 64 | Intake Air Temperature/Air Charge Temperature Circuit Grounded | C/O/R |
| 65 | Transmission Control Switch/Overdrive Cancel Switch Circuit Did Not Change States | R |
|  | Fuel Control (2.3L Turbo) | C |
| 66 | Mass Air Flow Sensor Circuit Below Minimum Voltage (Except 1.9L MFI & 2.3L Turbo) | C |
|  | Vane Air Flow Sensor Input Less Than Self Test Minimum (1.9L MFI & 2.3L Turbo) | O/C |
|  | Transmission Fluid Temperature Circuit Grounded (Others) | O/C |
| 67 | Park Neutral Position Switch Circuit Open; A/C On (Manual) | O |
|  | Manual Lever Position Sensor Out Of Range/A/C On | O/C |
|  | Clutch Switch Circuit Failure | C |
|  | A/C Input High (2.5L TBI) | O/C |
| 68 | Idle Tracking Switch Closed (1.9L, 2.3L, 2.5L & 3.8L CFI) | O/R |
|  | Vane Air Temperature Sensor Input Less Than Self Test Minimum (1.9L MFI & 2.3L Turbo) | O/C |
|  | Transmission Fluid Temperature Transmission Over Temp (Overheated) (Diesel) | C |
| 69 | 3–4 Shift Error | C |
|  | AXOD Neutral Position Switch Failed Open (3.0L) | C |
|  | AXOD 3/2 Pressure Switch Failed Closed (3.8L SFI) | C/O |
| 70 | EEC IV Data Transaxle Circuit Failed (3.8L SFI) | C |
| 71 | Idle Tracking Switch Closed On Pre-Position (1.9L, 2.3L & 2.5L CFI) | C |
|  | Software Re-Initalization Detected (1.9L EFI) | C |
|  | Cluster Center Control Circuit Failed (3.8L SFI) | C |
| 72 | Insufficient Manifold Absolute Pressure/Mass Air Flow Change During Dynamic Response Test | R |
|  | Power Interrupt Software Re-Initalization Detected (1.9L & 2.3L Turbo) | C |
|  | Message Center Control Assembly Circuit Failed (3.8L SFI) | C |
| 73 | Insufficient Throttle Position Change During Dynamic Response Test | O/R |
| 74 | Brake On/Off Circuit Open–Not During Self Test | R/C |
| 75 | rake On/Off Circuit Closed/ECA Input Open | R |
| 76 | Insufficient Vane Air Flow Sensor Output Change During Dynamic Response Test | R |
| 77 | Operator Error Dynamic Response Test | R |
| 78 | Re-Initalization Check | R |
| 79 | A/C On/Defrost On During Self Test | O |
| 81 | Secondary Air Diverter Circuit Failure | O |
|  | Turbo Boost (2.3L Turbo) | O |
|  | Insufficient IAS Output Voltage Change When Solenoid Is Activated (3.0L SHO) | O |
| 82 | Secondary Air Bypass Circuit Failure | O |
|  | Integrated Relay Control Module (2.3L Turbo) | O |
|  | Supercharger Bypass Circuit Failure (3.8L SC) | O |
| 83 | Integrated Relay Control Module (2.3L Turbo & 2.5L) | O |
|  | High Electric Drive Fan Circuit Failure (1.9L & 2.5L SFI) | O |
|  | EVP/EGRC/EGRV (2.3L OHC & 3.8L CFI) | O |
|  | Low Speed Fuel Pump Relay Circuit Open (3.0L SHO) | C/O |
|  | Integrated Relay Control Module (3.8L Turbo) | O |
|  | High Electric Drive Fan Circuit Failure (3.8L SFI & 3.8L SC) | O |
| 84 | EGR Valve Position/Pressure Feedback EGR/EGR Control Circuit Failure | O |
| 85 | Adaptive Fuel Lean Limit Reached (1.9L EFI) | C |
|  | Shift Solenoid 3/4–4/3 (2.3L Turbo) | O |
|  | CANP Circuit Failure (Others) | O |
| 86 | Shift Solenoid Circuit Failure (2.9L 3.0L & 4.0L) | O |
|  | Adaptive Fuel Rich Limit Reached (1.9L EFI) | C |
| 87 | Primary Fuel Pump Circuit Failure | O/C |
|  | Integrated Relay Control Module (2.3L Turbo & 2.5L & 3.8L MFI) | O |
| 88 | Loss Of Dual Plug Input Control | C |
|  | Shift Solenoid 3/4–4/3 (2.3 Turbo) | O |
|  | Integrated Relay Control Module (2.5 L HSC & 3.8L EFI) | O |
|  | Electric Drive Fan Circuit Failure (2.5L, 3.0L SHO & 3.8L SFI) | O |
|  | Throttle Kicker Fault (5.0L) | — |
| 89 | Converter Clutch Override Circuit Failure (2.9L, 3.0L & 4.0L) | O |
|  | Exhaust Heat Control (3.8L CFI) | O |
| 91 | Shift Solenoid 1 Circuit Failure | O |
|  | HO2S Circuit Indicates Lean | R |
|  | No HO2S Switching Detected | C |
| 92 | Shift Solenoid 2 Circuit Failure | O |
|  | HO2S Circuit Indicates Rich | R |
| 93 | Throttle Position Sensor Input Low At Maximum D.C. Motor Extension (1.9L, 2.3L & 2.5L TBI) | O |
|  | Converter Clutch Solenoid Circuit Failure (Others) | O |
| 94 | Torque Converter Clutch Solenoid Circuit Failure | O |
|  | Secondary Air Injection Inoperative | R |
| 95 | Integrated Relay Control Module (2.5L HSC & 3.8L EFI) | C/O |
|  | Fuel Pump Circuit Open-PCM To Motor Ground (Others) | C/O |
| 96 | Integrated Relay Control Module (2.5L HSC & 3.8L EFI) | C/O |
|  | High Speed Fuel Pump Relay Circuit Open (3.0L SHO) | C/O |
|  | Fuel Pump Circuit Open-Battery To PCM (Others) | C/O |
| 97 | Transmission Control Indicator Lamp Circuit Failure | O |
| 98 | Hard Fault Present | R |
|  | Electronic Pressure Control Driver Open In PCM (Diesel) | O |
| 99 | Electronic Pressure Control Circuit Failure | O/C |
|  | Electronic Engine System Has Not Learned To Control Idle | R |
| 111 | System Pass | C/O/R |
| 112 | Intake Air Temperature/Air Charge Temperature Sensor Circuit Below Minimum Voltage | C/O |
| 113 | Intake Air Temperature/Air Charge Temperature Sensor Circuit Above Maximum Voltage | C/O |
| 114 | Intake Air Temperature/Air Charge Temperature Higher Or Lower Than Expected | O/R |
| 116 | Engine Coolant Temperature Higher Or Lower Than Expected | O/R |
| 117 | ECT Sensor Circuit Below Minimum Voltage | C/O |
| 118 | ECT Sensor Circuit Above Maximum Voltage | C/O |
| 121 | Closed Throttle Voltage Higher Or Lower Than Expected | C/O/R |
| 121 | Indicates Throttle Position Voltage Inconsistent With Mass Air Flow Sensor | C/O/R |
| 122 | Throttle Position Sensor Circuit Below Minimum Voltage | C/O |
| 123 | Throttle Position Sensor Circuit Above Maximum Voltage | C/O |
| 124 | Throttle Position Sensor Voltage Higher Than Expected | C |
| 125 | Throttle Position Sensor Voltage Lower Than Expected | C |
| 126 | Manifold Absolute Pressure/Barometric Pressure Sensor Higher Or Lower Than Expected | C/O/R |
|  | Cylinder Identification Circuit Failure (3.0L SHO) | C |
| 128 | Manifold Absolute Pressure Sensor Vacuum Hose Damaged/Disconnected | C |
| 129 | Insufficient Manifold Absolute Pressure/Mass Air Flow Change During Dynamic Response Test KOER | R |
| 136 | System Indicates Lean (Bank No. 1) | R |
| 137 | System Indicates Rich (Bank No. 2) | R |
| 138 | Cold Start Injector Flow Insufficient | R |
| 139 | No HO2S Switch Detected | C |
| 141 | Flexible Fuel Control | R |
| 144 | No HO2S Switch Detected | C |
| 157 | Mass Air Flows Sensor Circuit Below Minimum Voltage | C |
| 158 | Mass Air Flow Sensor Circuit Above Maximum Voltage | C/O |
| 159 | Mass Air Flow Higher Or Lower Than Expected | O/R |
| 167 | Insufficient Throttle Position Change During Dynamic Response Test | R |
| 171 | Fuel System At Adaptive Limits | C |
|  | No HO2S Switching Detected | C |
| 172 | HO2S System Indicates Lean | C/R |
| 173 | HO2S System Indicates Rich | R |
| 174 | HO2S Switching Time Is Slow | C |
| 175 | Fuel System At Adaptive Limits (Bank No. 2) | C |
| 176 | System Indicates Lean (Bank No. 2) | C |
|  | Insufficient TP Change During Dynamic Response Test (4.9L) | R |
| 177 | System Indicates Rich (Bank No. 2) | C |
| 178 | HO2S Switch Time Is Slow (Bank No. 1) | C |
| 179 | Fuel System At Lean Adaptive Limit At Part Throttle, System Rich (1.9L, 2.0L. 2.3L HSC, 2.5L & 3.0L) | C |
|  | Fuel System Adaptive Lean Limit Is Reached (4.6L, 4.9L 5.0L, 5.8L, 7.0L & 7.5L) | C |
|  | Fuel System At Rich Adaptive Limit (Others) | C |
| 181 | Fuel System At Rich Adaptive Limit At Part Throttle, System Lean (1.9L, 2.0L, 2.3L HSC, 2.5L & 3.0L) | C |
|  | Fuel System Adaptive Rich Limit Is Reached (4.6L, 4.9L, 5.0L, 5.8L ,7.0L & 7.5L) | C |
|  | Fuel System At Lean Adaptive Limit (Others) | C |
| 182 | Fuel System At Lean Adaptive Limit At Idle, System Rich (1.9L, 2.3L, 2.5L, 3.0L, 3.8L SC, 4.6L ,7.0L & 7.5L) | C |
|  | Fuel System At Lean Adaptive Limit (Others) | C |
| 183 | Fuel System At Rich Adaptive Limit At Idle, System Lean (1.9L, 2.3L, 3.0L, 3.8L SC, 4.6L, 7.0L & 7.5L) | C |
|  | Fuel System At Rich Adaptive Limit (Others) | C |
| 184 | Mass Air Flow Sensor Circuit Voltage Higher Than Expected | C |
| 185 | Mass Air Flow Sensor Circuit Voltage Lower Than Expected | C |
| 186 | Injector Pulse Width Higher Than Expected (With BARO Sensor) | C |
|  | Injector Pulse Width Higher Or Mass Air Flow Lower Than Expected (Less BARO Sensor) | C |
| 187 | Injector Pulse Width Lower Than Expected (With BARO Sensor) | C |
|  | Injector Pulse Width Lower Or MAF Higher Than Expected (Less BARO Sensor) | C |
| 188 | Fuel System At Lean Adaptive Limit | C |
| 189 | Fuel System At Rich Adaptive Limit | C |
| 191 | Fuel System At Lean Adaptive Limit (Front) | C |
| 192 | Fuel System At Rich Adaptive Limit (Front) |  |
| 193 | Flexible Fuel Sensor Circuit Failure | O |
| 211 | Profile Ignition Pickup Circuit Failure | C |
| 212 | Loss Of Ignition Diagnostic Module Input To PCM/SPOUT Circuit Grounded | C |
| 213 | Spark Output (SPOUT) Circuit Open | R |
| 214 | Cylinder Identification Circuit Failure | C |
| 215 | PCM Detected Coil 1 Primary Circuit Failure (EI) | C |
| 216 | PCM Detected Coil 2 Primary Circuit Failure (EI) | C |
| 217 | PCM Detected Coil 3 Primary Circuit Failure (EI) | C |
| 218 | Loss Of Ignition Diagnostic Module Signal LH Side (Dual Plug EI) | C |
| 219 | Spark Timing Error | C |
| 221 | Spark Timing Error | C |
| 222 | Loss Of Ignition Diagnostic Module Signal RH Side (Dual Plug EI) | C |
| 223 | Loss Of Dual Plug Ignition Control (Dual Plug EI) | C |
| 224 | PCM Detected Coil 1,2,3 Or 4 Primary Circuit Failure (EI) | C |
|  | Erratic Ignition Diagnostic Module Input To Processor | C |
|  | Loss Of Tack Input To Processor/SPOUT Circuit Grounded (5.8L) | C |
| 225 | Knock Not Sensed During Dynamic Response Test KOER | R |
| 226 | Ignition Diagnostic Module Signal Not Received (EI) | O |
| 232 | PCM Detected Coil 1, 2, 3 Or 4 Primary Circuit Failure (EI) | C |
| 238 | PCM Detect Coil 4 Primary Circuit Failure | C |
| 244 | Cylinder Identification Sensor Circuit Failure (2.0L) | R |
| 311 | AIR System Inoperative/AIR System Inoperative During KOER (Bank No. 1 With Dual HO2S) | R |
| 312 | AIR Misdirected During KOER | R |
| 313 | AIR Not Bypassed During KOER | R |
| 314 | AIR System Inoperative During KOER | R |
| 326 | EGR Temperature Sensor Circuit Above Maximum Voltage (2.0L, 2.3L HSC & 3.0L SHO) | C |
|  | Pressure Feedback EGR Circuit Voltage Low (Others) | C/R |
| 327 | EGR Circuit Below Minimum Voltage | C/O/R |
| 328 | EGR Valve Position Sensor Closed Valve Voltage Lower Than Expected | C/O/R |
| 332 | Insufficient EGR Flow Detected | C/R |
| 334 | EGR Valve Position Sensor Closed Valve Voltage Higher Than Expected | C/O/R |
| 335 | Pressure Feedback EGR Sensor Voltage Higher Or Lower Than Expected During KOEO | O |
| 336 | Pressure Feedback EGR Circuit Voltage Higher Than Expected | C/R |
| 337 | EGR Valve Position/Pressure Feedback EGR Circuit Above Maximum Voltage | C/O/R |
| 338 | Engine Coolant Temperature Lower Than Expected | C |
| 339 | Engine Coolant Temperature Higher Than Expected | C |
| 341 | Octane Adjust Service Pin Open | O |
| 381 | Erratic A/C Compressor Clutch Cycling | C |
| 411 | Cannot Control RPM During KOER Low RPM Check | R |
| 412 | Cannot Control RPM During KOER High RPM Check | R |
| 415 | Idle Air Control Solenoid | C |
| 416 | Idle Air Control Solenoid | C |
| 452 | Insufficient Input From Vehicle Speed Sensor To PCM | C |
| 511 | Read Out Memory Test Failure KOEO | C/O/R |
| 512 | Keep Alive Memory Test Failure | C |
| 513 | PCM Internal Voltage Failure (KOEO) | C/O/R |
| 519 | Power Steering Pressure Switch Circuit Open KOEO | O |
| 521 | Power Steering Pressure Switch Circuit Did Not Change States KOER | R |
| 522 | Vehicle Not In Park Of Neutral During KOEO | O |
| 524 | Low Speed Fuel Pump Circuit Open | C/O |
| 525 | Vehicle In Gear With A/C On | C/O |
| 526 | Neutral Position Switch Circuit Closed, A/C On | O |
| 527 | Neutral Drive Switch Circuit Open, A/C On | O |
| 528 | Clutch Position Switch Circuit Failure | C |
| 529 | Data Communication Link Or Electronic Engine Controller Fault | C |
| 533 | Data Communication Link To Electronic Instrument Cluster Fault | C |
| 536 | Brake On/Off Circuit Failure/Not Actuated During KOER | C/R |
| 538 | Insufficient RPM Change During KOER Dynamic Response Test | R |
| 538 | Invalid Cylinder Balance Test Due To Throttle Movement During Test | R |
| 538 | Invalid Cylinder Balance Test Due To Cylinder Identification Sensor Circuit Failure | R |
| 539 | A/C On/Defrost On During KOEO | O/R |
| 542 | Fuel Pump Circuit Open–PCM To Motor Ground | C/O |
| 543 | Fuel Pump Circuit Open–Battery To Powertrain Control Module (PCM) | C/O |
| 551 | Intake Air Control Circuit Failure KOEO | O |
| 552 | AIR Bypass Circuit Failure KOEO | O |
| 553 | AIR Bypass Circuit Failure KOEO | O |
| 554 | Fuel Pump Relay Control Circuit Failure | O |
| 556 | Fuel Pump Relay Primary Circuit Failure | C/O |
| 557 | Low Speed Fuel Pump Primary Circuit Failure | C/O |
| 558 | EGR Vacuum Regulator Circuit Failure | O |
| 559 | A/C On Relay Circuit Failure | O |
| 563 | High Speed Electric Fan Circuit Failure | O |
| 564 | Electric Fan Circuit Failure | O |
| 565 | Canister Purge Circuit Failure KOEO | O |
| 566 | Shift Solenoid Circuit Failure KOEO | O |
| 569 | Auxiliary Canister Purge Circuit Failure | O |
| 571 | EGR Circuit Failure | O |
| 572 | EGR Circuit Failure | O |
| 578 | A/C Pressure Sensor Circuit | C |
| 579 | Insufficient A/C Pressure Change | C |
| 581 | Power To Fan Circuit Over Current | C |
| 582 | Fan Circuit Open | O |
| 583 | Power To Fuel Pump Over Current | C |
| 584 | Power Ground Circuit Open | C |
| 585 | Power To A/C Clutch Over Current | C |
| 586 | A/C Clutch Circuit Open | C |
| 587 | Vehicle Control Relay Module Communication Failure | C/O |
| 617 | 1–2 Shift Error | C |
| 618 | 2–3 Shift Error | C |
| 619 | 3–4 Shift Error | C |
| 621 | Shift Solenoid 1 Circuit Failure KOEO | C/O |
| 622 | Shift Solenoid 2 Circuit Failure KOEO | C/O |
| 623 | Transmission Control Indicator Lamp Circuit Failure | C/O |
| 624 | Electronic Pressure Control Circuit Failure | C/O |
| 625 | Hard Fault Is Present (5.8L) | C/O/R |
| 625 | Electronic Pressure Control Driver Open In PCM (Others) | C/.O |
| 626 | Converter Clutch Solenoid Circuit Failure KOEO | O |
| 627 | Torque Converter Clutch Solenoid Circuit Failure | O |
| 628 | Excessive Torque Converter Clutch Slippage | C |
| 629 | Torque Converter Clutch Solenoid Circuit Failure | C |
| 631 | Transmission Control Indicator Lamp Circuit Failure KOEO | O |
| 632 | Torque Converter Solenoid Circuit Did Not Change States During KOER | C |
| 633 | 4 X 4 Switch Closed During KOEO | O |
| 634 | Manual Lever Position Sensor Voltage Higher Or Lower Than Expected | C/O |
| 636 | Transmission Fluid Temperature Higher Or Lower Than Expected | O/R |
| 637 | Transmission Fluid Temperature Sensor Circuit Above Maximum Voltage | C/O |
| 638 | Transmission Fluid Temperature Sensor Circuit Below Minimum Voltage | C/O |
| 639 | Insufficient Input From Turbine Speed Sensor | C/R |
| 641 | Shift Solenoid 3 Circuit Failure | C/O |
| 643 | Torque Converter Clutch Circuit Failure | C/O |
| 645 | Improper Gear Ratio In 1st Gear | C |
| 646 | Improper Gear Ratio In 2nd Gear | C |
| 647 | Improper Gear Ratio In 3rd Gear | C |
| 648 | Improper Gear Ratio In 4th Gear | C |
| 649 | Electronic Pressure Control Higher Or Lower Than Expected | C |
| 651 | Electronic Pressure Control Circuit Failure | C |
| 652 | Torque Converter Clutch Solenoid Circuit Failure | C |
| 653 | Torque Converter Solenoid Did Not Change States During KOER | R |
| 654 | Manual Lever Position Sensor Indicating Not In Park During Self Test | O |
| 655 | Manual Lever Position Sensor Indicating Not In Neutral During Self Test | — |
| 656 | Torque Converter Clutch Slippage Error | C |
| 657 | Transmission Over Temp Condition | C |
| 659 | High Vehicle Speed In Park Indicated | R |
| 662 | Shift Solenoid 2 Circuit | O |
| 667 | Transmission Range Sensor Circuit Below Minimum Voltage | C/O |
| 668 | Transmission Range Sensor Circuit Above Maximum Voltage | C/O |
| 675 | Transmission Range Sensor Circuit Voltage Out Of Range | O/R |
| 998 | Hard Fault Present | O/R |

96 & newer vehicles are OBD2 systems

## EEC-V

| P0102 | MAF Circuit Low Input |
| --- | --- |
| P0103 | MAF Circuit High Input |
| P0106 | BARO Sensor Circuit Performance |
| P0107 | BARO Sensor Low Voltage Detected |
| P0108 | BARO Sensor High Voltage Detected |
| P0109 | BARO Sensor Circuit Intermittent |
| P0112 | IAT Circuit Low Input |
| P0113 | IAT Circuit High Input |
| P0117 | ECT Circuit Low Input |
| P0118 | ECT Circuit High Input |
| P0121 | TP Sensor Circuit Performance Problem |
| P0122 | TP Sensor Circuit Low Input |
| P0123 | TP Sensor Circuit High Input |
| P0125 | Insufficient Coolant Temperature To Enter Closed Loop |
| P0126 | Insufficient Coolant Temperature For Stable Operation |
| P0127 | Air Intake Temperature To High |
| P0131 | Upstream HO2S Circuit Out Of Range Low Voltage, Bank No. 1 |
| P0132 | Upstream HO2S Circuit Voltage High, Bank No. 1 |
| P0133 | Upstream HO2S Circuit Slow Response, Bank No. 1 |
| P0135 | Upstream HO2S Heater Circuit Fault, Bank No. 1 |
| P0136 | Downstream HO2S Circuit Fault, Bank No. 1 |
| P0138 | Downstream HO2S Circuit Voltage High, Bank No. 1 |
| P0140 | Downstream HO2S Circuit Faulty, Bank No. 1 |
| P0141 | Downstream HO2S Heater Circuit Fault, Bank No. 1 |
| P0151 | Upstream HO2S Circuit Out Of Range Low Voltage, Bank No. 2 |
| P0152 | Upstream HO2S Circuit Voltage High, Bank No. 2 |
| P0153 | Upstream HO2S Circuit Slow Response, Bank No. 2 |
| P0155 | Upstream HO2S Circuit Fault, Bank No. 2 |
| P0156 | Downstream HO2S Circuit Fault, Bank No. 2 |
| P0158 | Downstream HO2S Circuit Voltage High, Bank No. 2 |
| P0160 | Downstream HO2S Circuit Fault, Bank No. 2 |
| P0161 | Downstream HO2S Heater Circuit Fault, Bank No. 2 |
| P0171 | System Too Lean, Bank No. 1 |
| P0172 | System Too Rich, Bank No. 1 |
| P0174 | System Too Lean, Bank No. 2 |
| P0175 | System Too Rich, Bank No. 2 |
| P0176 | Flexible Fuel Sensor |
| P0180 | Engine Fuel Temperature Sensor Circuit, Electrical Fault |
| P0181 | Engine Fuel Temperature Sensor Circuit, Electrical Fault |
| P0182 | Engine Fuel Temperature Sensor Circuit, Electrical Fault |
| P0183 | Engine Fuel Temperature Sensor Circuit, Electrical Fault |
| P0186 | Engine Fuel Temperature Sensor B Circuit |
| P0187 | Engine Fuel Temperature Sensor Circuit |
| P0188 | Engine Fuel Temperature Sensor Circuit |
| P0190 | Fuel Rail Pressure Sensor Signal Low |
| P0191 | Fuel Rail Pressure Sensor Circuit Performance |
| P0192 | Fuel Rail Pressure Sensor Circuit Low Input |
| P0193 | Fuel Rail Pressure Sensor Circuit High Input |
| P0201–P0212 | Injector Circuits |
| P0217 P0218 | Engine Coolant Overtemp Condition |
| P0222 | Throttle Position Sensor B Circuit Input Low |
| P0223 | Throttle Position Sensor B Circuit Input High |
| P0230 | Fuel Pump Primary Circuit Fault |
| P0231 | Fuel Pump Secondary Circuit Low |
| P0232 | Fuel Pump Secondary Circuit High |
| P0234 | Supercharger Overboost Condition |
| P0243 | Supercharger Bypass Solenoid Malfunction |
| P0298 | Engine Oil Over Temperature Condition |
| P0300 | Random Misfire Detected |
| P0301–P0310 | Misfire Detected Cylinders 1–10 |
| P0320 | Ignition Engine Speed Input Circuit Fault |
| P0325 | A Spark Knock Occurred & Sensor Circuitry Did Not Respond |
| P0326 | Spark Knock Has Not Been Sensed |
| P0330 | A Spark Knock Occurred & Sensor Circuitry Did Not Respond |
| P0331 | Spark Knock Has Not Been Sensed |
| P0340 | CMP Sensor Circuit Fault |
| P0350 | Ignition Coil Primary Circuit Fault |
| P0351-P0360 | Ignition Coil (A through J) Primary/Secondary Circuit Fault |
| P0385 | Misfire Detection Sensor Circuit Failure |
| P0400 | EGR Flow Faulty |
| P0401 | Insufficient EGR Flow |
| P0402 | EGR Flow Excessive At Idle |
| P0411 | Secondary Air Injection System Flow Improper |
| P0412 | Secondary Air Injection System Control Valve Faulty |
| P0413 | Secondary Air Injection Solenoid Did Not Change When Activated |
| P0414 | Secondary Air Injection Solenoid Did Not Change When Activated |
| P0416 | Secondary Air Injection Solenoid Did Not Change When Activated |
| P0417 | Secondary Air Injection Solenoid Did Not Change When Activated |
| P0420 | Catalyst System Efficiency Below Limit, Bank No. 1 |
| P0421 | Catalyst System Efficiency Below Limit, Bank No. 1 |
| P0430 | Catalyst System Efficiency Below Limit, Bank No. 2 |
| P0431 | Catalyst System Efficiency Below Limit, Bank No. 2 |
| P0440 | Evaporative Emission Purge Control Solenoid Damaged |
| P0442 | Leak In Evaporative System Detected |
| P0443 | EVAP Canister Purge Control Solenoid Or Valve Faulty |
| P0446 | Evaporative Emission Control System Vent Control Malfunction |
| P0451 | Fuel Tank Pressure Circuit Noisy |
| P0452 | Fuel Tank Pressure Signal Circuit Input Below Minimum |
| P0453 | Fuel Tank Pressure Signal Circuit Input Above Maximum |
| P0455 | Leak Or Blockage Detected In Evaporative Emission Control System |
| P0456 | Very Small Leak Detected in Evaporative Emission Control System |
| P0457 | Fuel Filler Cap Loose Or Off |
| P0460 | Fuel Level Sensor Input Circuit Malfunction |
| P0500 | Vehicle Speed Sensor Fault |
| P0501 | Vehicle Speed Sensor Range/Performance |
| P0503 | VSS Intermittent |
| P0505 | Idle Air Control Valve System Fault |
| P0552 | Power Steering Pressure Sensor Shorted To Power |
| P0553 | Power Steering Pressure Sensor Shorted To Power |
| P0602 | Control Module Programming Error |
| P0603 | PCM KAM Test Error |
| P0605 | PCM Read Only Memory Error |
| P0703 | Brake On/Off Switch Input Fault |
| P0704 | Clutch Pedal Position Switch Input Circuit Fault |
| P0705 | Transmission Range Sensor Out Of Self Test Range Or Shorted To Ground |
| P0707 | Transmission Range Sensor Circuit Low Input |
| P0708 | Transmission Range Sensor Circuit High Input |
| P0712 | Transmission Fluid Temperature Sensor Circuit Input Low |
| P0713 | Transmission Fluid Temperature Sensor Circuit Input High |
| P0715 | turbine Shaft Speed Sensor Circuit Fault |
| P0717 | Transmission Concern |
| P0720 | Output Shaft Speed Sensor Circuit |
| P0721 | Output Shaft Speed Sensor Circuit |
| P0722 | No Signal From Output Shaft Speed Sensor |
| P0723 | Output Shaft Speed Sensor Circuit Intermittent Failure |
| P0731 | Improper Ratio For First Gear |
| P0732 | Improper Ratio For Second Gear |
| P0733 | Improper Ratio For Third Gear |
| P0734 | Improper Ratio For Fourth Gear |
| P0735 | Transmission Concern |
| P0736 | Improper Ratio For Reverse Gear |
| P0741 | TCC System Improper Mechanical Performance |
| P0743 | Torque Converter Clutch System Electrical Failure |
| P0746 | Electronic Pressure Control Solenoid Performance |
| P0750 | Shift Solenoid No. 1 Circuit |
| P0751 | Shift Solenoid No. 1 Performance |
| P0755 | Shift Solenoid No. 2 Circuit |
| P0756 | Shift Solenoid No. 2 Performance |
| P0760 | Shift Solenoid No. 3 Circuit |
| P0761 | Shift Solenoid No. 3 Performance |
| P0765 | Transmission Concern |
| P0781 | 1 To 2 Shift Error |
| P0782 | 2 To 3 Shift Error |
| P0783 | 3 To 4 Shift Error |
| P0784 | 4 To 5 Shift Error |
| P0796 | Transmission Concern |
| P0797 | Transmission Concern |
| P0799 | Transmission Concern |
| P0812 | Reverse Switch Input Circuit Malfunction |
| P0813 | Transmission Concern |
| P0815 | Transmission Concern |
| P1000 | OBD II Monitor Testing Incomplete |
| P1001 | KOER Test Cannot Be Completed |
| P1039 | Vehicle Speed Signal Missing Or Improper |
| P1051 | Brake Switch Signal Missing Or Improper |
| P1100 | Mass Air Flow Sensor Intermittent |
| P1101 | Mass Air Flow Sensor Out Of Self Test Range |
| P1109 | IAT Sensor 2 Intermittent |
| P1112 | IAT Sensor Intermittent |
| P1114 | IAT Sensor 2 Circuit Low Input |
| P1115 | IAT Sensor 2 Circuit High Input |
| P1116 | ECT Sensor Out Of Self Test Range |
| P1117 | ECT Sensor Intermittent |
| P1120 | TP Sensor Out Of Range Too Low |
| P1121 | TP Sensor Inconsistent With MAF Sensor |
| P1124 | TP Sensor Out Of Self Test Range |
| P1125 | TP Sensor Intermittent |
| P1127 | HO2S Heater Not On During KOER Test |
| P1128 | HO2S Sensor Signals Swapped |
| P1129 | HO2S Sensor Signals Swapped |
| P1130 | Lack Of Upstream HO2S Switch, Adaptive Fuel Limit, Bank No. 1 |
| P1131 | Lack Of Upstream HO2S Switch, Sensor Indicates Lean, Bank No. 1 |
| P1132 | Lack Of Upstream HO2S Switch, Sensor Indicates Rich, Bank No. 1 |
| P1135 | Ignition Switch Signal Missing Or Improper |
| P1137 | Lack Of Downstream HO2S Switch, Sensor Indicates Lean, Bank No. 1 |
| P1138 | Lack Of Downstream HO2S Switch, Sensor Indicates Rich, Bank No. 1 |
| P1150 | Lack Of Upstream HO2S Switch, Adaptive Fuel Limit, Bank No. 2 |
| P1151 | Lack Of Upstream HO2S Switch, Sensor Indicates Lean, Bank No. 2 |
| P1152 | Lack Of Upstream HO2S Switch, Sensor Indicates Rich, Bank No. 2 |
| P1157 | Lack Of Downstream HO2S Switch, Sensor Indicates Lean, Bank No. 2 |
| P1158 | Lack Of Downstream HO2S Switch, Sensor Indicates Rich, Bank No. 2 |
| P1168 | Fuel Rail Pressure Sensor In Range But Low |
| P1169 | Fuel Rail Pressure Sensor In Range But High |
| P1180 | Fuel Delivery System Low |
| P1181 | Fuel Delivery System High |
| P1183 | Engine Oil Temperature Sensor Circuit Malfunction |
| P1184 | Engine Oil Temperature Sensor Out Of Self-Test Range |
| P1220 | Series Throttle Control Fault |
| P1224 | Throttle Position Sensor B Out Of Self Test Range |
| P1229 | Supercharger Intercooler Pump Not Operating |
| P1230 | Open Power To Fuel Pump Circuit |
| P1231 | High Speed Fuel Pump Relay Activated |
| P1232 | Low Speed Fuel Pump Primary Circuit Failure |
| P1233 | Fuel Pump Driver Module Off Line |
| P1234 | Fuel Pump Driver Module Off Line |
| P1235 | Fuel Pump Control Out Of Range |
| P1236 | Fuel Pump Control Out Of Range |
| P1237 | Fuel Pump Secondary Circuit Fault |
| P1238 | Fuel Pump Secondary Circuit Fault |
| P1244 | Alternator Load Input Low |
| P1245 | Alternator Load Input High |
| P1246 | Alternator Load Input Problem |
| P1250 | Lack Of Power To FPRC Solenoid |
| P1260 | THEFT Detected, Engine Disabled |
| P1270 | Engine RPM Or Vehicle Speed Limiter Reached |
| P1285 | Cylinder Head Over Temperature Sensed |
| P1288 | Cylinder Head Temperature Sensor Out Of Self Test Range |
| P1289 | Cylinder Head Temperature Sensor Signal Greater Than Self Test Range |
| P1290 | Cylinder Head Temperature Sensor Signal Less Than Self Test Range |
| P1299 | Cylinder Head Temperature Sensor Detected Engine Overheating Condition |
| P1309 | Misfire Detection Monitor Not Enabled |
| P1351 | Ignition Diagnostic Monitor Circuit Input Fault |
| P1352 | Ignition Coil A Primary Circuit Fault |
| P1353 | Ignition Coil B Primary Circuit Fault |
| P1354 | Ignition Coil C Primary Circuit Fault |
| P1355 | Ignition Coil D Primary Circuit Fault |
| P1356 | Loss of Ignition Diagnostic Module Input To PCM |
| P1358 | Ignition Diagnostic Monitor Signal Out Of Self Test Range |
| P1359 | Spark Output Circuit Fault |
| P1364 | Ignition Coil Primary Circuit Fault |
| P1380 | Variable Cam Timing Solenoid Valve Circuit Short Or Open |
| P1381 | Variable Cam Timing Advance Excessive |
| P1383 | Variable Cam Timing Advance Excessive |
| P1390 | Octane Adjust Out Of Self Test Range |
| P1400 | Differential Pressure Feedback Electronic Sensor Circuit Low Voltage |
| P1401 | Differential Pressure Feedback Electronic Sensor Circuit High Voltage |
| P1403 | Differential Pressure Feedback Electronic Sensor Hoses Reversed |
| P1405 | Differential Pressure Feedback Electronic Sensor Circuit Upstream Hose |
| P1406 | Differential Pressure Feedback Electronic Sensor Circuit Downstream Hose |
| P1407 | EGR No Flow Detected |
| P1408 | EGR Flow Out Of Self Test Range |
| P1409 | EGR Vacuum Regulator Circuit Malfunction |
| P1410 | EGR Barometric Pressure Sensor VREF Voltage |
| P1411 | Secondary Air Is Not Being Diverted |
| P1413 | Secondary Air Injection System Monitor Circuit Low Voltage |
| P1414 | Secondary Air Injection System Monitor Circuit High Voltage |
| P1442 | Secondary Air Injection System Monitor Circuit High Voltage |
| P1443 | Evaporative Emission Control System, Vacuum System, Purge Control Solenoid Or Purge Control Valve Fault |
| P1444 | Purge Flow Sensor Circuit Input Low |
| P1445 | Purge Flow Sensor Circuit Input High |
| P1450 | Inability Of Evaporative Emission Control System To Bleed Fuel Tank |
| P1451 | Lack Of Response From Passive Anti-Theft System Module, Engine Disabled |
| P1452 | Inability Of Evaporative Emission Control System To Bleed Fuel Tank |
| P1455 | Substantial Leak Or Blockage In Evaporative Emission Control System |
| P1460 | Wide Open Throttle Air Conditioning Cutoff Circuit Malfunction |
| P1461 | Air Conditioning Pressure Sensor Circuit Low Input |
| P1462 | Air Conditioning Pressure Sensor Circuit high Input |
| P1463 | Air Conditioning Pressure Sensor Insufficient Pressure Change |
| P1464 | A/C Demand Out Of Self Test Range |
| P1469 | Low A/C Cycling Period |
| P1473 | Fan Secondary High With Fans Off |
| P1474 | Low Fan Control Primary Circuit |
|  | Hydraulic Cooling Fan Primary Circuit Failure |
| P1477 | Medium Fan Control Primary Function Failure |
| P1479 | High Fan Control Primary Circuit |
| P1480 | Fan Secondary Low With Low Fans On |
| P1481 | Fan Secondary Low With High Fans On |
| P1483 | Power To Cooling Fan Exceeded Normal Draw |
| P1484 | Variable Load Control Module Pin 1 Open |
| P1500 | Vehicle Speed Sensor Intermittent |
| P1501 | VSS Out Of Self-Test Range |
| P1502 | Invalid Or Missing Vehicle Speed Message Or Brake Data (1998) |
|  | VSS Intermittent (1999) |
| P1504 | IAC Circuit Malfunction |
| P1505 | IAC System At Adaptive Clip |
| P1506 | IAC Over Speed Error |
| P1507 | IAC Under Speed Error |
| P1512 | Intake Manifold Runner Control Stuck Closed |
| P1513 | Intake Manifold Runner Control Stuck Closed |
| P1516 | Intake Manifold Runner Control Input Error |
| P1517 | Intake Manifold Runner Control Input Error |
| P1518 | Intake Manifold Runner Control Fault, Stuck Open |
| P1519 | Intake Manifold Runner Control Stuck Open |
| P1520 | Intake Manifold Runner Control Circuit Fault |
| P1530 | Open Or Short To A/C Compressor Clutch Circuit |
| P1537 | Intake Manifold Runner Control Stuck Open |
| P1538 | Intake Manifold Runner Control Stuck Open |
| P1539 | Power To A/C Compressor Clutch Circuit Exceeded Normal Current Draw |
| P1549 | IMT Valve Vacuum Actuator Connection (1996-98) |
|  | Intake Manifold Communications Control Circuit Malfunction (1999) |
| P1550 | Power Steering Pressure Sensor Out Of Self Test Range |
| P1565 | Speed Control System Concern |
| P1566 | Speed Control System Concern |
| P1567 | Speed Control System Concern |
| P1568 | Speed Control System Concern |
| P1572 | Brake Pedal Switch Circuit |
| P1605 | PCM Keep Alive Memory Test Error |
| P1625 | Voltage To Vehicle Load Control Module Fan Circuit Not Detected |
| P1626 | Voltage To Vehicle Load Control Module Circuit Not Detected |
| P1633 | Keep Alive Memory Power Voltage Low |
| P1635 | Tire/Axle Ratio Out Of Acceptable Range |
| P1639 | Vehicle ID Block Not Programmed Or Is Corrupt |
| P1640 | Powertrain DTCs Available In Another Module |
| P1641 | Fuel Pump Driver Module Concern |
| P1650 | Power Steering Pressure Switch Out Of Self Test Range |
| P1651 | Power Steering Pressure Switch Input Fault |
| P1700 | Transmission Concern |
| P1701 | Reverse Engagement Error |
| P1702 | Transmission Concern |
| P1703 | Brake On/Off Switch Out Of Self Test Range |
| P1704 | Transmission Concern |
| P1705 | Manual Lever Position Sensor Out Of Self Test Range |
| P1709 | Park Or Neutral Position Switch Out Of Self Test Range |
| P1710 | Transmission Concern |
| P1711 | Transmission Fluid Temperature Sensor Out Of Self Test Range |
| P1713–P1728 | Transmission Concern |
| P1729 | 4x4 Low Switch Error |
| P1740 | Transmission Concern |
| P1741 | Torque Converter Clutch Control Error |
| P1742 | Torque Converter Clutch Solenoid Faulty |
| P1743 | Torque Converter Clutch Solenoid Faulty |
| P1744 | Torque Converter Clutch System Stuck In Off Position |
| P1745 | Transmission Concern |
| P1746 | Electronic Pressure Control Solenoid, Open Circuit |
| P1747 | Electronic Pressure Control Solenoid, Short Circuit |
| P1749 | Electronic Pressure Control Solenoid Failed Low |
| P1751 | Shift Solenoid No. 1 Performance |
| P1754 | Coast Clutch Solenoid Circuit Malfunction |
| P1756 | Shift Solenoid No. 2 Performance |
| P1760 | Transmission Concern |
| P1761 | Shift Solenoid No. 3 Performance |
| P1762 | Transmission Concern |
| P1767 | Transmission Concern |
| P1780 | Transmission Control Switch Circuit Out Of Self Test Range |
| P1781 | 4x4 Low Switch Out Of Self Test Range |
| P1783 | Transmission Over Temperature Condition |
| P1784–P1789 | Transmission Concern |
| P1900 | Output Shaft Speed Sensor Circuit Intermittent Failure |
| P1901 | Transmission Concern |
| U1020 | Module Communications Network Concern |
|  | SCP Invalid Or Missing Data For A/C Compressor Clutch (1998-99) |
| U1021, U1073 & U1131 | Failure In Communication Between PCM & Variable Load Control Module (1994-97) |
| U1039 | SCP Invalid Or Missing Data For Vehicle Speed (1998-2001) |
| U1039, U1051, U1135 & U1451 | Module Communications Network Concern (1994-97) |
| U1041 | SCP Invalid Or Missing Data For Vehicle Speed (1998-2001) |
| U1051 | SCP Invalid Or Missing Data For Brakes (1998-2001) |
| U1131 | SCP Invalid Or Missing Data For Fuel System (1998-2001) |
| U1147 | SCP Invalid Or Missing Data For Vehicle Security (1998-99) |
| U1256 | Variable Load Control Module Concern |
| U1262 | SCP Communications Bus Fault |
| U1451 | SCP Invalid Or Missing Data For Anti-Theft Module (1998-2001) |
| U1850 | Invalid Or Missing Fuel system Data |
| U2243 | SCP Invalid Or Missing Data For Exterior Environment (1998-2001) |
| UXXXX | PCM Concern (1994-97) |
|  | U DTCs Received During Self-Test Of Another Module (1998-2001) |

## MCU

| 4 CYL |  |
| --- | --- |
| 11 | Continue With Testing |
| 51 | Low Temperature Switch |
| 52 & 53 | Idle Tracking Switch |
| 53 Or 63 | Wide Open Throttle Vacuum Switch |
| 65 | Vehicle Modified For Use At High Altitude |
| V6 OR V8 |  |
| 12 | Improper Idle Speed |
| 25 | Knock Detection System Inoperative |
| 41 | EGO Always Lean |
| 42 | EGO Always Rich |
| 44 | Thermactor Check |
| 45 | Thermactor Check |
| 46 | Thermactor Air Bypass Check, Always Not Bypassing |
| 51 Or 55 | Vacuum Switch Open |
| 53 Or 54 | Open Temperature Switch |
| 65 | Vacuum Switch Closed |

## DI 7.3L Turbo Diesel

NOTE: Speed control DTC’s will be set during KOER Switch Test if the vehicle is not equipped w/ speed control. This is normal condition. On these vehicles ignore the following DTC’s; P0565, P0566, P0567, P0569 & P0571.

| P0107 | BARO Circuit Out Of Range Low |
| --- | --- |
| P0108 | BARO Circuit Out Of Range High |
| P0112 | IAT Circuit Out Of Range Low |
| P0113 | IAT Circuit Out Of Range High |
| P0122{1} | AP Sensor Signal ORL |
| P0123{1} | AP Sensor Signal ORH |
| P0195{5} | EOT Above/Below KOER Limits |
| P0196{5} | EOT Sensor Circuit Performance |
| P0197{1} | EOT Circuit Out Of Range Low |
| P0198{1} | EOT Circuit Out Of Range High |
| P0219{5} | Excessive Engine RPM |
| P0220{1} | Idle Validation Switch Fault |
| P0221{1} | Idle Validation Switch Fault |
| P0230{5} | Fuel Pump Monitor Circuit |
| P0231{5} | Fuel Pump Monitor Circuit |
| P0232{5} | Fuel Pump Monitor Circuit |
| P0235{1}{5} | MAP Inactive Signal |
| P0236{1}{2} | Turbo Boost Sensor A Circuit Performance |
| P0236{1}{3} | Turbo Boost Sensor A Circuit Performance |
| P0237{1}{2} | Turbo Boost Sensor A Circuit Performance |
| P0237{1}{3} | Turbo Boost Sensor A Circuit Low Input |
| P0238{1}{3} | Turbo Boost Sensor A Circuit High Input |
| P0261{1}{2} | Low Side Short To Ground Injector Circuit No. 1 |
| P0261{1}{3} | Injector Circuit Low, No. 1 Cylinder |
| P0262{2} | Low Side Short To Battery Injector Circuit No. 1 |
| P0262{3} | Injector Circuit High, No. 1 Cylinder |
| P0263 | Cylinder Fault No. 1, CCT |
| P0264{1}{2} | Low Side Short To Ground Injector Circuit No. 2 |
| P0264{1}{3} | Injector Circuit Low, No. 2 Cylinder |
| P0265{2} | Low Side Short To Battery Injector Circuit No. 2 |
| P0265{3} | Injector Circuit High, No. 2 Cylinder |
| P0266 | Cylinder Fault No. 2, CCT |
| P0267{1}{2} | Low Side Short To Ground Injector Circuit No. 3 |
| P0267{1}{3} | Injector Circuit Low, No. 3 Cylinder |
| P0268{2} | Low Side Short To Battery Injector Circuit No. 3 |
| P0268{3} | Injector Circuit High, No. 3 Cylinder |
| P0269 | Cylinder Fault No. 3, CCT |
| P0270{1}{2} | Low Side Short To Ground Injector Circuit No. 4 |
| P0270{1}{3} | Injector Circuit Low, No. 4 Cylinder |
| P0271{2} | Low Side Short To Battery Injector Circuit No. 4 |
| P0271{3} | Injector Circuit High, No. 4 Cylinder |
| P0272 | Cylinder Fault No. 4, CCT |
| P0273{1}{2} | Low Side Short To Ground Injector Circuit No. 5 |
| P0273{1}{3} | Injector Circuit Low, No. 5 Cylinder |
| P0274{2} | Low Side Short To Battery Injector Circuit No. 5 |
| P0274{3} | Injector Circuit High, No. 5 Cylinder |
| P0275 | Cylinder Fault No. 5, CCT |
| P0276{2} | Low Side Short To Ground Injector Circuit No. 6 |
| P0276{1}{3} | Injector Circuit Low, No. 6 Cylinder |
| P0277{2} | Low Side Short To Battery Injector Circuit No. 6 |
| P0277{3} | Injector Circuit High, No. 6 Cylinder |
| P0278 | Cylinder Fault No. 6, CCT |
| P0279{1}{2} | Low Side Short To Ground Injector Circuit No. 7 |
| P0279{1}{3} | Injector Circuit Low, No. 7 Cylinder |
| P0280{2} | Low Side Short To Battery Injector Circuit No. 7 |
| P0280{3} | Injector Circuit High, No. 7 Cylinder |
| P0281 | Cylinder Fault No. 7, CCT |
| P0282{1}{2} | Low Side Short To Ground Injector Circuit No. 8 |
| P0282{1}{3} | Injector Circuit Low, No. 8 Cylinder |
| P0283{2} | Low Side Short To Battery Injector Circuit No. 8 |
| P0283{3} | Injector Circuit High, No. 8 Cylinder |
| P0284 | Cylinder Fault No. 8, CCT |
| P0301{5} | Injector Driver Circuit |
| P0302{5} | Injector Driver Circuit |
| P0303{5} | Injector Driver Circuit |
| P0304{5} | Injector Driver Circuit |
| P0305{5} | Injector Driver Circuit |
| P0306{5} | Injector Driver Circuit |
| P0307{5} | Injector Driver Circuit |
| P0308{5} | Injector Driver Circuit |
| P0340 | Inactive CMP Sensor Circuit |
| P0341{2} | CMP Noise Rejection |
| P0341{1}{3} | CMP Sensor Circuit Performance |
| P0344{2} | CMP/SYNC Counts |
| P0344{1}{3} | CMP Sensor Circuit Intermittent |
| P0380{2} | GP Relay Output Circuit Fault |
| P0380{3} | GP Circuit Malfunction |
| P0381{2} | GP Lamp Output Circuit Fault |
| P0381{3} | GP Indicator Circuit Malfunction |
| P0460{3} | Fuel Tank Level Indicator Circuit Malfunction |
| P0470 | EBP Sensor Circuit Malfunction |
| P0471 | EBP Sensor Circuit Performance |
| P0472 | EBP Circuit Out Of Range Low |
| P0473 | EBP Circuit Out Of Range High |
| P0475{2} | EPR Circuit Fault |
| P0475{3} | EP Control Valve Malfunction |
| P0476{2} | EBP Pressure Above/Below Desired |
| P0476{3} | EP Control Valve Performance |
| P0478 | Excessive Backpressure |
| P0500 | Vehicle Speed Sensor Fault |
| P0541 | Manifold Intake Air Heater |
| P0542 | Manifold Intake Air Heater |
| P0560{2} | B+, Volt Too Low For CCT Test |
| P0560{3} | System Voltage Malfunction |
| P0562{2} | Battery Voltage Below Normal |
| P0562{3} | System Voltage Malfunction |
| P0563{2} | Battery Voltage Above Normal |
| P0563{3} | System Voltage Malfunction |
| P0565{2} | Speed Control ON Not Pressed |
| P0565{3} | Cruise‘‘ON’’ Signal Malfunction |
| P0566{2} | Speed Control OFF Not Pressed |
| P0566{3} | Cruise‘‘OFF’’ Signal Malfunction |
| P0567{2} | Speed Control RESUME Not Pressed |
| P0567{3} | Cruise‘‘ RESUME’’ Signal Malfunction |
| P0568{2} | Speed Control SET Not Pressed |
| P0568{3} | Cruise‘‘SET’’ Signal Malfunction |
| P0569 | Speed Control COAST Not Pressed |
| P0569 | Cruise‘‘COAST’’ Signal Malfunction |
| P0571{2} | Brake Pedal Applied Switch Fault |
| P0571{3} | Brake Switch Circuit A Malfunction |
| P0603 | KAM Memory Loss |
| P0605 | PCM ROM Test Fault |
| P0606 | PCM Inactive Background Fault |
| P0640 | Manifold Intake Air Heater |
| P0703{2} | Brake On/Off Switch Circuit Fault |
| P0703{3} | Brake Switch Circuit B Malfunction |
| P0704 | CPP Switch Input Circuit Fault |
| P0705{1}{2} | TR Sensor Circuit Fault |
| P0705{3}{4} | TR Sensor Circuit Fault |
| P0707{1}{2} | TR Sensor Circuit Low Input |
| P0707{3}{4} | TR Sensor Circuit Low Input |
| P0708{1}{2} | TR Sensor Circuit High Input |
| P0708{3}{4} | TR Sensor Circuit High Input |
| P0712 | Transmission Fluid Temperature Sensor Circuit Low Input |
| P0713 | Transmission Fluid Temperature Sensor Circuit High Input |
| P0715{5} | Transmission Concern |
| P0717{5} | Transmission Concern |
| P0718{5} | Transmission Concern |
| P0720{5} | Transmission Concern |
| P0721{5} | Transmission Concern |
| P0722{5} | Transmission Concern |
| P0732{5} | Transmission Concern |
| P0733{5} | Transmission Concern |
| P0741{2} | TCC Solenoid Fault |
| P0741{3} | TCC Solenoid Circuit Performance |
| P0743 | TCC System Electrical Failure |
| P0750 | Shift Solenoid No. 1 Fault |
| P0751 | Shift Solenoid A Performance |
| P0755 | Shift Solenoid No. 2 Fault |
| P0756 | Shift Solenoid B Performance |
| P0760 | Shift Solenoid C Malfunction |
| P0781{1}{2} | 1–2 Shift Error |
| P0781{3}{4} | 1–2 Shift Malfunction |
| P0782{1}{2} | 2–3 Shift Error |
| P0782{3}{4} | 2–3 Shift Malfunction |
| P0783{1}{2} | 3–4 Shift Error |
| P0783{3}{4} | 3–4 Shift Malfunction |
| P1000 | OBD II Monitor Checks Incomplete, More Driving Required |
| P1105{5} | Charging System Concern |
| P1106{5} | Charging System Concern |
| P1107{5} | Charging System Concern |
| P1108{5} | Charging System Concern |
| P1111{2} | System Pass |
| P1111 | System Pass, 49 State Except 1998–98.5 Econoline |
| P1118{3} | MAT Sensor Fault |
| P1119{3} | MAT Sensor Fault |
| P1139{3} | WIF Sensor Fault |
| P1140{3} | WIF Sensor Fault |
| P1184 | EOT Sensor Circuit Performance |
| P1209 | Injection Control System Pressure Peak Fault |
| P1210 | ICP Sensor Fault |
| P1211{2} | ICP Pressure Is Above/Below Desired, Faulty IPR Valve |
| P1211{1}{3} | ICP Pressure Uncontrollable, Pressure Above/Below Desired |
| P1212{2} | ICP Higher That Expected KOEO |
| P1212{1}{3} | ICP Voltage Not At Expected Level |
| P1218 | CID Stuck High |
| P1219 | CID Stuck Low |
| P1247 | Turbo Boost Pressure Low |
| P1248 | Turbo Boost Pressure Not Detected |
| P1249{3} | Waste Gate Control Solenoid |
| P1250{5} | Waste Gate Control Solenoid |
| P1261–1268 | High To Low Side Short, Cylinders No.1–8 |
| P1271–1278 | High To Low Side Open, Cylinders No.1–8 |
| P1280{2} | ICP Circuit Out Of Range, Low |
| P1280{1}{3} | ICP Circuit Out Of Range, Low |
| P1281{2} | ICP Circuit Out Of Range, High |
| P1281{1}{3} | ICP Circuit Out Of Range, High |
| P1282 | Excessive ICP Pressure |
| P1283{2} | IPR Circuit Fault |
| P1283{1}{3} | IPR Circuit Failure |
| P1284 | Aborted KOER, ICP Fault |
| P1291 | High Side No. 1 (RH) Short To Ground Or B+ |
| P1292 | High Side No. 2 (LH) Short To Ground Or B+ |
| P1293 | High Side Open Bank No. 1 (RH) |
| P1294 | High Side Open No. 2 (LH) |
| P1295{1} | Multiple Faults On Bank No. 1 (RH) |
| P1296{1} | Multiple Faults On Bank No. 2 (LH) |
| P1297 | High Sides Shorted Together |
| P1298 | IDM Fault |
| P1316 | Injector Circuit/IDM Codes Detected |
| P1391 | GP Circuit Low Input Bank No. 1 (RH) |
| P1392 | GP Circuit High Input Bank No. 1 (RH) |
| P1393 | GP Circuit Low Input Bank No. 2 (LH) |
| P1394 | GP Circuit High Input Bank No. 2 (LH) |
| P1395 | GP Monitor Fault Bank No. 1 |
| P1396 | GP Monitor Fault No. 2 |
| P1397 | System Voltage Out Of Self Test Range |
| P1464 | A/C On During KOER Or CCT Test |
| P1501 | Vehicle Moved During KOER |
| P1502{3} | Invalid Self Test-APCM Functioning |
| P1531 | Invalid CCT Test, Accelerator Pedal Movement |
| P1536 | Parking Brake Applied Fault |
| P1660 | Output Circuit Check Signal High |
| P1661 | Output Circuit Check Signal Low |
| P1662 | IDM EN Circuit Fault |
| P1663 | FDCS Circuit Fault |
| P1667 | CID Circuit Fault |
| P1668 | PCM/IDM Diagnostic Communication Error |
| P1670{3} | IDM Electronic Feedback |
| P1690{5} | Waste Gate Control Solenoid |
| P1702{5} | Transmission Concern |
| P1704 | Digital TRS Failed To Transition State |
| P1705 | TR Sensor Out Of Self Test Range |
| P1706{1}{2} | High Vehicle Speed In Park |
| P1706{3}{4} | High Vehicle Speed In Park |
| P1711 | TFT Sensor Out Of Self Test Range |
| P1713{5} | Transmission Concern |
| P1714 | Shift Solenoid A Inductive Signature Malfunction |
| P1715 | Shift Solenoid B Inductive Signature Malfunction |
| P1718{5} | Transmission Concern |
| P1727 | Coast Clutch Solenoid Inductive Signature Malfunction |
| P1728{1}{2} | Transmission Slip Error |
| P1728{3}{4} | Transmission Slip Error, Converter Clutch Failed |
| P1729 | 4x4 Low Switch Error |
| P1740 | TCC Inductive Signature Malfunction |
| P1742 | TCC Failed On |
| P1744 | TCC System Performance |
| P1746 | EPC Solenoid Open Circuit |
| P1747 | EPC Solenoid Short Circuit |
| P1748{1}{2} | EPC Fault |
| P1748{3}{4} | EPC Malfunction |
| P1751 | Shift Solenoid A Performance |
| P1754 | Coast Clutch Solenoid Circuit Fault |
| P1756 | Shift Solenoid B Performance |
| P1760{5} | Transmission Concern |
| P1779 | TCIL Circuit Out Of Self Test Range |
| P1780 | TCS Circuit Out Of Self Test Range |
| P1781 | 4x4 Low Circuit Out Of Self Test Range |
| P1783{1}{2} | Transmission Over Temperature Condition |
| P1783{3}{4} | Transmission Over Temperature Condition |
| P1902{3} | Kickdown Solenoid Relay |
| P1903{3} | Kickdown Solenoid Relay |
| P1904{3} | Kickdown Solenoid Relay |

{1}—MIL lit when fault is present.
{2}—1997 models.
{3}—1998–2001 models.
{4}—Transmission Control Indicator Lamp (TCIL) lit when fault is present.
{5}—2001 & Earlier models.