| Code retrieval info |  |
| --- | --- |
| 90 | Geo 1.6L |
| 91 | Geo 1.6L |
| 95 | Geo 1.6L |

## GM Import Trouble Code Info

| #### 87-88 Sprint Turbo,89 & later Metro, Tracker, Storm. |  |
| --- | --- |
| 12 | Diagnostic function working |
| 13 | Oxygen sensor or circuit |
| 14 | Coolant temperature sensor or circuit (open) |
| 15 | Coolant temperature sensor or circuit (shorted) |
| 21 | Throttle position sensor or circuit (open) |
| 22 | Throttle position sensor or circuit (shorted) |
| 23 | Intake air temperature sensor or circuit (open) |
| 24 | Vehicle speed sensor (VSS) or circuit |
| 25 | Intake air temperature sensor or circuit (shorted) |
| 31 | High turbo pressure (87 - 88) |
| 31 | Barometric pressure sensor or circuit (89 -95) |
| 32 | Barometric pressure sensor or circuit (89 - 95) |
| 32 | EGR system (91 - 93) |
| 33 | Airflow meter (turbo) |
| 33 | Manifold Absolute Pressure (MAP) sensor (90 - 91) |
| 41 | Ignition signal problem |
| 42 | Crank angle sensor (except Storm) |
| 42 | Camshaft position sensor circuit (94-95) |
| 42 | Electronic Spark Timing (EST) (Storm) |
| 44 | ECM idle switch circuit open |
| 44 | Oxygen sensor or circuit - lean exhaust |
| 45 | Oxygen sensor or circuit - rich exhaust |
| 46 | Idle speed control motor |
| 51 | EGR system (except Storm) |
| 51 | ECM (Storm) |
| 53 | ECM ground circuit |
| ON Steady | ECM faulty |

| #### 89 - 92 GEO Metro TBI |  |
| --- | --- |
| 13 | Oxygen Sensor circuit. |
| 14 | Coolant Temperature Sensor circuit. High temperature indicated. |
| 15 | Coolant Temperature Sensor circuit. Low temperature indicated. |
| 21 | Throttle Switch circuit (Manual Transmission). |
| 21 | Throttle Position Sensor Signal Voltage High (Automatic Transmission). |
| 22 | Throttle Position Sensor Signal Voltage Low. |
| 23 | Intake Air temperature (MAT) Sensor. Low temperature indicated. |
| 24 | Vehicle Speed Sensor circuit. |
| 25 | Intake Air temperature (MAT) Sensor. High temperature indicated. |
| 31 | MAP Sensor circuit - Signal Voltage Low Vacuum. |
| 32 | MAP Sensor circuit - Signal Voltage High Vacuum. |
| 33 | MAP Sensor circuit. |
| 41 | Ignition Signal circuit. |
| 42 | Crank Angle Sensor. |
| 46 | Idle Speed Control Motor circuit. |
| 51 | EGR circuit. |
| 53 | Ground circuit. |

| 93 - 95 |  |
| --- | --- |
| 13 | Oxygen Sensor circuit. Signal voltage does not change. |
| 14 | Engine Coolant Temperature Sensor circuit High Voltage Input. Low temperature indicated. |
| 15 | Engine Coolant Temperature Sensor circuit Low Voltage Input. High temperature indicated. |
| 21 | Throttle Position Sensor circuit High Voltage Input. |
| 22 | Throttle Position Sensor circuit Low Voltage Input. |
| 23 | Intake Air temperature Sensor circuit - High Voltage Input. Low temperature indicated. |
| 24 | Vehicle Speed Sensor circuit. |
| 25 | Intake Air temperature Sensor circuit - Low Voltage Input. High temperature indicated. |
| 31 | MAP Sensor circuit Low Voltage Input - Low manifold pressure. |
| 32 | MAP Sensor circuit High Voltage Input - High manifold pressure. |
| 41 | Ignition Signal circuit. |
| 42 | Crankshaft Position Sensor circuit - No Signal for 2 seconds. |
| 46 | Idle Speed Control Motor circuit. |
| 51 | EGR circuit. |

| #### Sprint(non-turbo) |  |
| --- | --- |
| 12 | Diagnostic function working |
| 13 | Oxygen sensor circuit |
| 14 | Coolant temperature sensor circuit |
| 21 | Throttle position switches or circuit |
| 23 | Intake air temperature circuit |
| 32 | Barometric Pressure circuit |
| 51 | Possible faulty ECM |
| 52 | Fuel cut solenoid or circuit |
| 53 | Secondary air circuit |
| 54 | Mixture control solenoid or circuit |
| 55 | Bowl vent solenoid or circuit |

| #### GEO, Sprint, Nova & Spectrum |  |
| --- | --- |
| 12 | No distributor reference pulse to ECM |
| 13 | Oxygen sensor circuit |
| 14 | Coolant sensor - high or low temp |
| 21 | Throttle Position Sensor (TPS) voltage high or low |
| 23 | Intake Air Temperature IAT sensor circuit voltage high or low |
| 24 | Vehicle Speed Sensor (VSS) or circuit |
| 32 | EGR (Exhaust gas recirculation) EVRV shorted to ground on start-up |
| 33 | Manifold Absolute Pressure (MAP) sensor voltage high or low |
| 42 | Electronic Spark Timing (EST) circuit |
| 44 | Oxygen sensor or circuit indicates lean exhaust (heated type) |
| 45 | Oxygen sensor or circuit indicates rich exhaust |
| 51 | ECM or EEPROM |

| #### Spectrum |  |
| --- | --- |
| 12 | No distributor reference pulses to ECM |
| 13 | Oxygen sensor circuit |
| 14 | Coolant sensor circuit shorted. |
| 15 | Coolant sensor circuit open. |
| 16 | Coolant sensor circuit open. |
| 21 | Idle switch out of adjustment (or open circuit) non-Turbo t |
| 21 | Throttle Position Sensor (TPS) voltage high-- Turbo |
| 22 | Fuel cut off relay or circuit open--non-Turbo |
| 22 | Throttle Position Sensor (TPS) voltage low--Turbo |
| 23 | Open or grounded Mixture Control(M/C) solenoid or circuit-- non-Turbo |
| 23 | Manifold Air Temperature MAT sensor or circuit--Turbo |
| 24 | Vehicle Speed Sensor (VSS) or circuit--Turbo |
| 25 | Open or grounded vacuum switching valve or circuit--non-Turbo |
| 25 | Air Switching Valve (ASV) or circuit--Turbo |
| 31 | Wastegate control--Turbo |
| 33 | Manifold Absolute Pressure (MAP) sensor voltage high--Turbo |
| 34 | Manifold Absolute Pressure (MAP) sensor voltage low--Turbo |
| 42 | Electronic Spark Timing (EST) circuit EST circuit is grounded or open. Turbo |
| 42 | Fuel cut off relay or circuit non-Turbo type |
| 43 | Detonation (knock) sensor or circuit--Turbo type |
| 44 | Oxygen sensor or circuit indicates lean exhaust non-Turbo |
| 45 | Oxygen sensor or circuit indicates rich exhaust |
| 51 | ECM or PROM |
| 53 | Shorted switching unit or faulty ECM non-Turbo |
| 54 | Mixture Control (M/C) solenoid or circuit, or faulty ECM non-Turbo |
| 55 | Faulty ECM non-Turbo |

### 90 Geo 1.6L

### Accessing Trouble Codes

To get a trouble code out of the ECM, it is necessary to plug a spare fuse into the position for that purpose located in the fuse box.

### Clearing Trouble Codes

- Clearing Stored Codes. (Long Term Memory) - Turn "OFF" the ignition. - Remove the tail/dome fuse for 20 seconds. - Re-install fuse. - Resetting the "CHECK ENGINE" light. ONLY after the maintenance service has been performed (Federal Only). - Locate the cancel switch behind the access panel, below the steering column. - To reset the switch, move the switch to "OFF." NOTE: Federal equipped vehicles have a cancel switch which needs to be reset after the 50,000, 80,000, and 100,000 mile maintenance service is completed. At mileage intervals of 50,000, 80,000 and 100,000 miles, the Check Engine lamp will be illuminated indicating required service and maintenance is to be performed. After performing the required service, the lamp may be reset by moving the slider switch located behind the access panel, directly below the steering column.

### Trouble Codes

| ### 89 - 92 GEO |  |
| --- | --- |
| 13 | Oxygen Sensor circuit. |
| 14 | Coolant Temperature Sensor circuit. High temperature indicated. |
| 15 | Coolant Temperature Sensor circuit. Low temperature indicated. |
| 21 | Throttle Position Sensor Signal Voltage High (Automatic Transmission). |
| 22 | Throttle Position Sensor Signal Voltage Low. |
| 23 | Intake Air temperature (MAT) Sensor. Low temperature indicated. |
| 24 | Vehicle Speed Sensor circuit. |
| 25 | Intake Air temperature (MAT) Sensor. High temperature indicated. |
| 31 | MAP Sensor circuit - Signal Voltage Low Vacuum. |
| 32 | MAP Sensor circuit - Signal Voltage High Vacuum. |
| 33 | MAP Sensor circuit. |
| 41 | Ignition Signal circuit. |
| 42 | Crank Angle Sensor. |
| 44 | Idle switch open or improper adjustment. |
| 45 | Idle Switch Circuit Grounded or Improper Adjustment. |
| 46 | Idle Speed Control Motor circuit. |
| 51 | EGR circuit. |
| 53 | Ground circuit. |

### 91 Geo 1.6L

### Accessing Trouble Codes

To get a trouble code out of the ECM, it is necessary to jumper the BLACK wire and the BLUE/YELLOW wire at the ECM check connector, located under the hood by the battery.

### Clearing Trouble Codes

- Clearing Stored Codes. (Long Term Memory) - Turn "OFF" the ignition. - Remove the tail/dome fuse for 20 seconds. - Re-install fuse. - Resetting the "CHECK ENGINE" light. ONLY after the maintenance service has been performed (Federal Only). - Locate the cancel switch behind the access panel, below the steering column. - To reset the switch, move the switch to "OFF." NOTE: Federal equipped vehicles have a cancel switch which needs to be reset after the 50,000, 80,000, and 100,000 mile maintenance service is completed.

### Trouble Codes

| ### 89 - 92 GEO |  |
| --- | --- |
| 13 | Oxygen Sensor circuit. |
| 14 | Coolant Temperature Sensor circuit. High temperature indicated. |
| 15 | Coolant Temperature Sensor circuit. Low temperature indicated. |
| 21 | Throttle Position Sensor Signal Voltage High (Automatic Transmission). |
| 22 | Throttle Position Sensor Signal Voltage Low. |
| 23 | Intake Air temperature (MAT) Sensor. Low temperature indicated. |
| 24 | Vehicle Speed Sensor circuit. |
| 25 | Intake Air temperature (MAT) Sensor. High temperature indicated. |
| 31 | MAP Sensor circuit - Signal Voltage Low Vacuum. |
| 32 | MAP Sensor circuit - Signal Voltage High Vacuum. |
| 33 | MAP Sensor circuit. |
| 41 | Ignition Signal circuit. |
| 42 | Crank Angle Sensor. |
| 44 | Idle switch open or improper adjustment. |
| 45 | Idle Switch Circuit Grounded or Improper Adjustment. |
| 46 | Idle Speed Control Motor circuit. |
| 51 | EGR circuit. |
| 53 | Ground circuit. |

95 Geo Tracker

Federal

PROCEDURE

- Locate the duty check data link connector in the RH rear engine compartment near the battery.

- Ground terminal B of the connector (blue/yellow wire) to terminal C of the connector (black ground wire).

- Turn the ignition key to the ON position, and observe the Malfunction Indicator Lamp (MIL) .

- The MIL will begin to flash a code 12 continuously, if no other DTC's are stored. If other codes are stored they will be flashed in sequence from the smallest to the largest beginning with Code 12.

California

PROCEDURE

- Locate the duty check data link connector in the RH rear engine compartment near the battery.

- Ground terminal 2 of the connector (blue/yellow wire) to terminal 4 of the connector (black ground wire).

- Turn the ignition key to the ON position, and observe the Malfunction Indicator Lamp (MIL) .

- The MIL will begin to flash a code 12 continuously, if no other DTC's are stored. If other codes are stored they will be flashed in sequence from the smallest to the largest beginning with Code 12.

| 95 Geo Tracker 1.6L |  |
| --- | --- |
| DTC 13 | Heated Oxygen Sensor (H02S) Circuit |
| DTC 14 | Engine Coolant Temperature (ECT) Circuit (High Voltage) |
| DTC 15 | Engine Coolant Temperature (ECT) Circuit (Low Voltage) |
| DTC 21 | Throttle Position Sensor (TPS) Circuit (High Voltage) |
| DTC 22 | Throttle Position Sensor (TPS) Circuit (Low Voltage) |
| DTC 23 | Intake Air Temperature (IAT) Circuit (High Voltage) |
| DTC 24 | Vehicle Speed Sensor (VSS) Circuit |
| DTC 25 | Intake Air Temperature (IAT) Circuit (Low Voltage) |
| DTC 31 | Manifold Absolute Pressure (MAP) Sensor Circuit (High Voltage) |
| DTC 32 | Manifold Absolute Pressure (MAP) Sensor Circuit (Low Voltage) |
| DTC 41 | Ignition Signal Circuit (No Signal) |
| DTC 42 | Camshaft Position (CMP) Sensor (No Signal for 2 seconds) |
| DTC 44 | Idle Switch Circuit (Circuit Open or Misadjusted) |
| DTC 45 | Idle Switch Circuit (Circuit Grounded or Misadjusted) |
| DTC 51 | Exhaust Gas Recirculation (EGR) Circuit |
| DTC 53 | Ground Circuit (Circuit Open) |