# Honda Trouble Codes

85 Accord, 85-87 Civic

ECU Location

The ECU is located under the passenger's seat. Codes are displayed on a series of four LEDs. With the ignition switch in the "on" position and the engine off, codes will be displayed in ascending order.

Note:

Red * indicates LED on. Blue * indicates LED off.

| LED Display | Symptom | Possible causes |
| --- | --- | --- |
| * * * * with dash warning lamp on | No-start | Disconnected ECU ground, loose connection at ECU main relay resistor, faulty ECU. |
| * * * * with dash warning lamp on | No-start | Short circuit in the warning lamp or instrument cluster wiring, disconnected ECU ground wire, faulty ECU. |
| * * * * | System does not operate. | Faulty ECU. |
| * * * * | System does not operate. | Faulty ECU. |
| * * * * | Fuel fouled spark plugs, engine stalls, hesitation. | MAP sensor electircal connector disconnected, open circuit in MAP sensor wiring, faulty MAP sensor. |
| * * * * | System does not operate. | Faulty ECU. |
| * * * * | Hesitation, fuel fouled spark plugs, engine stalls. | MAP sensor vacuum hose disconnected or broken. |
| * * * * | High idle speed when engine is cold, extended high idle or hard starting at low temperatures. | Coolant temperature sensor or ciruit open. |
| * * * * | Hesitation, poor throttle response, engine does not respond to throttle input when cold. | Throttle angle sensor disconnected, throttle angle sensor circuit open, faulty throttle angle sensor. |
| * * * * | Engine does not rev, high idle speed, erratic idle. | Crank angle sensor circuit shorted or open. Spark plug wire causing interference with crank angle sensor signal. Faulty crank angle sensor. |
| * * * * | Same as above. | Same as above. |
| * * * * | High idle speed, erratic idle when cold. | Intake air temperature sensor disconnected or open intake air temp. circuit. Faulty intake air temperature sensor. |
| * * * * | High idle speed at all times. | Idle mixture adjuster sensor disconnected, idle mixture adjuster sensor ciruit open, faulty idle mixture adjuster. |
| * * * * | System does not operate. | Faulty ECU. |
| * * * * | Poor acceleration at high altitude when cold. | Atmospheric pressure sensor (BARO sensor) disconnected, atmospheric pressure sensor circuit open, faulty atmoshperic pressure sensor. |
| * * * * | System does not operate. | Faulty ECU. |
| * * * * | Same as above. | Same as above. |

86-87 Accord and Prelude

ECU Location

The ECU is located under the driver's seat. With the ignition switch in the "on" position and the engine OFF, the red LED on the ECU will flash the trouble codes. For example, 2 flashes, pause, 3 flashes would mean code 23. Codes are displayed in ascending order.

88-90 Civic, Accord, and Prelude

ECU Location

The ECU is located behind the passenger's side kick panel.

88-91 Civic, Accord, and Prelude

Retrieving Codes

When the CHECK ENGINE warning light has been reported on, turn the ignition ON, pull down the passenger's side carpet inspection flap from under the dashboard, and monitor the LED on the top of the control unit. The LED indicates a system failure code by its blinking frequency.

The control unit LED can indicate any number of simultaneous component problems by blinking separate codes, one after another.

If codes other than those listed are indicated, count the number of blinks again. If the indicator is in fact blinking these codes, substitute a known good ECU and recheck. If the indication goes away, replace the original ECU.

The CHECK ENGINE warning light and control unit LED may come on, indicating a system problem, when in fact, there is a poor or intermittent electrical connection. First, check the electrical connections, clean or repair connections if necessary.

### 92-95 Accord, Civic, Civic Del Sol and Prelude

Retrieving Codes

With ignition off, insert jumper wire in service check connector terminals, located behind right side of dash on Accord, Civic and Civic Del Sol and near center console on Prelude.
Turn ignition switch to ON position. Diagnostic Trouble Codes (DTCs) will be indicated by a series of long and short flashes on MIL. 
The number of long flashes indicates the number in the 10s column. The number of short flashes indicates the number in the 1s column. For example, 4 long flashes followed by 3 short flashes would indicate DTC 43.

Image used with permission Copyright © 2001 ALLDATA LLC.

Clearing Codes

Disconnect the negative battery cable for at least 10 seconds to clear codes.

### Accord, Civic, CRX, CR-V, del Sol, Odyssey & Prelude

PGM-CARB

If the PGM-CARB warning lamp is lit, turn ignition On, pull passenger side carpet inspection flap downward under instrument panel and observe LED on top of control unit. The LED indicates a system failure by its blinking frequency.

The control unit LED can indicate any number of simultaneous component conditions by blinking separate codes, one after another.

If DTCs 7, 9, 11, 12, 13 or any higher than 14 are indicated, count the number of blinks again. If the indicator is in fact blinking these DTCs, substitute a known good control unit and inspect again. If indication goes away, replace the control unit. The PGM-CARB warning lamp and control unit LED may light, indicating a system problem when, in fact, there is a poor or intermittent electrical connection. Clean or repair connections as required.

### PGM-FI Exc del Sol, Odyssey, 90–98 Accord, 90–91 Prelude 2.1L, 
92–98 Prelude & 91–98 Civic

In the event of a PGM-FI system electrical malfunction, an instrument panel mounted indicator lamp will light and stay lit. If the lamp is on, or if a system malfunction is suspected even though the lamp is off, it is necessary to inspect the electronic control unit LED display. The LED display is on the ECM (ECU), which is located just below the RH side of the instrument panel. The indicator lamps operate whenever the ignition is On and a DTC is stored in the ECM (ECU) memory.

When an abnormality is detected the ECM (ECU) display blinks. The LED blinks a certain number of times, with the number of blinks indicating the number of the DTC. After the LED blinks a number of times there will be a two second pause and the LED will blink out the number of the next DTC.

### 90–95 Accord Four Cylinder, 90–91 Prelude 2.1L, 92–95 Prelude & 91–95 Civic

If the malfunction indicator/Check Engine lamp remains lit or lights while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory. To access the DTC memory, connect a jumper wire between the check connector terminals. On Accord and Civic models, the check connector is located under the RH side of the instrument panel. On 90–91 Prelude models equipped with 2.1L engine, the check connector is located in the engine compartment. On 92–95 Prelude models, the check connector is located in the passenger compartment behind the center console.

On all models, turn the ignition On. The ECM indicates stored DTCs by flashing the malfunction indicator/Check Engine lamp. DTCs one through nine are indicated by individual short blinks. DTCs 10 and above are indicated by long and short blinks. One long blink is equivalent to 10 short blinks. The DTC can be determined by adding the long and short blinks.

### 95–97 Accord V6 & 96–97 Accord 4 Cyl.

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located in the center console behind the ashtray. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

To access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located under the passengers side of the instrument panel. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. 95-97 Accord V6 & 96-97 Accord 4 cyl. In Console.
Service check connector location. 95-97 Accord V6 & 96-97 Accord 4 cyl. Passenger side, up under glove box.

### 98–01 Accord

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located under the drivers side of the instrument panel. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

Data Link Connector (DLC) locations. 98–01 Accord Drivers side, up under RH side of dash.

### 96–01 Civic

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located under the drivers side of the instrument panel. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

To access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located under the passengers side of the instrument panel. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. 96-01 Civic Drivers side, up under RH side of dash.
Service check connector location. 96-01 Civic Up underneath dash on passenger side, below or behind glove box.

### CR-V

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located behind the righthand side of the center console. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

To access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located behind the righthand side of the center console. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. CR-V Behind passenger side of console, below dash to the Right of glove box.

Service check connector location. CR-V Up underneath dash on passenger side, below or behind glove box.

### 96–97 del Sol

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located on the righthand side of the center console behind a removable cover. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

To access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located under the righthand side of the instrument panel. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. 96-97 del Sol Behind passenger side of console, below dash to the Right of glove box.

Service check connector location. 96-97 del Sol Passenger side, up under glove box.

### Odyssey

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

On 96–01 models, to access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located behind the righthand side of the center console. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

On 95–98 models, to access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located under the righthand side of the instrument panel. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. 96-98 Odyssey Behind passenger side of console, below dash to the Right of glove box.

Data Link Connector (DLC) locations. 99-01 Odyssey Driver's side, up under RH side of dash near the console.

Service check connector location. 95-98 Odyssey Behind passenger side of console, below dash to the Right of glove box.

### 96 Prelude

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located under the center console beverage holder. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

To access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located behind the front console. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. 96 Prelude In console between seats.

Service check connector location. 96 Prelude Up under Driver's side of the dash, on ht eright near or behind the console.

### 97–01 Prelude

If the malfunction indicator/Check Engine lamp remains lit or comes on while the engine is operating, the ECM has detected a malfunction and has stored a DTC in its memory.

To access the DTC memory using a scan tool, connect the Honda PGM Tester or an OBD II scan tool to the 16-pin DLC. The DLC is located on right side of center console behind the access cover. Turn the ignition On and begin troubleshooting as indicated by the tester, following its prompts to diagnose the error. If unsure how to operate the scan tool or PGM tester, refer to the tool user’s manuals for specific operating instructions.

To access DTC memory using the MIL, connect the SCS service connector to the SCS check connector. The SCS 2 pin connector is located on right side of center console behind the access cover. Place ignition switch in the On position begin MIL flash code diagnosis.

Data Link Connector (DLC) locations. 97–01 Prelude Behind passenger side of console, below dash to the Right of glove box.

Service check connector location. 97–01 Prelude Behind cover on the passenger side of console.

Passport

For Passport procedure, refer to ‘‘Isuzu’’ Rodeo.

## Trouble Codes

| 95 &earlier LED, MIL & Check Engine Lamp | OBD1 |
| --- | --- |
| 96 & later | OBD2 |
| Total Electronics (TE) System | TE codes |

Disclaimer:We cannot guarantee the accuracy of all trouble codes.We try and make them as accurate as we can, but sometimes we all make mistakes. 
Warning: Please check for manufacturer specific codes with an appropriate manual

### Diagnostic Trouble Code Identification

LED, MIL & Check Engine Lamp Flash

| 0 | Engine Control Module |
| --- | --- |
| 1 | HO2S Or O2S |
| 2 | HO2S Or O2S{2} |
|  | VSS{1} |
| 3 | MAP Sensor |
| 4 | CKP Sensor{2} |
|  | Vacuum Switch{1} |
| 5 | MAP Sensor |
| 6 | ECT Sensor |
| 7 | TP Sensor{2} |
|  | M/T Clutch Switch Signal{1} |
|  | A/T Shift Position Signal{1} |
| 8 | TDC{2} |
|  | Ignition Coil Signal{1} |
| 9 | No. 1 Cylinder Position Sensor |
| 10 | IAT Sensor |
| 12 | EGR System |
| 13 | BARO Sensor |
| 14 | IAC Valve{2} |
|  | EACV{1} |
| 15 | Ignition Output Signal |
| 16 | Fuel Injector System |
| 17 | VSS |
| 19 | Lock-Up Solenoid Valve |
| 20 | Electric Load Detector |
| 21 | VTEC Solenoid Valve |
| 22 | VTEC Pressure Switch |
| 23 | KS |
| 30 | A/T FI Signal A |
| 31 | A/T FI Signal B |
| 41 | HO2S Heater |
| 43 | Fuel Supply System |
| 48 | HO2S{3} |

{1}PGM-CARB.
{2}PGM-FI four cylinder engines.
{3}D5Z1 engine except California.

### OBD II

| OBDII | MIL |  |
| --- | --- | --- |
| P0106 | 5 | MAP Circuit Range Or Performance |
| P0107 | 3 | MAP Circuit Low Input |
| P0108 | 3 | MAP Circuit High Input |
| P0111 | 10 | IAT Sensor Circuit Range/Performance |
| P0112 | 10 | IAT Sensor Circuit Low Input |
| P0113 | 10 | IAT Sensor Circuit High Input |
| P0116 | 86 | ECT Circuit Range Or Performance |
| P0117 | 6 | ECT Circuit Low Input |
| P0118 | 6 | ECT Circuit High Input |
| P0122 | 7 | TP Sensor Circuit Low Input |
| P0123 | 7 | TP Sensor Circuit High Input |
| P0131 | 1 | Primary HO2S Circuit Low Voltage (Sensor 1) |
| P0132 | 1 | Primary HO2S Circuit High Voltage (Sensor 1) |
| P0133 | 61 | Primary HO2S Circuit Slow Response (Sensor 1) |
| P0135 | 41 | Front HO2S Heater Circuit Fault (Sensor 1) |
| P0137 | 63 | Secondary HO2S Circuit Low Voltage (Sensor 2) |
| P0138 | 63 | Secondary HO2S Circuit High Voltage (Sensor 2) |
| P0139 | 63 | Secondary HO2S Circuit Slow Response (Sensor 2) |
| P0141 | 65 | Secondary HO2S Heater Circuit Fault (Sensor 2) |
| P0171 | 45 | System Too Lean |
| P0172 | 45 | System Too Rich |
| P0300 | — | Random Misfire |
| P0301 | 71 | Misfire Cyl. 1 Or Random Misfire |
| P0302 | 72 | Misfire Cyl. 2 Or Random Misfire |
| P0303 | 73 | Misfire Cyl. 3 Or Random Misfire |
| P0304 | 74 | Misfire Cyl. 4 Or Random Misfire |
| P0305 | 75 | Misfire Cyl. 5 Or Random Misfire |
| P0306 | 76 | Misfire Cyl. 6 Or Random Misfire |
| P0325 | 23 | KS Circuit Malfunction |
| P0335 | 4 | CKP Sensor Circuit Low Input |
| P0336 | 4 | CKP Sensor Range/Performance |
| P0401 | 80 | EGR Insufficient Flow Detected |
| P0420 | 67 | Catalyst System Efficiency Below Threshold |
| P0441 | 92 | EVAP Emission Control System Improper Purge Flow |
| P0451 | 91 | Fuel Tank Pressure Sensor Range/Performance |
| P0452 | 91 | Fuel Tank Pressure Sensor Circuit Low Input |
| P0453 | 91 | Fuel Tank Pressure Sensor Circuit High Input |
| P0500 | 17 | VSS Circuit Malfunction (M/T) |
| P0501 | 17 | VSS Circuit Range/Performance (A/T) |
| P0505 | 14 | ICS Malfunction |
| P0560 | 34 | Powertrain Control Module (PCM) Backup Voltage Circuit Low Voltage |
| P0700, P0715, P0720, P0725, P0730, P0740, P0753, P0758, P0763, P0780 | 70 | A/T Concerns |
| P1106 | 13 | BARO Circuit Range/Performance |
| P1107 | 13 | BARO Circuit Low Input |
| P1108 | 13 | BARO Circuit High Input |
| P1121 | 7 | Throttle Position Lower Than Expected |
| P1122 | 7 | Throttle Position Higher Than Expected |
| P1128 | 5 | MAP Lower Than Expected |
| P1129 | 5 | MAP Higher Than Expected |
| P1149 | 61 | Primary HO2S (Sensor 1) Circuit Range/Performance Problem |
| P1162 | 48 | Primary HO2S (No. 1) Circuit Malfunction |
| P1163 | 61 | Primary HO2S (No. 1) Circuit Slow Response |
| P1164 | 61 | Primary HO2S (No. 1) Circuit Range/Performance |
| P1165 | 61 | Primary HO2S (No. 1) Circuit Range/Performance |
| P1166 | 41 | Primary HO2S (No. 1) Heater System Electrical |
| P1167 | 41 | Primary HO2S (No. 1) Heater System |
| P1168 | 48 | Primary HO2S (No. 1) LABEL Low Input |
| P1169 | 48 | Primary HO2S (No. 1) LABEL High Input |
| P1253 | 21 | VTEC System Malfunction |
| P1257, P1258, P1259 | 22 | VTEC System Malfunction |
| P1297 | 20 | Electrical Load Detector Circuit Low Input |
| P1298 | 20 | Electrical Load Detector Circuit High Input |
| P1300 | 71–74 | Multiple Cylinder Misfire Detected |
| P1336 | 54 | CSF Sensor Intermittent Interruption |
| P1337 | 54 | CSF Sensor No Signal |
| P1359 | 8 | CKP/TDC Sensor Connector Disconnection |
| P1361 | 8 | TDC Sensor Intermittent Interruption |
| P1362 | 8 | TDC Sensor No Signal |
| P1366 | 58 | TDC Sensor No. 2 Intermittent Interruption |
| P1367 | 58 | TDC Sensor No 2 Signal |
| P1381 | 9 | Cylinder Position Sensor Intermittent Interruption |
| P1382 | 9 | Cylinder Position Sensor No Signal |
| P1456 | 90 | EVAP Emission Control System Leak Detected (Fuel Tank System) |
| P1457 | 90 | EVAP Emission Control System Leak Detected (Control Canister System) |
| P1459 | 92 | EVAP Emission Purge Flow Switch Malfunction |
| P1486 | — | Thermostat Range/Performance Problem |
| P1491 | 12 | EGR Valve Lift Insufficient Detected |
| P1498 | 12 | EGR Valve Lift Sensor High Voltage |
| P1508 | 14 | IAC Valve Circuit Failure |
| P1509 | 14 | IAC Valve Circuit Failure |
| P1519 | 14 | Idle Air Control Valve Circuit Failure |
| P1607 | — | ECM/PCM Internal Circuit Failure A |
| P1655 | 30 | SEAF/SEFA/TMA/TMB Signal Line Failure |
| P1660 | 70 | A/T FI Signal A Circuit Failure |
| P1676 | — | FPTDR Signal Line Failure |
| P1678 | — | FPTDR Signal Line Failure |
| P1681 | 30 | A/T FI Signal A Low Input |
| P1682 | 30 | A/T FI Signal A High Input |
| P1686 | 31 | A/T FI Signal B Low Input |
| P1687 | 31 | A/T FI Signal B High Input |
| P1705, P1706, P1738, P1739, P1753, P1758, P1768, P1773, P1785, P1786, P1790, P1791, P1792, P1793, P1794 | 70 | A/T Concerns |
| P1870, P1873, P1879, P1885, P1886, P1888, P1890, P1891 | 70 | A/T Concerns |

### DTC ID Total Electronics (TE) System

| 1 | Normal |
| --- | --- |
| 2 | No Engine Speed Sensor Pulse |
| 3 | Engine Speed Sensor Circuit |
| 4 | Rack Position Sensor Circuit |
| 6 | Actuator Power Supply Circuit |
| 7 | Actuator Power Supply Relay Circuit |
| 8 | Actuator Circuit |
| 9 | Intake Air Control Magnetic Valve Circuit (H06C-TT, TU, TV) |
| 11 | Coolant Temperature Sensor System |
| 12 | Pre-Stroke Sensor Setting Fault |
| 19 | Pre-Stroke Sensor Circuit |
| 21 | Starter Switch Fault |
| 22 | Control Unit Fault |