## Hyundai Code Retrieval

### Accessing Trouble Codes

### With Multi-Use Tester

Except 1993-95 Scoupe

- Turn ignition switch to Off.

- Connect harness connector of multi-use tester to diagnostic connector in fuse box, located on left kick panel.

- Connect power-source terminal of multi-use tester to cigarette lighter socket.

- Turn ignition switch On, then use multi-use tester to read self-diagnostic codes.

- Repair any malfunctions found using tester.

- Turn ignition switch Off, then disconnect battery ground cable for 10 seconds.

- Check that no malfunction codes are displayed with multi-use tester.

- Disconnect multi-use tester. If a test is to made during cranking, the power to cigarette lighter will be interrupted therefore, use a separate battery harness for multi-use tester.

### With Analog Voltmeter

Except 1993-95 Scoupe

- Connect voltmeter to self-diagnosis connector, located in fuse box on left kick panel.

- Turn ignition switch On and note an immediate indication of ECM (ECU) memory contents will begin. If system is in normal condition, pointer of voltmeter indicates a normal pattern. If any abnormality is in the memory, pointer of voltmeter will deflect indicating an abnormal condition.

- After recording abnormal items, check and repair each part.

- After repairs have been completed, disconnect battery ground cable for at least 15 seconds to erase codes from memory.

### Using On-Board Diagnostic Lamp

1993-95 Scoupe

- With ignition switch in the On position, ground the No. 10 terminal (L wire) of diagnostic connector for approximately 2.5 seconds.

- The diagnostic connector is located on the left hand kick panel. The first diagnostic trouble code or no fault detected will be indicated by the on-board diagnostic lamp. The onboard diagnostic lamp will indicated DTC's by flashing frequencies and 2 second pauses. As an example, one flash, followed by a 2 second pause, followed by two flashes and another 2 second pause, followed by two flashes and a 2 second pause, followed by three flashes and another 2 second pause, indicates a failure code 1223. The fault code will be repeated until the No. 10 terminal of the diagnostic connector is grounded to proceed to the next fault code.

- The last output consist of the end of output code 3333.

### Clearing Trouble Codes

To erase any diagnostic codes, disconnect battery ground cable for 15 seconds and the ECM (ECU) memory will be erased.

### COMPUTER CODES for Hyundai Vehicles.

If codes other than these are indicated, repeat self-diagnosis. 
If codes reappear, substitute a known good ECM, and recheck codes.

| CODE | Hyundai Stellar vehicles. |
| --- | --- |
| 1 | Oxygen sensor or circuit |
| 2 | Ignition signal |
| 3 | Airflow sensor or circuit |
| 4 | Atmospheric pressure sensor or circuit |
| 5 | Throttle position sensor or circuit |
| 6 | Idle Speed Control (ISC) motor position sensor or circuit |
| 7 | Coolant temperature sensor or circuit |
| 8 | TDC sensor or circuit |
| 9 | System normal |

| CODE | Hyundai Sonata, Excel(90 on), Scoupe (91 & 92), Elantra. |
| --- | --- |
| 1 | Electronic Control Unit (ECU) (one long needle sweep) |
| 2 | ECU system normal |
| 11 | Oxygen sensor or circuit |
| 12 | Airflow sensor or circuit |
| 13 | Intake air temperature sensor or circuit |
| 14 | Throttle Position Sensor (TPS) or circuit |
| 15 | Motor position sensor or circuit |
| 21 | Coolant temperature sensor or circuit |
| 22 | Crank angle sensor or circuit |
| 23 | TDC sensor or circuit |
| 24 | Vehicle Speed Sensor or circuit |
| 25 | Barometric pressure sensor or circuit |
| 41 | Fuel injector or circuit |
| 42 | Fuel pump or circuit |
| 43 | EGR system |
| 44 | Ignition coil |
| 59 | Oxygen sensor |

| CODE | Hyundai Scoupe (93 through 95) and Accent vehicles. |
| --- | --- |
| 1122 | ECM failure (ROM/RAM) |
| 1169 | ECM failure |
| 1233 | ECM failure (ROM) |
| 1234 | ECM failure (RAM) |
| 2121 | Turbo boost sensor control valve |
| 3112 | NO. 1 fuel injector |
| 3114 | Idle air control (opening failure) |
| 3116 | NO. 3 fuel injector |
| 3117 | Mass airflow sensor |
| 3121 | Turbo boost sensor failure |
| 3122 | Idle air control (closing failure) |
| 3128 | Heated oxygen sensor |
| 3135 | EVAP purge control solenoid valve |
| 3137 | Alternator output low |
| 3145 | Engine coolant temperature (ECT) sensor |
| 3149 | Air conditioning compressor |
| 3152 | Turbocharger overboost |
| 3153 | Throttle position switch (TPS) |
| 3159 | Vehicle speed sensor (VSS) |
| 3211 | Knock sensor |
| 3222 | Phase sensor |
| 3224 | ECM failure (knock evaluation circuit) |
| 3232 | Crank position sensor |
| 3233 | Same as 3224 |
| 3234 | NO. 2 fuel injector |
| 3235 | NO. 4 fuel injector |
| 3241 | ECM failure (injector or purge solenoid) |
| 3242 | ECM failure (IAC or air conditioning relay) |
| 3243 | ECM failure |
| 4133 | ECM failure |
| 4151 | Air/fuel control |
| 4152 | Air/fuel adaptive failure |
| 4153 | Air/fuel multiple adaptive failure |
| 4154 | Air/fuel additive adaptive failure |
| 4155 | ECM failure (A/C relay, IAC, PCV or injector) |
| 4156 | Same as code 3121 |
| 4444 | Normal |
| 3333 | End of trouble codes |