# Infiniti

### Without OBD II

On G20 models, the ECU is located behind the center console. On J30, M30 and Q45 models, the ECU is located behind the RH kick panel.

If a malfunction occurs within the ECU/fuel injection system, a diagnostic trouble code may be stored in the ECU and the ‘‘Check Engine’’ lamp will blink. There is also a lamp built into the ECU which will concurrently blink. To read Code output, proceed as follows:

- Turn ignition On, but do not start engine.

- Using a small screwdriver, turn diagnostic mode selector switch, located on side of ECU, fully clockwise.

- Wait two seconds, then rotate switch to full counterclockwise position.

- A malfunction code can be read as the long (0.6 second) blink, indicating the first digit of a two digit number and the short (0.3 second) blink, indicating the second digit.

Ensure that the diagnostic switch is to the full counterclockwise position whenever the vehicle is in use.

## w/OBD II

On G20 and I30 models, connect a suitable Scan Tool to the DLC, located under instrument panel fuse box cover.

On J30, Q45 and QX4 models, connect a suitable Scan Tool to the DLC, located under or on lower portion of instrument panel.

## Clearing Codes

If using a scan tool, follow tool manufacturer’s instructions to clear Codes.

A second, though more time consuming method of clearing Codes involves using a small screwdriver to turn the diagnostic mode selector switch, located on the side of the ECM/ECU, fully clockwise. After waiting two seconds, rotate the switch to the full counterclockwise position. The diagnosis will end and memory will then be cleared.

### Check Engine Lamp

| 11 | Crank Angle Sensor |
| --- | --- |
| 12 | Mass Air Flow Sensor |
| 13 | ECT Sensor |
| 14 | Vehicle Speed Sensor |
| 16 | TCS Signal |
| 21 | Ignition Signal Circuit |
| 25 | Idle Air Control Valve |
| 22 | Fuel Pump Control |
| 31 | ECM |
| 32 | EGR Function |
| 33 | Heated Oxygen Sensor |
| 34 | Knock Sensor |
| 35 | EGR Temperature Sensor (RH) |
| 41 | Intake Air Temperature Sensor |
| 42 | Fuel Pump Sensor |
| 43 | Throttle Position Sensor |
| 45 | Injector Leak |
| 46 | Secondary Throttle Sensor |
| 51 | Injector Analysis |
| 53 | EGR Temperature Sensor (LH) |
| 54 | Signal Circuit From A/T Control Unit To ECM |
| 65-71 | Single Cylinder Or Multiple Cylinder Misfire |
| 72 | Three Way catalyst Function |
| 76 | Fuel Injection System Function |
| 77 | Rear Heated Oxygen Sensor |
| 82 | Crankshaft Position Sensor |
| 84 | A/T Diagnosis Communication Line |
| 91 | Front Heated Oxygen Sensor Heater |
| 103 | Park/Neutral Position Switch |
| 105 | EGR & Canister Control Solenoid |

### OBD2 Codes

| P0000 | No Self Diagnostic Failure Indicated |
| --- | --- |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130 | Closed Loop, Bank 1 Or Front O2S, Bank 1 |
| P0131 | Front O2 Sensor, Bank 1 Lean Shift Monitoring |
| P0132 | Front O2 Sensor, Bank 1 Rich Shift Monitoring |
| P0133 | Front O2 Sensor, Bank 1 Response Monitoring |
| P0134 | Front O2 Sensor, Bank 1 High Voltage |
| P0135 | Front HO2S Heater, Bank 1 |
| P0136 | Rear O2S, Bank 1 |
| P0137 | Rear O2S, Bank 1 Minimum Voltage Monitoring |
| P0138 | Rear O2S, Bank 1 Maximum Voltage Monitoring |
| P0139 | Rear O2S, Bank 1 Response Monitoring |
| P0140 | Rear O2S, Bank 1 High Voltage |
| P0141 | Rear H02S Heater, Bank 1 |
| P0150 | Closed Loop, Bank 2 Or Front O2S, Bank 2 |
| P0151 | Front O2 Sensor, Bank 2 Lean Shift Monitoring |
| P0152 | Front O2 Sensor, Bank 2 Rich Shift Monitoring |
| P0153 | Front O2 Sensor, Bank 2 Response Monitoring |
| P0154 | Front O2 Sensor, Bank 2 High Voltage |
| P0155 | Front HO2S Heater, Bank 2 |
| P0156 | Rear H02S Sensor, LH Bank |
| P0157–P0160 | Rear O2S, Bank 2 |
| P0161 | Rear H02S Heater, LH Bank Or Bank 2 |
| P0171 | Fuel System Lean, Bank 1 |
| P0172 | Fuel System Rich, Bank 1 |
| P0174 | Fuel System Lean, Bank 2 |
| P0175 | Fuel System Rich, Bank 2 |
| P0180 | Tank Fuel Temp Sensor |
| P0300 | Random Misfire |
| P0301–P0308 | Misfire, Cylinder No. 1–8 |
| P0325 | Knock Sensor, Bank 1 |
| P0330 | Knock Sensor, Bank 2 |
| P0335 | CKP Sensor (Pos) |
| P0340 | CMP |
| P0400 | EGR System |
| P0402 | EGRC/BPT Valve |
| P0403 | EGR Volume Control Valve Circuit |
| P0420 | TW Catalyst System, Bank 1 |
| P0430 | TW Catalyst System, Bank 2 |
| P0440 | EVAP Small Leak |
| P0443 | Purge Control/V & S/V |
| P0446 | Vent Control Valve |
| P0450 | EVAP Pressure Sensor |
| P0500 | Vehicle Speed Sensor |
| P0505 | IACV/AAC Valve |
| P0510 | Closed TP Sensor |
| P0600 | A/T Comm Line |
| P0605 | ECM/ECU |
| P0705 | PNP Or Inhibitor Switch |
| P0710 | ATF Temp Sensor |
| P0720 | VSS A/T |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1ST Signal |
| P0732 | A/T 2ND Signal |
| P0733 | A/T 3RD Signal |
| P0734 | A/T 4TH Signal Or TCC |
| P0740 | TCC SV Solenoid |
| P0744 | A/T TCC Signal |
| P0745 | Line Pressure S/V |
| P0750 | Shift Solenoid/V A |
| P0755 | Shift Solenoid/V B |
| P1105 | MAP/BARO Switch SOL/V |
| P1110 | Intake Valve Timing Control, LH Bank |
| P1120 | TP Sensor 2 |
| P1125 | Tandem TP Sensor |
| P1130 | Swirl Control Valve Control Solenoid Valve |
| P1135 | Intake Valve Timing Control, RH Bank 2 |
| P1140 | Intake Valve Timing Control Position Sensor, LH Bank 1 |
| P1145 | Intake Valve Timing Control Position Sensor, RH Bank 2 |
| P1148 | Closed Loop, Bank 1 |
| P1165 | Swirl Control Valve Control Vacuum Check Switch |
| P1168 | Closed Loop, Bank 2 |
| P1210 | Traction Control Signal Circuit |
| P1220 | FPCM |
| P1320 | Ignition Signal, Primary |
| P1335 | CKP Sensor (Ref) |
| P1336 | CKP Sensor (POS) Cog |
| P1400 | EGRC Solenoid/V |
| P1401 | EGR Temp Sensor |
| P1402 | EGR System |
| P1440 | EVAP Small Leak |
| P1441 | VC/V Bypass/V |
| P1443 | Canister Control Vacuum Check Switch |
| P1444 | Purge Volume Control/V |
| P1445 | Purge Volume Control/V |
| P1446 | Vent Control Valve |
| P1447 | EVAP Purge Flow |
| P1448 | Vent Control Valve |
| P1490 | VC/V Bypass/V |
| P1491 | VC Cut/V Bypass/V |
| P1492 | Purge Control/V S/V |
| P1493 | Purge Control/V & S/V |
| P1605 | A/T Diag Comm Line |
| P1705 | TP Sensor A/T |
| P1706 | PNP Switch |
| P1760 | Overrun Clutch S/V |
| P1900 | Cooling Fan |