## Mazda Code Retrieval

### Accessing Trouble Codes 
96 & later are OBD2 systems

| 91 | Mazda B2000 pickup |
| --- | --- |

Carbureted Models

System malfunctions detected and stored in the emission control unit can be displayed using the self-diagnosis checker tool No. 40-H018-9A1. Any current or memorized malfunctions are indicated by a displayed code number and a corresponding buzzer. A monitor lamp on the checker indicates oxygen sensor operation. A normal air/fuel ratio is indicated by a continuous flashing of the lamp. The lamp will not light continuously if the air/fuel is lean and will not light at all if the mixture is rich. Refer to the manufacturers instructions for operating procedures of the self-checker tool.

Fuel Injected Models

A system selector tool No. 49-B019-9AO or equivalent on OBD systems and tool No. 49-T088-OAO or equivalent on OBD II systems, can be used to diagnose the main input or output devices. Systems are displayed as code numbers appearing on the system checker. Refer to the manufacturers instructions for operating procedures.

### Clearing Trouble Codes

Carbureted Models

Disconnect battery ground cable for a minimum of 10 seconds to clear trouble code stored in the memory.

Fuel Injected Models With OBD

Disconnect battery ground cable for a minimum of 10 seconds, then depress brake pedal five times to clear trouble code's stored in the memory.

Fuel Injected Models With OBD II

Connect SST (NGS) tool to data link connector 2. Then select "Clear Diagnosis Codes" function and erase diagnostic trouble codes from the NGS memory.

# Diagnostic Codes

## Mazda

| Carbureted Vehicles |  |
| --- | --- |
| 1 | No1 Ign Pulse |
| 9 | Water Thermo Sensor |
| 13 | Vacuum Sensor |
| 14 | Atmospheric Pressure Sensor |
| 15 | Oxygen Sensor |
| 16 | EGR Position Sensor |
| 17 | Feedback System |
| 18 | A/F Solenoid Valve |
| 22 | Slow Fuel Cut Solenoid |
| 23 | Coasting Richer Solenoid or Circuit |
| 26 | Purge Solenoid Valve or Circuit |
| 28 | Duty Solenoid Vacuum Valve or Circuit |
| 29 | Duty Solenoid Vent Valve or Circuit |
| 30 | ACV Solenoid Valve or Circuit |
| 34 | Idle-Up Solenoid Valve (for A/C) or Circuit |
| 35 | Idle-Up Solenoid Valve (for A/T) or Circuit |
| 45 | Vacuum Solenoid Valve or Circuit |

| Fuel Injected (Before 95) Except RX-7 (RX-7 See Below) |  |
| --- | --- |
| 1 | Ignition Pulse. |
| 2 | Distributor - No 2 Signal. |
| 2 | No 2 Signal (626 and MX-6). |
| 2 | Crank Angle Sensor - No 2 Signal (1992-1995 929, MX-3 1.8L). |
| 3 | Distributor - G or G1 Signal. |
| 3 | Crank Angle Sensor - G Signal (MX-3 1.8L). |
| 4 | Distributor - No #1 Signal or G2 Signal. |
| 4 | Crank Angle Sensor - No 1 Signal (MX-3 1.8L). |
| 5 | Knock Sensor. |
| 6 | Vehicle Speed Sensor. |
| 6 | Air Flow Meter (MX-3 1.8L). |
| 7 | Knock Sensor - right hand side. |
| 8 | Air Flow Meter. |
| 9 | Water Thermo Sensor. |
| 10 | Intake Air Thermo Sensor - Air Flow Meter. |
| 11 | Intake Air Thermo Sensor - Dynamic Chamber. |
| 12 | Throttle Sensor. |
| 14 | Atmospheric Pressure Sensor. |
| 15 | Oxygen Sensor or Heated Oxygen Sensor - left hand side. |
| 16 | EGR Position/Function Sensor. |
| 17 | Feedback System or Closed Loop System - left hand side. |
| 23 | Oxygen Sensor or Heated Oxygen Sensor - right hand side. |
| 24 | Feedback System or Closed Loop System - right hand side. |
| 25 | Solenoid Valve - Pressure Regulator. |
| 26 | Solenoid Valve. |
| 27 | Solenoid Valve. |
| 28 | Solenoid Valve - EGR Vacuum side. |
| 29 | Solenoid Valve - EGR Vent side. |
| 30 | Cold Start Injector Relay. |
| 34 | IAC Valve. |
| 36 | Heated Oxygen Sensor - right hand side. |
| 37 | Heated Oxygen Sensor - left hand side. |
| 40 | Solenoid Valve - Triple Induction Control System & 02S Replay. |
| 40 | Solenoid Valve - Variable Resonance Induction System Solenoid Valve - VRIS 1. |
| 42 | Wastegate Solenoid Valve. |
| 46 | Solenoid Valve - VRIS 2. |
| 65 | A/C Signal - PCMT. |
| 67 | Coolant Fan Relay No. 1. |
| 69 | ECT Sensor. |

| Fuel Injected (Before 95) RX-7 |  |  |
| --- | --- | --- |
| 1 | Ignition Coil - Trailing Side (1989-1991). |  |
| 1 | Crank Angle Sensor (1984-1988). |  |
| 2 | Crank Angle Sensor - No Signal. |  |
| 2 | Air Flow Meter (1984-1988). |  |
| 3 | Crank Angle Sensor - G Signal. |  |
| 3 | Water Thermo Sensor (1984-1988). |  |
| 4 | Intake Air Thermo Sensor. |  |
| 5 | Knock Sensor. |  |
| 5 | Oxygen Sensor (1984-1988). |  |
| 6 | Speedometer Sensor. |  |
| 6 | Throttle Sensor (1984-1988). |  |
| 7 | Boost Sensor (With Turbocharger, 1984-1988). |  |
| 7 | Pressure Sensor (Without Turbocharger, 1984-1988). |  |
| 8 | Air Flow Meter. |  |
| 9 | Water Thermo Sensor. |  |
| 9 | Atmospheric Pressure Sensor (1984-1988). |  |
| 10 | Intake Air Thermo Sensor. |  |
| 11 | Intake Air Thermo Sensor (1992-1995). |  |
| 11 | Throttle Sensor - Full Range (1989-1991). |  |
| 12 | Throttle Sensor - Full Range. |  |
| 12 | Coil with Igniter - Trailing Side (1984-1988). |  |
| 13 | Pressure Sensor. |  |
| 14 | Atmospheric Pressure Sensor. |  |
| 15 | Oxygen Sensor. |  |
| 15 | IAT Sensor - Dynamic Chamber or Intake Air Pipe (1984-1988). |  |
| 16 | EGR Switch. |  |
| 17 | Feedback System. |  |
| 18 | Throttle Sensor - Narrow Range. |  |
| 20 | Metering Oil Pump Position Sensor. |  |
| 23 | Fuel Thermo Sensor. |  |
| 25 | Solenoid Valve - Pressure Regulator Control. |  |
| 26 | Metering Oil Pump - Stepping Motor. |  |
| 27 | Metering Oil Pump. |  |
| 30 | Split Air Solenoid Valve. |  |
| 31 | Solenoid Valve - Relief. |  |
| 32 | Solenoid Valve - Switch. |  |
| 33 | Port Air Solenoid Valve. |  |
| 34 | Solenoid Valve - Idle Speed Control. |  |
| 37 | Metering Oil Pump. |  |
| 38 | Solenoid Valve - Accelerated Warm Up System. |  |
| 39 | Solenoid Valve - Relief 2. |  |
| 40 | Solenoid Valve - Purge Control. |  |
| 42 | Solenoid Valve - Turbo Pre Control. |  |
| 43 | Solenoid Valve - Wastegate Control. |  |
| 44 | Solenoid Valve - Turbo Control. |  |
| 45 | Solenoid Valve - Charge Control. |  |
| 46 | Solenoid Valve - Charge Relief. |  |
| 50 | Solenoid Valve - Double Throttle Control. |  |
| 51 | Fuel Pump Relay. |  |
| 54 | Air Pump Relay. |  |
| 71 | Injector - Front Secondary. |  |
| 73 | Injector - Rear Secondary. |  |
| 76 | Slip Lockup Off Signal. |  |
| 77 | Torque Reduced Signal. |  |
| ### 1991 Mazda Truck B Series Truck |  |  |

Test Connector

When problems occur in ECU monitored input or output devices, test the system to find the cause. During testing, failures of input and output devices are retrieved from the ECU as malfunction code numbers and displayed on the dash mounted malfunction indicator light (MIL).

| NOTE: | The ECU constantly checks for input device malfunctions. However, it only checks for output device malfunctions in a 3 second period after the ignition switch is turned "ON" with test connector grounded. If there is more than one malfunction code present, the lowest number code is displayed first, followed by subsequent codes in numerical order. This procedure is only applicable to vehicles with an MIL system that will display codes. |
| --- | --- |

INSPECTION PROCEDURE

- Warm up engine to normal operating temperature and turn ignition "OFF."

- Ground test connector (Green: 1-pin) with a jumper wire.

| NOTE: | Test connector is located next to self-diagnostic connector. |
| --- | --- |

- Turn ignition switch "ON."

- If MIL illuminates, goes off, and stays off, no codes are present.

- If MIL illuminates, goes off, and then starts flashing, codes are present.

- Start engine and check for additional codes.

- If any codes are retrieved, refer to Malfunction Code Charts under DIAGNOSTIC CHARTS. Repair as required.

AFTER-REPAIR PROCEDURE

- Erase ECU malfunction code memory by disconnecting the negative battery cable and depressing the brake pedal for at least five seconds. Reconnect battery cable.

- Ensure engine is at normal operating temperature. Ground test connector.

- Turn ignition switch "ON."

- If MIL illuminates, goes off, and stays off, no codes are present.

- If MIL illuminates, goes off, and then starts flashing, codes are present.

- Start the engine and check for additional codes.

- If any codes are retrieved, repair problem and repeat after-repair procedure.

Diagnostic codes

| 1 | No1 Ign Pulse |
| --- | --- |
| 9 | Water Thermo Sensor |
| 13 | Vacuum Sensor |
| 14 | Atmospheric Pressure Sensor |
| 15 | Oxygen Sensor |
| 16 | EGR Position Sensor |
| 17 | Feedback System |
| 18 | A/F Solenoid Valve |
| 22 | Slow Fuel Cut Solenoid |
| 23 | Coasting Richer Solenoid or Circuit |
| 26 | Purge Solenoid Valve or Circuit |
| 28 | Duty Solenoid Vacuum Valve or Circuit |
| 29 | Duty Solenoid Vent Valve or Circuit |
| 30 | ACV Solenoid Valve or Circuit |
| 34 | Idle-Up Solenoid Valve (for A/C) or Circuit |
| 35 | Idle-Up Solenoid Valve (for A/T) or Circuit |
| 45 | Vacuum Solenoid Valve or Circuit |