# Mitsubishi Trouble Code Info ### 96 & later are OBD2 systems and require a scanner. ### Accessing Diagnostic Trouble Codes - Turn ignition switch to OFF position - Locate Data Link Connector (DLC), next to fuse box. - Connect voltmeter positive lead to DLC terminal #1 and Negative lead to terminal #12 (ground). - Turn Ignition switch to ON position. - codes are read out by 12 volt pulses of voltmeter. - A constant repetition of short pulses is normal. - Signals will appear on voltmeter a long and short 12 volt pulses. - long pulses represent tens - short pulses represent ones (4 long pulses and 3 short pulses indicates code 43) 94-95 Mitsubishi Diamante Retrieval Without Scan Tool Other 95 models require a diagnostic trouble code check harness(part # MB991529) to ground terminal #1 images used with permission Copyright © 2001 ALLDATA LLC. - Ground Data Link Connector terminal # 1. - Set the ignition switch to "ON", the Malfunction Indicator (check engine) Lamp will illuminate for a particular duration and a certain number of times. - Clear the trouble codes by disconnecting the vehicle battery negative terminal for 16 seconds or longer EXAMPLES No Codes--normal - The Lamp will illuminate for 0.5 seconds, then stay Off for 0.5 seconds, this pattern will repeat until the ignition is turned OFF. Code 13 - The Lamp will illuminate one time for approximately 0.5 second. This will represent one unit of ten. There will be approximately a one second delay and then the Lamp will illuminate three times for approximately 0.2 seconds with 0.2 second delay between deflections. This will represent three units of one. Code 23 - The Lamp will illuminate two times for approximately 0.5 second with approximately 0.3 second delay between deflections. This will represent two units of ten. There will be approximately a one second delay and then the Lamp will illuminate three times for approximately 0.2 seconds with 0.2 second delay between deflections. This will represent three units of one.. ### Clearing Trouble Codes Diagnostic trouble codes may be cleared by disconnecting the battery ground cable for at least 20 seconds. Reconnect negative battery cable and recheck codes to confirm the repair.

# Diagnostic Codes

## Mitsubishi

| Without OBD-II |  |
| --- | --- |
| 11 | Oxygen sensor fault. |
| 12 | Airflow sensor fault. |
| 13 | Intake air temp sensor fault. |
| 14 | Throttle position sensor (TPS) fault. |
| 15 | Idle speed control (ISC) motor position sensor fault. |
| 21 | Coolant temp sensor fault. |
| 22 | Crankshaft position (CKP) sesnor fault. |
| 23 | Camshaft position (CMP) sensor fault. |
| 24 | Vehicle speed sensor (VSS) fault. |
| 25 | Barometric (BARO) pressure sensor fault. |
| 31 | Knock sensor fault. |
| 32 | MAP sensor faulty. |
| 36 | Ignition timing adjustment signal fault. |
| 39 | Oxygen (O2) sensor fault. |
| 41 | Injector(s) fault |
| 42 | Fuel pump fault. |
| 43 | EGR fault. |
| 44 | Ignition coil (cylinders #1 & #4) fault. |
| 52 | Ignition coil (cylinders #2 & #5) fault. |
| 53 | Ignition coil (cylinders #3 & #6) fault. |
| 55 | Idle air control (IAC) valve position sensor fault. |
| 59 | Rear Oxygen (O2) sesnor fault. |
| 61 | Transaxle control module torque reduction signal fault. |
| 62 | Variable Induction control (VIC) valve position sensor fault. |
| 71 | Traction control (TC) vacuum valve solenoid fault. |
| 72 | Traction Control (TC) vent valve solenoid fault. |

| ECI Voltmeter or Test Light |  |
| --- | --- |
| 1 | Oxygen Sensor. |
| 2 | Ignition Signal. |
| 3 | Air Flow Sensor. |
| 4 | Pressure Sensor. |
| 5 | Throttle Position Sensor. |
| 6 | Idle Speed Control Motor Position Switch. |
| 7 | Engine Coolant Temperature Sensor. |

| MPFI 86-88 4 Cyl. Voltmeter or Test Light |  |
| --- | --- |
| 10000 | Oxygen Sensor. |
| 01000 | Crank Angle Sensor. |
| 11000 | Air Flow Sensor. |
| 00100 | Atmospheric Pressure Sensor. |
| 10100 | Throttle Position Sensor. |
| 01100 | Motor Position Sensor. |
| 11100 | Engine Coolant Temperature Sensor. |
| 00010 | No. 1 Cylinder TDC Sensor. |

| MPFI V6 and 89-95 4 Cyl. (Exc. 95 Eclipse) Voltmeter or Test Light |  |
| --- | --- |
| 11 | Oxygen Sensor. |
| 12 | Air Flow Sensor. |
| 13 | Intake Air Temperature Sensor. |
| 14 | Throttle Position Sensor. |
| 15 | Motor Position Sensor. |
| 21 | Engine Coolant Temperature Sensor. |
| 22 | Crank Angle Sensor. |
| 23 | TDC/CMP Sensor. |
| 24 | Vehicle Speed Sensor. |
| 26 | BARO Sensor. |
| 31 | Knock Sensor. |
| 32 | Manifold Absolute Pressure Sensor. |
| 36 | Ignition Timing Adjustment Signal. |
| 39 | Oxygen Sensor. |
| 41 | Injector. |
| 42 | Fuel Pump. |
| 43 | EGR. |
| 44 | Ignition Coil, 4 Cylinder. |
| 44 | Ignition Coil Power Transistor Unit, V6 Cylinders 1 & 4. |
| 52 | Ignition Coil Power Transistor Unit, V6 Cylinders 2 & 5. |
| 53 | Ignition Coil Power Transistor Unit, V6 Cylinders 3 & 6. |
| 55 | Idle Air Control Valve/Servo Valve Position Sensor. |
| 59 | Oxygen Sensor (Rear), 4 Cylinder. |
| 59 | Left hand Bank Heated Oxygen Sensor, V6. |
| 61 | Cable from Transaxle Control Unit, For Transmission |
| 61 | Torque Reduction Signal (A/T). |
| 62 | Induction Control Valve Position Sensor (Non Turbo). |
| 69 | Right hand Bank Heated Oxygen Sensor. |
| 71 | Traction Control Vacuum Solenoid. |
| 72 | Traction Control Ventilation Solenoid. |

95-98 Eclipse (Non-Turbo) Retrieval Without Scan Tool

- Cycle the ignition key On-Off-On-Off-On within 5 seconds.

images used with permission Copyright © 2001 ALLDATA LLC.

- Count the number of times the malfunction indicator lamp (check engine lamp) on the instrument panel flashes on and off. The number of flashes represents the trouble code. There is a slight pause between the flashes representing the first and second digits of the code. Longer pauses (approx. 4 seconds) separate individual trouble codes.

- Repair the malfunction while referring to the inspection chart for diagnostic trouble codes.

- After disconnecting the battery cable from the battery (-) terminal for 10 seconds or more, reconnect the cable.

- Start the engine and run it at idle for about 15 minutes after the engine has warmed up.

| MPFI 95-98 Eclipse (Non-Turbo) Check Engine/Light Scan Tool |  |
| --- | --- |
| 11 | Intermittent Loss Of Crankshaft Position Sensor. |
| 11 | Misfire Adaptive Numerator at Limit. |
| 11 | No Crankshaft Reference Signal at PCM. |
| 11 | Timing Belt Skipped 1 Tooth or More. |
| 13 | No Change In Manifold Absolute Pressure (MAP) From Start To Run. |
| 14 | 5-Volt Supply Output Too Low. |
| 14 | Manifold Absolute Pressure (MAP) Sensor Voltage Too High. |
| 14 | Manifold Absolute Pressure (MAP) Sensor Voltage Too Low. |
| 14 | No 5 Volts To Manifold Absolute Pressure (MAP) Sensor. |
| 15 | No Vehicle Speed Sensor Signal. |
| 16 | Knock Sensor No. 1 Circuit. |
| 17 | Closed Loop Temperature Not Reached. |
| 17 | Engine Is Cold Too Long. |
| 21 | Downstream Heated Oxygen Sensor Heater Failure. |
| 21 | Downstream Heated Oxygen Sensor Shorted To Voltage. |
| 21 | Downstream Heated Oxygen Sensor Stays at Center. |
| 21 | Downstream Heated Oxygen Sensor Stays at Center. |
| 21 | Downstream Heated Oxygen Sensor Voltage Grounded. |
| 21 | Slow Upstream Heated Oxygen Sensor Circuit during Catalyst Monitor. |
| 21 | Upstream Heated Oxygen Sensor Heater Failure. |
| 21 | Upstream Heated Oxygen Sensor Response. |
| 21 | Upstream Heated Oxygen Sensor Shorted To B+. |
| 21 | Upstream Heated Oxygen Sensor Stays at Center. |
| 21 | Upstream Heated Oxygen Sensor Voltage Grounded. |
| 22 | Engine Coolant Temperature Sensor Voltage High. |
| 22 | Engine Coolant Temperature Sensor Voltage Low. |
| 23 | Intake Air Temperature Sensor Voltage High. |
| 23 | Intake Air Temperature Sensor Voltage Low. |
| 24 | No 5 Volts To Throttle Position Sensor. |
| 24 | Throttle Position Sensor Voltage Disagrees With Manifold Absolute Pressure. |
| 24 | Throttle Position Sensor Voltage High. |
| 24 | Throttle Position Sensor Voltage Low. |
| 25 | Idle Air Control Motor Circuits. |
| 25 | Target Idle Not Reached (plus or minus 200 RPM). |
| 27 | Injector No. 1 Control Circuit. |
| 27 | Injector No. 2 Control Circuit. |
| 27 | Injector No. 3 Control Circuit. |
| 27 | Injector No. 4 Control Circuit. |
| 31 | EVAP Leak Monitor Pinched Hose found. |
| 31 | EVAP Purge Flow Monitor Failure. |
| 31 | EVAP Solenoid Circuit. |
| 31 | EVAP Solenoid Switch or Mechanical Fault. |
| 31 | EVAP System Large Leak. |
| 31 | EVAP System Small Leak. |
| 31 | EVAP Vent Solenoid Circuit. |
| 31 | Engine Coolant Temp sensor voltage too high. |
| 32 | EGR Solenoid Circuit. |
| 32 | EGR System Failure. |
| 33 | A/C Clutch Relay Circuit. |
| 35 | High Speed Fan Control Relay Circuit. |
| 35 | High Speed Radiator Fan Relay Circuit. |
| 35 | Low Speed Fan Control Relay Circuit. |
| 36 | PAIR Solenoid Circuit - M/T. |
| 36 | Too Little or Too Much Secondary Air. |
| 37 | Park/Neutral Position Switch Failure. |
| 41 | Alternator Field Not Switching Properly. |
| 42 | Fuel Level Sensing Unit Voltage Too High. |
| 42 | Fuel Level Sensing Unit Voltage Too Low. |
| 42 | Fuel Level Unit No Change Over Miles. |
| 42 | Fuel Pump Relay Control Circuit. |
| 42 | MFI (ASID) Relay Control Circuit. |
| 42 | No MFI (ASD) Relay Output Voltage at PCM. |
| 43 | Cylinder No. 1 Misfire. |
| 43 | Cylinder No. 2 Misfire. |
| 43 | Cylinder No. 3 Misfire. |
| 43 | Cylinder No. 4 Misfire. |
| 43 | Ignition Coil No. 1 Primary Circuit. |
| 43 | Ignition Coil No. 2 Primary Circuit. |
| 43 | Multiple Cylinder Misfire. |
| 44 | Battery Temperature Sensor Voltage Too High. |
| 44 | Battery Temperature Sensor Voltage Too Low. |
| 44 | Battery Temperature Voltage out of limit. |
| 45 | EATX Controller Trouble Code Present. |
| 46 | Charging System Voltage Too High. |
| 47 | Charging System Voltage Too Low. |
| 51 | Fuel System Lean. |
| 52 | Fuel System Rich. |
| 53 | Internal Controller Failure. |
| 53 | PCM Failure - Serial Peripheral Interface Communications. |
| 54 | No Cam Signal at PCM. |
| 61 | BARO Pressure Out Of Range. |
| 62 | PCM Failure - Service Reminder Indicator Mile Not Stored. |
| 63 | PCM Failure - EEPROM Write Denied. |
| 64 | Catalytic Converter Efficiency Failure. |
| 65 | Power Steering Pressure Switch Failure. |
| 66 | No CCD Messages From TCM - A/T. |

| 95 Eclipse (Turbo) 96-98 Vehicles OBD2 |  |
| --- | --- |
| P0100 | Volume Air Flow circuit. |
| P0105 | BARO Pressure circuit. |
| P0110 | Intake Air Temperature circuit. |
| P0115 | Engine Coolant Temperature circuit. |
| P0120 | Throttle Position circuit. |
| P0125 | Excessive Time to Enter Closed Loop Fuel Control. |
| P0130 | Heated Oxygen Sensor circuit Bank 1 Sensor 1. |
| P0135 | Heated Oxygen Sensor Heater circuit Bank 1 Sensor 1. |
| P0136 | Heated Oxygen Sensor circuit Bank 1 Sensor. |
| P0141 | Heated Oxygen Sensor Heater circuit Bank 1 Sensor 2. |
| P0150 | Heated Oxygen Sensor circuit Bank 2 Sensor 1. |
| P0155 | Heated Oxygen Sensor Heater circuit Bank 2 Sensor 1. |
| P0156 | Heated Oxygen Sensor circuit Bank 2 Sensor 2. |
| P0161 | Heated Oxygen Sensor Heater circuit Bank 2 Sensor 2. |
| P0170 | Fuel Trim Bank 1. |
| P0173 | Fuel Trim Bank 2. |
| P0201 | Injector circuit - Cylinder 1. |
| P0202 | Injector circuit - Cylinder 2. |
| P0203 | Injector circuit - Cylinder 3. |
| P0204 | Injector circuit - Cylinder 4. |
| P0205 | Injector circuit - Cylinder 5. |
| P0206 | Injector circuit - Cylinder 6. |
| P0300 | Random Misfire Detected. |
| P0301 | Misfire Detected - Cylinder 1. |
| P0302 | Misfire Detected - Cylinder 2. |
| P0303 | Misfire Detected - Cylinder 3. |
| P0304 | Misfire Detected - Cylinder 4. |
| P0305 | Misfire Detected - Cylinder 5. |
| P0306 | Misfire Detected - Cylinder 6. |
| P0325 | Knock Sensor 1 circuit. |
| P0335 | Crankshaft Position Sensor circuit. |
| P0340 | Crankshaft Position Sensor circuit. |
| P0400 | EGR Flow. |
| P0403 | EGR Solenoid. |
| P0420 | Catalytic Efficiency below threshold (Bank 1). |
| P0421 | Warm-Up Catalyst Efficiency below threshold (Bank 1). |
| P0431 | Warm-Up Catalyst Efficiency below threshold (Bank 2). |
| P0440 | EVAP System. |
| P0442 | EVAP System Leak Detected. |
| P0443 | EVAP System Purge Control Valve circuit. |
| P0446 | EVAP System Vent Control. |
| P0450 | EVAP System Pressure Sensor. |
| P0455 | EVAP Control System Leak Detected. |
| P0500 | Vehicle Speed Sensor. |
| P0505 | Idle Control System. |
| P0510 | Closed Throttle Position Switch. |
| P0551 | Power Steering Pressure Sensor circuit Range/Performance. |
| P0700 | Transaxle Control System. |
| P0705 | Transmission Range Sensor circuit (PRNDL - Input). |
| P0710 | Transmission Fluid Temperature Sensor circuit. |
| P0715 | Input/Turbine Speed Sensor circuit. |
| P0720 | Output Speed Sensor circuit. |
| P0725 | Engine Speed Input circuit. |
| P0740 | Torque Converter Clutch System. |
| P0750 | Shift Solenoid "A". |
| P0755 | Shift Solenoid "B". |
| P0760 | Shift Solenoid "C". |
| P0765 | Shift Solenoid "D". |
| P1103 | Turbocharger Wastegate Actuator. |
| P1104 | Turbocharger Wastegate Solenoid. |
| P1105 | Fuel Pressure Solenoid. |
| P1300 | Ignition Timing Adjustment circuit. |
| P1400 | Manifold Differential Pressure Sensor circuit. |
| P1500 | Alternator FR Terminal circuit. |
| P1600 | Serial Communication Link. |
| P1715 | Pulse Generator Assembly. |
| P1750 | Solenoid Assembly. |
| P1751 | A/T Control Relay. |
| P1791 | Engine Coolant Temperature Level Input circuit. |
| P1795 | Throttle Position Input circuit to TCM. |