## Nissan Trouble Code Info

ENTERING SELF-DIAGNOSTICS

## Using (MIL)

88–94

- Turn ignition On.

- On all models except Pathfinder, Pickup and Van, proceed as follows:

  - Turn diagnostic mode selector on ECU fully clockwise and wait until inspection lamps flash. Number of flashes displayed indicates corresponding mode.

  - Note number of flashes, then immediately turn diagnostic mode selector fully counterclockwise.

- 3. On Pathfinder, Pickup and Van models, proceed as follows:

  - Turn diagnostic mode selector on ECU to On position and wait until inspection lamps flash.

  - Note number of flashes, then immediately turn diagnostic mode selector to Off position.

- On all models, when ignition is turned Off during diagnosis, in each mode, and then turned back On again after power to the ECU has dropped off completely, diagnosis will automatically return to Mode I.

- For description of modes, proceed as follows:

  - Mode I (Mixture ratio feedback control monitor A): during a closed loop condition, the green inspection lamp turns on when lean condition is present and goes off when a rich condition is present. During an open loop condition, the green inspection lamp stays off.

  - Mode II (Mixture ratio feedback control monitor B): during a closed loop condition, the red inspection lamp turns on and off simultaneously with the green inspection lamp when mixture ratio is controlled within a predetermined value. During an open loop condition, the red inspection lamp stays off. The green inspection lamp function is the same as in Mode I.

  - Mode III (Self diagnosis): in this mode the DTC is indicated by both the red and greed inspection lamp. The red inspection lamp corresponds to units of 10; the green inspection lamp corresponds to units of one.

  - Mode IV (Switches on/off diagnosis): during this mode, the inspection lamps monitor switch on/off condition of the throttle valve switch, starter switch and vehicle speed sensor.

  - Mode V (Real time diagnosis): if a malfunction is present during a driving test, the inspection lamps will display the malfunction condition immediately.

READING DIAGNOSTIC CODES

The Electronic Control Unit (ECU) monitors several engine sensors and actuators. If a problem occurs the information is stored in the ECU's memory for retrieval.
The codes are displayed by the red diode on the ECU. The codes are interpreted by the number of times and duration the diode flashes.
The red diode will flash for 0.6 of a second with 0.6 of a second delay between flashes "X" number of times, this will represent units of ten.
Then there will be a delay 0.9 of a second and the red diode will flash for 0.3 of a second with 0.3 of a second delay between flashes "X" number of times, this will represent units of one.
There will be a delay of 2.1 seconds between codes.
For example, if the red diode flashes once and then flashes twice rapidly this should be interpreted as code 12. The codes are displayed in numerical order starting with the lowest and ending with the highest.

95–99

- Turn ignition On.

- Turn diagnostic mode selector on ECU fully clockwise and wait until inspection lamps flash. Number of flashes displayed indicates corresponding mode.

- Note number of flashes, then immediately turn diagnostic mode selector fully counterclockwise.

- If ignition is turned Off during diagnosis, in each mode, and then turned back on again after power to the ECU has dropped off completely, diagnosis will automatically return to Mode I.

- For description of modes, proceed as follows:

  - (Mode I): with engine stopped, system in bulb check state. with engine running, system in malfunction warning state.

  - (Mode II): with engine stopped, system in self diagnostic results state. with engine running, system in front O2S monitor state.

## Using Consult

- Turn ignition Off.

- Connect ‘‘Consult’’ the data link connector (DLC), located behind fuse box cover.

- Turn ignition On.

- Touch ‘‘Start.’’

- Touch ‘‘Engine.’’

- Perform each diagnostic test mode according to each service procedure.

## Using Generic Scan Tool

- Turn ignition Off.

- Connect scan tool’ to its 16-pin DLC, located at lower edge of instrument panel near steering column.

- Turn ignition On.

- Enter program according to screen instructions or in operation manual.

- Perform each diagnostic test mode according to each service procedure.

### Clearing Trouble Codes

- On pre-95 models, MIL stored memory will be erased after selecting Mode III or Mode IV.

- On 95–99 models, MIL stored memory will be erased by switching from Mode II to Mode I.

- On all models, MIL stored memory will be erased if battery terminal is disconnected.

- On 95–97 models, to erase DTCs using Consult, select ERASE in the SELF DIAG RESULTS mode.

- On 98–99 models, to erase DTCs using Consult, turn Consult On, select A/T, SELF DIAG RESULTS and ERASE. Select BACK, ENGINE, SELF DIAG RESULTS and ERASE.

- On all models, to erase DTCs using generic scan tool (GST), select mode 4.

## Nissan UD Diesel

ID-TICS System

- Place diagnostic switch in the Off position.

- Carry out diagnosis under the following three conditions:

  - Place ignition switch in the On position.

  - Crank engine.

  - Start and operate engine.

- The trouble codes will be displayed by the flashing Engine Control Lamp and the Overheat Warning Lamp and the Overheat Warning Buzzer.

### Trouble Code Interpretation

MID-TICS System

The diagnostic trouble codes will be displayed by the check engine lamp. For example, a code 12 will be displayed by the check engine lamp flashing 1 time for approximately 1.2 seconds, followed by a .6 second pause, then 2 short flashes. Each trouble code will be displayed 3 times before the next trouble code is displayed.

## Clearing Trouble Codes

88-94

- Turn the ignition key on.

- Turn the diagnostic mode selector on the ECU fully clockwise and wait more than two seconds then turn the selector fully counterclockwise. The ECCS system is now in mode 2.

- Turn the diagnostic mode selector on the ECU fully clockwise and wait more than two seconds then turn the selector fully counterclockwise. The ECCS system is now in mode 1. ECU memory is now erased.

MID-TICS System

- Release diagnostic switch.

- Place ignition switch in On position.

- Check that Overheat Warning Lamp is displaying Diagnostic Display Code.

- Connect memory clear connector 2400400Z09, or a suitable jumper wire to reset switch terminals. The reset switch is located behind the electrical unit board.

- Check that Overheat Warning Lamp and Buzzer display shows Diagnostic Display Code 0–1. The System memory will be cleared in approximately 10 seconds.

- Remove connector or jumper wire from reset switch electrical connector.

- Connect electrical connector to reset switch.

- Place ignition switch in Off position.

- Place ignition switch in On position, then check Overheat Warning Lamp and Buzzer. Diagnostic Display Code 0–1 should be indicated.

## Trouble Code Info

Pre-88 Nissan

88–94 & 95 Pathfinder, Quest, Truck & 300ZX

95 Altima, Maxima, Sentra, 200SX, 240SX & 96–00 All

MID-TICS System (Diesel)

## Pre-88 Nissan Trouble Code Info

ENTERING SELF-DIAGNOSTICS

1) Turn ignition switch to ON position. Use a screwdriver to turn ECM diagnostic mode selector fully clockwise.
2) Wait for inspection lights to begin flashing. At this
time, inspection lights will flash mode options (i.e., one flash for Mode I, 2 flashes for Mode II, etc.). As soon as inspection light flashes desired mode number (3 flashes for self-diagnostic mode), turn mode selector off (fully counterclockwise)
3) If mode selector is kept in ON position (fully clockwise), mode selections will continuously cycle. This cycling will not erase memory.

image used with permission Copyright © 2002 ALLDATA LLC.

RETRIEVING CODES

NOTE: Not all trouble codes will activate MIL (CHECK ENGINE light)

Trouble codes are read using Red LED and Green LED inspection lights on side of ECM. After selecting Mode III, trouble codes corresponding to mode will start flashing. Trouble codes are indicated by number of flashes from ECM Red LED and Green LED inspection lights. Count number of flashes. Red LED indicates first digit of code, and Green LED indicates second digit of code. For example, 3 flashes of Red LED followed by 2 flashes of Green LED indicate Code 32.

SELF-DIAGNOSTIC SYSTEM

Self-diagnostic system can be operated in any of 5 modes. Modes are manually changed using screwdriver through access port of ECM. With screwdriver turned fully clockwise, inspection lights will begin to flash. Count number of flashes. First flash is Mode I, second flash is 
Mode II, etc. When desired mode has been indicated, turn screwdriver fully counterclockwise. In different modes, Red LED and Green LED perform different functions.

Mode I (Exhaust Gas Monitor)
This is normal vehicle operating mode. Green LED will indicate loop status. If Green LED is not blinking, vehicle is in open loop or a fault exists with oxygen sensor or sensor circuit. If Green LED is blinking, vehicle is in closed loop. If a malfunction occurs in Mode I, Red LED and MIL (CHECK ENGINE light) will illuminate, indicating an engine control system malfunction has occurred.

Mode II (Mixture Ratio Feedback Monitor)
In Mode II, both Red LED and Green LED are used to monitor air/fuel mixture feedback control. Green LED will function as in Mode I. In open loop, Red LED and Green LED will remain on or off. For results to be valid, vehicle must be in closed loop. In closed loop, Red LED will indicate if system is running rich (light off), lean (light on) or at ideal air/fuel ratio (blinking synchronized with Green LED)

Mode III (Self Diagnostics)
When Mode III is accessed, codes stored in ECM memory will be flashed by Green LED and Red LED on side of ECM. Red LED will flash first digit of code, while Green LED will flash second digit of code. For example: 3 flashes of Red LED followed by 5 flashes of Green LED indicate a Code 35.

Mode IV (Switch Check)
This mode is used for checking engine control systems switch status. When idle switch or starter switch is activated, Red LED will come on and go off as status changes. For vehicle speed sensor status, Green LED will remain off when vehicle speed is less than 12 MPH and
come on when vehicle speed is greater than 12 MPH.

Mode V (Test Mode)
Mode V represents a real-time diagnostic test of camshaft position sensor, ignition signal and mass airflow output signal. This mode is accessed for an in-bay running test of vehicle. Red and Green LED5 must be monitored carefully during this test, with special attention paid to number of flashes before each pause. Malfunction code will be displayed only once and will not be stored in memory. If Red LED blinks on and off evenly, a fault exists in camshaft position sensor. If Green LED flashes twice before a pause, a fault exists in airflow meter. If Green LED blinks 4 times before a pause, a fault exists in ignition signal.

## Trouble Codes

| Without Onboard Diagnostics (OBD-II) |  |
| --- | --- |
| 11 | Camshaft position sensor. |
| 12 | Mass airflow (MAF) sensor. |
| 13 | Engine coolant temperature (ECT) sensor.. |
| 14 | Vehicle speed (VSS) sensor. |
| 21 | No ignition referance. |
| 23 | Throttle Valve Switch |
| 24 | Transmission Switch |
| 31 | Engine control module (ECM).- A/C, light,fan and PS switch |
| 32 | EGR sensor.- starter switch |
| 33 | Oxygen sensor. |
| 34 | Knock (detonation) sensor (3.0 L). |
| 35 | EGR temp sensor. |
| 41 | Air temp sensor. |
| 43 | Throttle position sensor (TPS). |
| 44 | Normal Operation |
| 45 | Injector leak. |
| 51 | Injector circuit (3.0 L). |
| 55 | No malfunction recorded. |

## 88–94 & 95 Pathfinder, Quest, Truck & 300ZX

| MIL | Consult Or GST |  |
| --- | --- | --- |
| 11 | P0340 | Crank Angle Sensor/CMP Sensor |
| 12 | P0100 | Air Flow Meter/MAF Sensor |
| 13 | P0115 | ECT Sensor |
| 14 | P0500 | VSS |
| 21 | P1320 | Ignition Signal |
| 22 | P0505 | Fuel Pump |
| 23 | P0605 | Idle Switch (On/Off Diagnosis) |
| 24 | P0400 | Throttle Valve Switch |
| 25 | P0150 | ISC Valve |
| 28 | P1900 | Cooling Fan Circuit (Overheat) |
| 31 | P0325 | ECM |
| 32 | P1401 | EGR Function |
| 33 | P0402 | HO2S |
| 34 & 37 | P0130 | Knock Sensor |
| 35 | P0150 | Exhaust Gas Temperature Sensor |
| 36 | P0402 | EGRC-BPT Valve |
| 38 | P0150 | LH Bank Closed Loop (B2) |
| 41 | P0110 | IAT Sensor |
| 42 | P0180 | Fuel Temperature Sensor |
| 43 | P0120 | TP Sensor |
| 45 | — | Injector Leak |
| 47 | P1335 | CKP Sensor |
| 51 | — | Injector Circuit |
| 53 | P0130 | O2S |
| 54 | P0600 | A/T Control |
| 55 | P0000 | No Malfunction |
| 63 | P0306 | No. 6 Cylinder Misfire |
| 64 | P0305 | No. 5 Cylinder Misfire |
| 65 | P0304 | No. 4 Cylinder Misfire |
| 66 | P0303 | No. 3 Cylinder Misfire |
| 67 | P0302 | No. 2 Cylinder Misfire |
| 68 | P0301 | No. 1 Cylinder Misfire |
| 71 | P0300 | Random Misfire |
| 72 | P0420 | TWC Function RH Bank |
| 73 | P0430 | TWC Function LH Bank |
| 76 | P0170 | Fuel Injection System Function RH Bank |
| 77 | P0136 | Rear HO2S Circuit |
| 82 | P0335 | CKP Sensor |
| 84 | P1605 | A/T Diagnosis Communication Line |
| 85 | P1110 | VTC Solenoid Valve Circuit |
| 86 | P0173 | Fuel Injection System Function LH Bank |
| 87 | P0443 | Canister Control Solenoid Valve Circuit |
| 91 | P0135 | Front HO2S Heater Circuit RH Bank |
| 94 | P1550 | TCC Solenoid Valve |
| 95 | P1336 | CKP Sensor |
| 98 | P0125 | ECT Sensor |
| 101 | P0155 | Front HO2S Heater Circuit LH Bank |
| 103 | P0705 | PNP Switch Circuit |
| 105 | P1400 | EGR And EGR Canister Control Solenoid Valve Circuit |
| 108 | P0443 | Canister Purge Control Valve Circuit |
| — | P0600 | Continuous Improper A/T Control Unit Voltage To ECM |

## 95 Altima, Maxima, Sentra, 200SX, 240SX & 96–00 All

[ 95-00 Altima ] [ FRONTIER & XTERRA 2.4L ] [ FRONTIER & XTERRA 3.3L ] 
[ 95-00 Maxima ] [ 95-99 SENTRA & 200SX w/GA16DE ] 
[ 95-99 SENTRA & 200SX w/SR20DE ] [ 95-98 240SX ] [ 96 300ZX ] [ 96-00 Pathfinder ] [ 96-97 Quest ] [ 98 Quest ] [99-00 Quest ] [ 96-97 Truck ]

| 95–00 ALTIMA |  |
| --- | --- |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S |
| P0135 | Front HO2S Heater |
| P0136–P0140 | Rear HO2S |
| P0141 | Rear HO2S Heater |
| P0170 | Fuel Injection System Function |
| P0171 | Fuel Injection System Lean |
| P0172 | Fuel Injection System Rich |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0304 | Misfire, Cylinders 1–4 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor, Detectable & Non-Detectable Circuit |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve |
| P0420 | TWC Function |
| P0440 | EVAP System Small Leak |
| P0443 | EVAP Canister Purge Volume Control Solenoid Valve |
| P0446 | EVAP Canister Vent Control Valve Circuit |
| P0450 | EVAP System Pressure Sensor |
| P0455 | EVAP Gross Leak |
| P0460 | Fuel Level Sensor (Slosh) |
| P0461 | Fuel Level Sensor |
| P0464 | Fuel Level Sensor/Circuit |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP |
| P0600 | A/T Communication Line |
| P0605 | ECM/ECCS Module |
| P0705 | PNP/Inhibitor Switch |
| P0710 | ATF Temperature Sensor |
| P0720 | VSS |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Signal |
| P0732 | A/T 2nd Signal |
| P0733 | A/T 3rd Signal |
| P0734 | A/T 4th Or TCC Signal |
| P0740 | TCC Solenoid Valve |
| P0744 | TCC Solenoid & Valve Function |
| P0745 | Line Pressure Solenoid Valve |
| P0750 | Shift Solenoid Valve ‘‘A’’ |
| P0755 | Shift Solenoid Valve ‘‘B’’ |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1126 | Thermostat Function |
| P1148 | Closed Loop Control |
| P1320 | Ignition Signal |
| P1336 | CKP |
| P1400 | EGR & Canister Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP System Small Leak |
| P1441 | EVAP Vacuum Sensor Leak |
| P1444 | Purge Volume Control Valve |
| P1446 | Vacuum Cut Valve |
| P1447 | EVAP Purge Flow Monitor |
| P1448 | Vacuum Cut Valve |
| P1464 | Fuel Level Sensor/Circuit |
| P1490 | Vacuum Cut/Bypass Valve |
| P1491 | Vacuum Cut Valve Bypass Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor |
| P1706 | Park/Neutral Position Switch |
| P1760 | Overrun Clutch Solenoid Valve |
| P1900 | Cooling Fan |
| FRONTIER & XTERRA 2.4L |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S |
| P0135 | Front HO2S Heater |
| P0137–P0140 | Rear HO2S |
| P0141 | Rear HO2S Heater |
| P0171 | Fuel Injection System Lean |
| P0172 | Fuel Injection System Rich |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0304 | Misfire, Cylinders 1–4 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor, Detectable & Non-Detectable Circuit |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve |
| P0420 | TWC Function |
| P0440 | EVAP System Small Leak |
| P0443 | EVAP Canister Purge Volume Control Solenoid Valve |
| P0446 | EVAP Canister Vent Control Valve Circuit |
| P0450 | EVAP Control System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP |
| P0605 | ECM/ECCS Module |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1148 | Closed Loop Control |
| P1320 | Ignition Signal |
| P1336 | CKP |
| P1400 | EGR & Canister Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP System Small Leak |
| P1444 | Purge Volume Control Valve |
| P1446 | Vacuum Cut Valve |
| P1447 | EVAP Purge Flow Monitor |
| P1448 | Vacuum Cut Valve |
| P1490 | Vacuum Cut/Bypass Valve |
| P1491 | Vacuum Cut Valve Bypass Valve |
| P1706 | Park/Neutral Position Switch |
| P1775 | TCC Solenoid Valve Circuit |
| P1776 | TCC Clutch Solenoid Valve |
| FRONTIER & XTERRA 3.3L |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S RH Bank |
| P0135 | Front HO2S Heater RH Bank |
| P0136–P0140 | Rear HO2S RH Bank |
| P0141 | Rear HO2S Heater RH Bank |
| P0150–P0154 | Front HO2S LH Bank |
| P0155 | Front HO2S Heater LH Bank |
| P0157–P0161 | Rear HO2S LH Bank |
| P0171 | Fuel Injection System Lean RH Bank |
| P0172 | Fuel Injection System Rich RH Bank |
| P0174 | Fuel Injection System Lean LH Bank |
| P0175 | Fuel Injection System Rich LH Bank |
| P0180 | Tank Fuel Temperature Sensor |
| P0217 | Engine Over Temp |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Misfire Cylinders 1–6 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve Function |
| P0420 | TWC Function |
| P0430 | TWC Function |
| P0440 | EVAP System Small Leak |
| P0443 | EVAP Canister Purge Control Valve & Solenoid Valve |
| P0446 | EVAP Canister Vent Control |
| P0450 | EVAP System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP Switch |
| P0600 | A/T Control & Communication Line |
| P0705 | Park Neutral Position Switch/Circuit |
| P0710 | ATF Temp Sensor/Circuit |
| P0720 | Vehicle Speed Sensor/Circuit |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1ST Gear Function |
| P0732 | A/T 2ND Gear Function |
| P0733 | A/T 3RD Gear Function |
| P0734 | A/T 4TH Gear Function |
| P0740 | TCC Solenoid/Circuit |
| P0744 | A/T TCC Switch/Valve Function |
| P0745 | Low Pressure Solenoid/Circuit |
| P0750 | Shift Solenoid A/Circuit |
| P0755 | Shift Solenoid B/Circuit |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1148 | Closed Loop B1 |
| P1168 | Closed Loop B2 |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGR Valve & EVAP Canister Purge Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP System Small Leak |
| P1444 | Purge Volume Control Valve |
| P1446 | EVAP Canister Vent Control Valve |
| P1447 | EVAP System Purge Flow Monitoring |
| P1448 | Vacuum Cut Valve |
| P1490 | Vacuum Cut Valve Bypass Valve |
| P1491 | Vacuum Cut Valve Bypass Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | Throttle Position Sensor/Circuit A/T |
| P1706 | PNP Switch/Circuit |
| P1760 | Over/Run Clutch Solenoid/Circuit |
| 95–00 MAXIMA |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S Bank 1 |
| P0135 | Front HO2S Heater Bank 1 |
| P0136–P0140 | Rear HO2S |
| P0141 | Rear HO2S Heater |
| P0150–P0154 | Front HO2S Bank 2 |
| P0155 | Front HO2S Heater Bank 2 |
| P0157-P0160 | Rear HO2S Bank 2 |
| P0161 | Rear HO2S Heater Bank 2 |
| P0170 | Fuel Injection System Function |
| P0171 | Fuel System Lean Bank 1 |
| P0172 | Fuel System Rich Bank 1 |
| P0173 | Fuel Injection System Function |
| P0174 | Fuel System Lean Bank 2 |
| P0175 | Fuel System Rich Bank 2 |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Cylinder Misfire |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve |
| P0403 | EGR Volume Control Valve Circuit |
| P0420 | TWC Function RH Bank |
| P0430 | TWC Function LH Bank |
| P0440 | EVAP Small Leak |
| P0443 | Canister Purge Control/Solenoid Valve |
| P0446 | Vent Control Valve |
| P0450 | EVAP System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP Switch |
| P0600 | A/T Communication Line |
| P0605 | ECM |
| P0705 | PNP/Inhibitor Switch |
| P0710 | Fluid Temperature Sensor |
| P0720 | VSS |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Signal |
| P0732 | A/T 2nd Signal |
| P0733 | A/T 3rd Signal |
| P0734 | A/T 4th Signal Or TCC |
| P0740 | TCC Solenoid Valve |
| P0744 | A/T TCC Signal |
| P0745 | Line Pressure Solenoid Valve |
| P0750 | Shift Solenoid Valve ‘‘A’’ |
| P0755 | Shift Solenoid Valve ‘‘B’’ |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1130 | Swirl Control Valve Control Solenoid Valve |
| P1148 | Closed Loop RH Bank |
| P1165 | Swirl Control Valve Control Vacuum Check Valve |
| P1168 | Closed Loop LH Bank |
| P1220 | FPCM |
| P1320 | Ignition Signal Primary |
| P1335 | CKP Sensor (REF) |
| P1336 | CKP Sensor |
| P1400 | EGR Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP Small Leak |
| P1441 | VC/V Bypass Valve |
| P1444 | Purge Volume Control Valve |
| P1445 | Purge Volume Control Valve |
| P1446 | Vent Control Valve |
| P1447 | EVAP Purge Flow Monitor |
| P1448 | Vent Control Valve |
| P1490 | Vacuum Cut Valve Bypass Valve |
| P1491 | Vacuum Cut Bypass Valve |
| P1492 | Purge Control Valve Solenoid Valve |
| P1493 | Purge Control Valve & Solenoid Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor |
| P1706 | Park/Neutral Position Switch Circuit |
| P1760 | Overrun Clutch Solenoid Valve |
| 95–99 SENTRA & 200SX w/GA16DE |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S |
| P0135 | Front HO2S Heater |
| P0136–P0140 | Rear HO2S |
| P0141 | Rear HO2S Heater |
| P0170 | Fuel Injection System Function |
| P0171 | Fuel Injection System Lean |
| P0172 | Fuel Injection System Rich |
| P0180 | Fuel Tank Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0304 | Misfire Cylinders 1–4 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor, Detectable & Non-Detectable Circuit |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve |
| P0420 | TWC Function |
| P0440 | EVAP System Small Leak |
| P0443 | EVAP Canister Purge Control Valve |
| P0446 | EVAP Canister Vent Control Valve |
| P0450 | EVAP Control System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP Switch |
| P0605 | ECM |
| P0705 | PNP Switch |
| P1105 | MAP/BAR Switch Solenoid |
| P1110 | Intake Valve Timing Control Solenoid |
| P1148 | Closed Loop |
| P1320 | Ignition Signal Primary |
| P1336 | CKP Sensor |
| P1400 | EGR Valve & Canister Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP System Small Leak |
| P1441 | Vacuum Cut Valve Bypass Valve |
| P1444 | Purge Volume Control Valve |
| P1445 | EVAP Canister Purge Volume Control Valve |
| P1446 | Vent Control Valve |
| P1447 | EVAP Control System Purge Flow Monitoring |
| P1448 | Vent Control Valve |
| P1490 | Vent Control Bypass Valve |
| P1491 | Vent Control Cut Valve Bypass Valve |
| P1492 | Purge Control Solenoid Valve |
| P1493 | Purge Control Valve & Solenoid Valve |
| P1550 | TCC Solenoid Valve |
| P1706 | PNP Switch |
| P1775 | TCC Solenoid Valve |
| P1776 | TCC Solenoid Valve |
| P1900 | Cooling Fan |
| 95–99 SENTRA & 200SX w/SR20DE |  |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130 | Front HO2S |
| P0131 | Front HO2S Lean Shift Monitoring |
| P0132 | Front HO2S Rich Shift Monitoring |
| P0133 | Front HO2S Response Monitoring |
| P0134 | Front HO2S High Voltage |
| P0135 | Front HO2S Heater |
| P0136 | Rear HO2S |
| P0137 | Rear HO2S Min. Voltage Monitoring |
| P0138 | Rear HO2S Max. Voltage Monitoring |
| P0139 | Rear HO2S Response Monitoring |
| P0140 | Rear HO2S High Voltage |
| P0141 | Rear HO2S Heater |
| P0170 | Fuel Injection System Function |
| P0171 | Fuel Injection System Lean |
| P0172 | Fuel Injection System Rich |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0304 | Misfire Cylinders 1–4 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor, Detectable & Non-Detectable Circuits |
| P0400 | EGR Function (Close) |
| P0402 | EGRC-BPT Valve |
| P0420 | TWC Function |
| P0440 | EVAP Control System Small Leak |
| P0443 | EVAP System Canister Purge Control Valve |
| P0446 | EVAP Canister Vent Control Valve |
| P0450 | EVAP Control System Pressure Sensor |
| P0500 | VSS |
| P0505 | Idle Air Control Valve–Auxiliary Air Control Valve |
| P0510 | Closed Throttle Position Switch |
| P0605 | ECM |
| P0705 | PNP/Inhibitor Switch |
| P0710 | Transaxle Fluid Temperature Sensor |
| P0720 | VSS |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Signal |
| P0732 | A/T 2nd Signal |
| P0733 | A/T 3rd Signal |
| P0734 | A/T 4th Signal Or TCC |
| P0740 | TCC Solenoid Valve |
| P0744 | A/T TCC S/V Function |
| P0745 | Line Pressure Solenoid Valve |
| P0750 | Shift Solenoid Valve ‘‘A’’ |
| P0755 | Shift Solenoid Valve ‘‘B’’ |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1110 | Intake Valve Timing Control |
| P1148 | Closed Loop Control |
| P1220 | Fuel Pump Control Module Circuit |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGR Valve & Canister Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System Function (Open) |
| P1440 | EVAP Small Leak |
| P1441 | Vacuum Cut Valve Bypass Valve |
| P1444 | EVAP Purge Volume Control Valve |
| P1445 | EVAP Canister Purge Volume Control Valve |
| P1446 | EVAP Canister Vent Control Valve (Close) |
| P1447 | EVAP Control System Purge Flow Monitoring |
| P1448 | EVAP Vent Control Valve (Open) |
| P1490 | Vacuum Cut Valve Bypass Valve Circuit |
| P1491 | Vacuum Cut Valve Bypass Vale |
| P1492 | EVAP Canister Purge Control Valve/Solenoid Valve Circuit |
| P1493 | EVAP Canister Purge Control Valve/Solenoid Valve |
| P1550 | TCC Solenoid Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor |
| P1706 | Park/Neutral Position Switch |
| P1760 | Overrun Clutch Solenoid Valve |
| P1900 | Cooling Fan |
| 95–98 240SX |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S |
| P0135 | Front HO2S Heater |
| P0136–P0140 | Rear HO2S |
| P0141 | Rear O2S Heater |
| P0170 | Fuel Injection System Function |
| P0171 | Fuel System Lean |
| P0172 | Fuel System Rich |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0304 | Misfire Cylinders 1–4 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor, Detectable & Non-Detectable Circuit |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve |
| P0420 | TWC Function |
| P0440 | EVAP Small Leak |
| P0443 | Purge Control Valve |
| P0446 | Vent Control Valve |
| P0450 | EVAP System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP Switch |
| P0600 | A/T Control & Communication Line |
| P0605 | ECM |
| P0705 | PNP/Inhibitor Switch |
| P0710 | Fluid Temperature Sensor |
| P0720 | VSS |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Signal |
| P0732 | A/T 2nd Signal |
| P0733 | A/T 3rd Signal |
| P0734 | A/T 4th Signal |
| P0740 | TCC Solenoid |
| P0744 | A/T TCC Signal |
| P0745 | Line Pressure Solenoid Valve |
| P0750 | Shift Solenoid Valve ‘‘A’’ |
| P0755 | Shift Solenoid Valve ‘‘B’’ |
| P1105 | MAP/BARO Switch/Solenoid Valve |
| P1148 | Closed Loop |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGR & Canister Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1440 | EVAP Small Leak |
| P1441 | Vacuum Cut Valve Bypass Valve |
| P1444 | Purge Volume Control Valve |
| P1445 | Purge Volume Control Valve |
| P1446 | Vent Control Valve |
| P1447 | EVAP Purge Flow Monitor |
| P1448 | Vent Control Valve |
| P1490 | Vacuum Cut Bypass Valve |
| P1491 | Vacuum Cut/Bypass Valve |
| P1492 | Purge Control Valve Solenoid/Valve |
| P1493 | Purge Control Valve & Solenoid/Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor |
| P1760 | Overrun Clutch Solenoid Valve |
| 96 300ZX |  |
| P0100 | MAF Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130 | Front HO2S RH Bank |
| P0135 | Front HO2S Heater RH Bank |
| P0136 | Rear HO2S RH Bank |
| P0141 | Rear HO2S Heater RH Bank |
| P0150 | Front HO2S LH Bank |
| P0155 | Front HO2S Heater LH Bank |
| P0156 | Rear HO2S LH Bank |
| P0161 | HO2S Heater LH Bank |
| P0171 | Fuel Injection System Lean RH Bank |
| P0172 | Fuel Injection System Rich RH Bank |
| P0174 | Fuel Injection System Lean LH Bank |
| P0175 | Fuel Injection System Rich LH Bank |
| P0180 | Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Misfire Cylinders 1–6 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0420 | TWC Function |
| P0430 | TWC Function |
| P0443 | EVAP Canister Purge Control Solenoid Valve |
| P0500 | VSS |
| P0505 | IACV-AAC Valve |
| P0600 | A/T Control & Communication Line |
| P0605 | ECM-ECCS Control Module |
| P0705 | PNP Switch |
| P1150 | Wastegate Valve Control Solenoid Valve |
| P1155 | Wastegate Valve Control Solenoid Valve |
| P1160 | Turbocharger Boost Sensor |
| P1220 | Fuel Pump Control Module |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGRC Solenoid Valve |
| P1401 | Temperature Sensor |
| P1443 | Canister Control Vacuum Check Switch |
| P1605 | A/T Diagnosis Communication Line |
| P1900 | Cooling Fan |
| 96–00 PATHFINDER |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S RH Bank |
| P0135 | Front HO2S Heater RH Bank |
| P0136–P0140 | Rear HO2S RH Bank |
| P0141 | Rear HO2S Heater RH Bank |
| P0150–P0154 | Front HO2S LH Bank |
| P0155 | Front HO2S Heater LH Bank |
| P0156–P0160 | Rear HO2S LH Bank |
| P0161 | Rear HO2S Heater LH Bank |
| P0171 | Fuel Injection System Lean RH Bank |
| P0172 | Fuel Injection System Rich RH Bank |
| P0174 | Fuel Injection System Lean LH Bank |
| P0175 | Fuel Injection System Rich LH Bank |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Misfire Cylinders 1–6 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve Function |
| P0420 | TWC Function |
| P0430 | TWC Function |
| P0440 | EVAP System Small Leak |
| P0443 | EVAP Canister Purge Control Valve & Solenoid Valve |
| P0446 | EVAP Canister Vent Control |
| P0450 | EVAP System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP Switch |
| P0600 | A/T Control & Communication Line |
| P0605 | ECM-ECCS Control Module |
| P0705 | PNP/Inhibitor Switch |
| P0710 | Fluid Temperature Sensor |
| P0720 | VSS |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Signal |
| P0732 | A/T 2nd Signal |
| P0733 | A/T 3rd Signal |
| P0734 | A/T 4th Signal Or TCC |
| P0740 | TCC Solenoid Valve |
| P0744 | TCC Signal |
| P0745 | Line Pressure Solenoid Valve |
| P0750 | Shift Solenoid Valve ‘‘A’’ |
| P0755 | Shift Solenoid Valve ‘‘B’’ |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1148 | Closed Loop RH Bank |
| P1168 | Closed Loop LH Bank |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGR Valve & EVAP Canister Purge Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP System Small Leak |
| P1441 | Vacuum Cut Valve Bypass Valve |
| P1444 | Purge Volume Control Valve |
| P1445 | Purge Volume Control Valve |
| P1447 | EVAP System Purge Flow Monitoring |
| P1448 | Vacuum Cut Valve |
| P1490 | Vacuum Cut Valve Bypass Valve |
| P1491 | Vacuum Cut Valve Bypass Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor A/T |
| P1706 | PNP Switch/Circuit |
| P1760 | Overrun Clutch Solenoid Valve |
| 96–97 QUEST |  |
| P0100 | MAF Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130 | Front HO2S |
| P0135 | Front HO2S Heater |
| P0136 | Rear HO2S Sensor |
| P0141 | Rear HO2S Heater |
| P0171 | Fuel Injection System Lean |
| P0172 | Fuel Injection System Rich |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Misfire Cylinders 1–6 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Function |
| P0420 | TWC Function |
| P0500 | VSS |
| P0505 | IACV-AAC Valve |
| P0600 | A/T Control & Communication Line |
| P0605 | ECM-ECCS Control Module |
| P0705 | PNP-Inhibitor Switch |
| P0710 | Fluid Temperature Sensor |
| P0720 | VSS |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Signal |
| P0732 | A/T 2nd Signal |
| P0733 | A/T 3rd Signal |
| P0734 | A/T 4th Signal |
| P0740 | TCC Solenoid Valve |
| P0744 | TCC Signal |
| P0745 | Line Pressure Solenoid Valve |
| P0750 | Shift Solenoid Valve ‘‘A’’ |
| P0755 | Shift Solenoid Valve ‘‘B’’ |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGR Valve & EVAP Canister Purge Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor |
| P1760 | Overrun Clutch Solenoid Valve |
| P1900 | Cooling Fan |
| 98 QUEST |  |
| P0000 | No Self Diagnostic Codes Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130 | Front HO2S Circuit |
| P0131 | Front HO2S Lean Shift Monitoring |
| P0132 | Front HO2S Rich Shift Monitoring |
| P0133 | Front HO2S Response Monitoring |
| P0134 | Front HO2S High Voltage |
| P0135 | Front HO2S Heater |
| P0137 | Rear HO2S Minimum Voltage Monitoring |
| P0138 | Rear HO2S Maximum Voltage Monitoring |
| P0139 | Rear HO2S Response Monitoring |
| P0140 | Rear HO2S High Voltage |
| P0141 | Rear HO2S Heater |
| P0171 | Fuel Injection System Lean |
| P0172 | Fuel Injection System Rich |
| P0180 | Tank Fuel Temperature Sensor |
| P0300–P0306 | Misfire Cylinders 1–6/Multiple Cylinder Misfire |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Function |
| P0420 | TWC Function |
| P0440 | EVAP Control System Small Leak |
| P0443 | EVAP Canister Purge Volume Control Valve |
| P0446 | EVAP Vent Control Valve Circuit |
| P0450 | EVAP Control System Pressure Sensor |
| P0500 | VSS |
| P0505 | IACV-AAC Valve |
| P0510 | Closed Throttle Position Switch |
| P0600 | A/T Control & Communication Line |
| P0605 | ECM-ECCS Control Module |
| P0705 | Park/Neutral Position Switch Circuit |
| P0710 | Automatic Transmission Fluid Temperature Sensor Circuit |
| P0720 | VSS Circuit |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1st Gear Function |
| P0732 | A/T 2nd Gear Function |
| P0733 | A/T 3rd Gear Function |
| P0734 | A/T 4th Gear Function |
| P0740 | TCC Solenoid Circuit |
| P0744 | A/T TCC Solenoid Valve Duty |
| P0745 | Line Pressure Solenoid Valve Circuit |
| P0750 | Shift Solenoid A Circuit |
| P0755 | Shift Solenoid B Circuit |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1148 | Closed Loop Control |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGRC Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR Function Open |
| P1440 | EVAP Control System Small Leak |
| P1444 | EVAP Canister Purge Control Valve |
| P1446 | EVAP Canister Vent Control Valve Closed |
| P1447 | EVAP Control System Purge Flow Monitoring |
| P1448 | EVAP Canister Vent Control Valve Open |
| P1490 | Vacuum Cut Valve Bypass Valve |
| P1491 | Vacuum Cut Valve Bypass Valve |
| P1492 | EVAP Canister Purge Control Valve/Solenoid Valve Circuit |
| P1493 | EVAP Canister Purge Control Valve/Solenoid Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | TP Sensor Circuit A/T |
| P1706 | Park/Neutral Position Switch |
| P1760 | Overrun Clutch Solenoid Circuit |
| 99–00 QUEST |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130–P0134 | Front HO2S RH Bank |
| P0135 | Front HO2S Heater RH Bank |
| P0136–P0140 | Rear HO2S RH Bank |
| P0141 | Rear HO2S Heater RH Bank |
| P0171 | Fuel Injection System Lean RH Bank |
| P0172 | Fuel Injection System Rich RH Bank |
| P0174 | Fuel Injection System Lean LH Bank |
| P0175 | Fuel Injection System Rich LH Bank |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Misfire Cylinders 1–6 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve Function |
| P0420 | TWC Function |
| P0430 | TWC Function |
| P0440 | EVAP System Small Leak |
| P0443 | EVAP Canister Purge Control Valve & Solenoid Valve |
| P0446 | EVAP Canister Vent Control |
| P0450 | EVAP System Pressure Sensor |
| P0500 | VSS |
| P0505 | IAC-AAC Valve |
| P0510 | CTP Switch |
| P0600 | A/T Control & Communication Line |
| P0705 | Park Neutral Position Switch Circuit |
| P0710 | ATF Temp Sensor/Circuit |
| P0720 | Vehicle Speed Sensor Circuit A/T |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1ST Gear Function |
| P0732 | A/T 2ND Gear Function |
| P0733 | A/T 3RD Gear Function |
| P0734 | A/T 4TH Gear Function |
| P0740 | TCC Solenoid Valve |
| P0744 | A/T TCC Switch/Valve Function |
| P0745 | Low Pressure Solenoid/Circuit |
| P0750 | Shift Solenoid A/Circuit |
| P0755 | Shift Solenoid B/Circuit |
| P1105 | MAP/BARO Switch Solenoid Valve |
| P1148 | Closed Loop |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGR Valve & EVAP Canister Purge Control Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP System Small Leak |
| P1444 | Purge Volume Control Valve |
| P1446 | EVAP Canister Vent Control Valve |
| P1447 | EVAP System Purge Flow Monitoring |
| P1448 | Vacuum Cut Valve |
| P1490 | Vacuum Cut Valve Bypass Valve |
| P1491 | Vacuum Cut Valve Bypass Valve |
| P1605 | A/T Diagnosis Communication Line |
| P1705 | Throttle Position Sensor/Circuit A/T |
| P1706 | PNP Switch/Circuit |
| P1760 | Over/Run Clutch Solenoid/Circuit |
| 96–97 TRUCK |  |
| P0000 | No Self Diagnostic Failure Indicated |
| P0100 | MAF Sensor |
| P0105 | Absolute Pressure Sensor |
| P0110 | IAT Sensor |
| P0115 | ECT Sensor |
| P0120 | TP Sensor |
| P0125 | ECT Sensor |
| P0130 | Front O2S B1 |
| P0131 | Front O2S B1 |
| P0132 | Front O2S B1 |
| P0133 | Front O2S B1 |
| P0134 | Front O2S B1 |
| P0135 | Front O2S Heater |
| P0136 | Rear O2S B1 |
| P0137 | Rear O2S B1 |
| P0138 | Rear O2S B1 |
| P0139 | Rear O2S B1 |
| P0140 | Rear O2S B1 |
| P0141 | Rear O2S Heater |
| P0150 | Front O2S B2 |
| P0151 | Front O2S B2 |
| P0152 | Front O2S B2 |
| P0153 | Front O2S B2 |
| P0154 | Front O2S B2 |
| P0155 | Front O2S Heater B2 |
| P0157 | Rear O2S B2 |
| P0158 | Rear O2S B2 |
| P0159 | Rear O2S B2 |
| P0160 | Rear O2S B2 |
| P0161 | Rear O2S Heater B2 |
| P0171 | Fuel Injection System Lean Bank 1 |
| P0172 | Fuel Injection System Rich Bank 1 |
| P0174 | Fuel Injection System Lean Bank 2 |
| P0175 | Fuel Injection System Rich Bank 2 |
| P0180 | Tank Fuel Temperature Sensor |
| P0300 | Multiple Cylinder Misfire |
| P0301–P0306 | Misfire Cylinders 1–6 |
| P0325 | Knock Sensor |
| P0335 | CKP Sensor |
| P0340 | CMP Sensor |
| P0400 | EGR Function |
| P0402 | EGRC-BPT Valve Function |
| P0420 | TWC System B1 |
| P0430 | TWC System B2 |
| P0440 | EVAP Control System Small Leak |
| P0443 | EVAP Canister Purge Control Valve/Solenoid Valve |
| P0446 | EVAP Canister Vent Control Valve |
| P0450 | EVAP System Control Pressure Sensor |
| P0455 | EVAP System Gross Leak |
| P0500 | VSS |
| P0505 | IACV-AAC Valve |
| P0510 | CTP Switch |
| P0600 | A/T Communication Line |
| P0605 | ECM-ECCS Control Module |
| P0705 | PNP Switch |
| P0710 | ATF Temp Sensor/Circuit |
| P0720 | Vehicle Speed Sensor/Circuit |
| P0725 | Engine Speed Signal |
| P0731 | A/T 1ST Gear Function |
| P0732 | A/T 2ND Gear Function |
| P0733 | A/T 3RD Gear Function |
| P0734 | A/T 4TH Gear Function |
| P0740 | TCC Solenoid/Circuit |
| P0744 | A/T TCC Switch/Valve Function |
| P0745 | Low Pressure Solenoid/Circuit |
| P0750 | Shift Solenoid A/Circuit |
| P0755 | Shift Solenoid B/Circuit |
| P1105 | MAP/BARO Pressure Switch Solenoid Valve |
| P1130 | Swirl Control Valve Control Solenoid Valve |
| P1148 | Closed Loop B1 |
| P1165 | Swirl Control Valve Control Vacuum Switch |
| P1168 | Closed Loop B2 |
| P1320 | Ignition Signal |
| P1336 | CKP Sensor |
| P1400 | EGRC Solenoid Valve |
| P1401 | EGR Temperature Sensor |
| P1402 | EGR System |
| P1440 | EVAP Small Leak |
| P1441 | Vacuum Cut Valve Bypass Valve |
| P1444 | Purg Volume Control Valve |
| P1445 | EVAP Canister Purge Volume Control Valve |
| P1446 | Vent Control Valve |
| P1447 | EVAP Control System Purge Flow Monitoring |
| P1448 | Vent Control Valve |
| P1490 | Vent Control Valve Bypass Valve |
| P1491 | Vent Control Cut Valve Bypass Valve |
| P1550 | TCC Solenoid Valve |
| P1605 | A/T Diagnostic Communication Line |
| P1705 | Throttle Position Sensor Circuit A/T |
| P1706 | Park Neutral Position Switch/Circuit |
| P1760 | Over/Run Clutch Solenoid/Circuit |