Toyota Trouble Code Info

96 & later are OBD2 systems and require a scantool
Note: We are finding that some 94 & 95 Toyota vehicles are 
OBD2 compliant and require a scan tool. 
Check your emissions decal under the hood to confirm 
if you have an OBD2 compliant vehicle.

#### RETRIEVING CODES for vehicles before 96

NOTE: Not all trouble codes will activate MIL (CHECK ENGINE light)

- Before proceeding, make sure the:

  - CHECK ENGINE light circuit is functional. It should be ON when the ignition switch is ON with the engine stopped.

  - Battery voltage is above 11 volts.

  - Throttle valve is fully closed (Throttle Position Sensor IDL points closed).

  - Accessory switches (A/C, etc.) are OFF.

  - Engine is at normal operating temperature (if possible).

- Turn ignition switch to ON position.

  - Do not start the engine.

  - Place a jumper wire across TEl and El terminals in engine check connector.

- Count number of flashes from CHECK ENGINE light.

  - If system is operating normally (with no detected faults), the CHECK ENGINE light will blink continuously
and evenly about 2 times a second.

- Otherwise, the light will blink a number of times equal to the trouble code as follows:

  - The light blinks only (.5 second ON, .5 second OFF) when indicating a number.

  - The light will be OFF for 1.5 seconds between the first digit and the second digit of the code.

  - If more than one code is stored, the light will be OFF for 2.5 seconds before the next code is displayed.

  - Once all code(s) have been displayed, the light will be OFF for 4.5 seconds and then the whole sequence will repeat.

- The diagnostic code series will continue to repeat as long as the check connecter terminals TEl and El are connected.

- When finished, remove the jumper wire.

- After repairing the malfunction, clear the codes from the ECUs memory.

### Clearing Trouble Codes

All trouble codes must be purged from the ECUs memory after repairing the faults. Otherwise, code(s) would remain in the ECUs memory indefinitely and would appear along with a new code in the event of a future problem.

- Remove the EFI fuse (15A) for 30 seconds with the ignition switch OFF to clear any codes. It may take longer than 30 seconds, depending on ambient temperature (the lower the temperature, the longer the fuse must be left out).

- Trouble codes can also be cleared by disconnecting the vehicle battery. However, other memory functions (clock, radio, etc.), will need to be reset.

- After the codes are cleared, road test the vehicle (as applicable for the fault) and recheck if any codes reset. If so, it indicates that the trouble area has not been repaired correctly.

## Trouble Codes

| OBD1 Codes |  |
| --- | --- |
| 1 | Normal Condition. |
| 2 | Air Flow Meter signal. |
| 3 | Ignition signal. |
| 4 | Engine Coolant Temperature Sensor signal. |
| 5 | Oxygen Sensor. |
| 6 | RPM signal (Crank Angle Pulse). |
| 7 | Throttle Position Sensor signal. |
| 8 | Intake Air Temperature Sensor signal. |
| 9 | Vehicle Speed Sensor signal. |
| 10 | Starter signal. |
| 11 | Switch signal. |
| 11 | ECU/ECM. |
| 12 | Knock Control Sensor signal. |
| 12 | RPM signal. |
| 13 | Knock Control CPU (ECM). |
| 13 | RPM signal. |
| 14 | Turbocharger Pressure. |
| 14 | Ignition signal. |
| 21 | Oxygen Sensor. |
| 22 | Engine Coolant Temperature Sensor signal. |
| 23 | Intake Air Temperature Sensor signal. |
| 24 | Intake Air Temperature Sensor signal. |
| 25 | Air-Fuel Ratio Lean. |
| 26 | Air-Fuel Ratio Rich. |
| 27 | Sub Oxygen Sensor signal. |
| 28 | No. 2 Oxygen Sensor signal. |
| 31 | Air Flow Meter signal (Vacuum Sensor signal). |
| 32 | Air Flow Meter signal. |
| 34 | Turbocharger Pressure signal. |
| 35 | Turbocharger Pressure Sensor signal. |
| 35 | HAC Sensor signal. |
| 41 | Throttle Position Sensor signal. |
| 42 | Vehicle Speed Sensor signal. |
| 43 | Starter signal. |
| 51 | Switch signal. |
| 52 | Knock Sensor signal. |
| 53 | Knock Sensor signal. |
| 54 | Inter-cooler ECM signal. |
| 71 | EGR System. |
| 72 | Fuel Cut Solenoid signal. |
| 78 | Fuel Pump Control signal. |
| 81 | TCM Communication. |
| 83 | TCM Communication. |
| 84 | TCM Communication. |
| 85 | TCM Communication. |

| ### OBD II |  |
| --- | --- |
| P0100 | Mass Air Flow Circuit. |
| P0101 | Mass Air Flow Circuit Range/Performance. |
| P0105 | Manifold Absolute Pressure Circuit. |
| P0106 | MAP/BARO Circuit Range/Performance. |
| P0110 | Intake Air Temperature Circuit. |
| P0115 | Engine Coolant Temperature Circuit. |
| P0116 | Engine Coolant Temperature Range/Performance. |
| P0120 | Throttle Pedal Sensor Switch "A" Circuit. |
| P0121 | Throttle Pedal Sensor Switch "A" Circuit Range/Performance. |
| P0125 | Insufficient Coolant Temperature for Closed Loop Fuel Control. |
| P0130 | Heated Oxygen Sensor Circuit - Bank 1 Sensor 1. |
| P0133 | Heated Oxygen Sensor Circuit Slow Response Bank 1 Sensor 1. |
| P0135 | Heated Oxygen Sensor Circuit - Bank 1 Sensor 1. |
| P0136 | Heated Oxygen Sensor Circuit - Bank 1 Sensor 2. |
| P0141 | Heated Oxygen Sensor Circuit - Bank 1 Sensor 2. |
| P0150 | Heated Oxygen Sensor Circuit - Bank 2 Sensor 1. |
| P0153 | Heated Oxygen Sensor Circuit Slow Response Bank 2 Sensor 1. |
| P0155 | Heated Oxygen Sensor Circuit - Bank 2 Sensor 1. |
| P0156 | Heated Oxygen Sensor Circuit - Bank 2 Sensor 2. |
| P0161 | Heated Oxygen Sensor Circuit - Bank 2 Sensor 2. |
| P0170 | Fuel Trim malfunction. |
| P0171 | System too Lean. |
| P0172 | System too Rich. |
| P0201 | Injector Circuit - Cyl. 1. |
| P0202 | Injector Circuit - Cyl. 2. |
| P0203 | Injector Circuit - Cyl. 3. |
| P0204 | Injector Circuit - Cyl. 4. |
| P0205 | Injector Circuit - Cyl. 5. |
| P0206 | Injector Circuit - Cyl. 6. |
| P0300 | Random Misfire. |
| P0301 | Misfire Detected - Cyl. 1. |
| P0302 | Misfire Detected - Cyl. 2. |
| P0303 | Misfire Detected - Cyl. 3. |
| P0304 | Misfire Detected - Cyl. 4. |
| P0305 | Misfire Detected - Cyl. 5. |
| P0306 | Misfire Detected - Cyl. 6. |
| P0325 | Knock Sensor 1 Circuit. |
| P0330 | Knock Sensor 2 Circuit. |
| P0335 | Crankshaft Position Sensor "A" Circuit. |
| P0336 | Crankshaft Position Sensor "A" Circuit Range/Performance. |
| P0340 | CMP Sensor Circuit malfunction. |
| P0385 | Crankshaft Position Sensor "B" NE2 Circuit. |
| P0401 | EGR Flow Insufficient. |
| P0402 | EGR Flow Excessive. |
| P0420 | Catalyst System Efficiency below Threshold. |
| P0430 | Catalyst System Efficiency below Threshold Bank 2. |
| P0440 | EVAP Control System. |
| P0441 | EVAP Control System Improper Purge Flow. |
| P0446 | EVAP Control System Vent Control. |
| P0450 | EVAP Control System Pressure Sensor. |
| P0451 | EVAP Control System Pressure Sensor Range/Performance. |
| P0500 | Vehicle Speed Sensor. |
| P0505 | Idle Speed Control System. |
| P0510 | Close Throttle Position Switch. |
| P1100 | BARO Sensor Circuit. |
| P1120 | Accelerator Pedal Position Sensor Circuit. |
| P1121 | Accelerator Pedal Position Sensor Range/Performance Problem. |
| P1125 | Throttle Control Motor Circuit. |
| P1126 | Magnetic Clutch Circuit. |
| P1127 | ETCS Actuator Power Source Circuit. |
| P1128 | Throttle Control Motor Lock. |
| P1129 | Electric Throttle Control System. |
| P1130 | Air-Fuel Sensor Circuit Range/Performance. |
| P1133 | Air-Fuel Sensor Circuit Response. |
| P1135 | Air-Fuel Sensor Heater Circuit Response. |
| P1150 | A/F Sensor Circuit Range/Performance. |
| P1153 | A/F Sensor Circuit Response. |
| P1155 | A/F Sensor Heater Circuit. |
| P1200 | Fuel Pump Relay Circuit. |
| P1300 | Igniter Circuit - No. 1. |
| P1310 | Igniter Circuit - No. 2. |
| P1335 | No Crankshaft Position Sensor Signal - Engine Running. |
| P1349 | VVT System. |
| P1400 | Sub-Throttle Position Sensor. |
| P1401 | Sub-Throttle Position Sensor Range/Performance Problem. |
| P1405 | Turbo Pressure Sensor Circuit. |
| P1406 | Turbo Pressure Sensor Range/Performance Problem. |
| P1410 | EGR Valve Position Sensor Circuit. |
| P1411 | EGR Valve Position Sensor Circuit Range/Performance. |
| P1500 | Starter Signal Circuit. |
| P1510 | Boost Pressure Control Circuit. |
| P1511 | Boost Pressure Low. |
| P1512 | Boost Pressure High. |
| P1520 | Stop Lamp Switch Signal. |
| P1565 | Cruise Control Main Switch Circuit. |
| P1600 | ECM. |
| P1605 | Knock Control CPU. |
| P1630 | Traction Control System. |
| P1633 | ECM. |
| P1652 | Idle Air Control Valve Control Circuit. |
| P1656 | OCV Circuit. |
| P1658 | Wastegate Valve Control Circuit. |
| P1661 | EGR Circuit. |
| P1662 | EGR by-pass Valve Control Circuit. |
| P1780 | Park/Neutral Switch. |