## Volkswagen Trouble Codes

Some years and models may not be applicable.

RETRIEVING CODES (DIGIFANT System)

- Turn the ignition ON. Connect the black end of the jumper cable to the black diagnostic connector in center console under shift boot. Connect the white end of the jumper to the white diagnostic connector.

- After five seconds the OBD CHECK light should begin to flash. Remove the jumper, but do not turn off the ignition. Record the flashes.

- When code 4444 or 0000 appears, then either turn off the ignition or reinstall the jumper to end code readout.

Possible wiring faults should be checked and corrected before replacing a suspected component. Once the fault is corrected, the permanent fault memory should be erased as described below.

CLEARING CODES

  - Make sure the ignition is OFF. Connect the jumper cable as described in RETRIEVING CODES. Turn the ignition ON.

  - After 5 seconds remove the jumper cable. The OBD light should flash 4444. Turn the ignition Off or reinstall the jumper cable to end the procedure.

RESETTING ECU

The Digifant I ECU must be reset to its reference setting after certain conditions or repairs as follows:

- The blue coolant temperature sensor connector has been disconnected with the engine running

- The following components have been replaced:

  - Digifant ECU

  - Air Flow Sensor

  - Throttle Valve Potentiometer

  - Throttle Valve Body

When resetting the ECU, the engine must be at normal operating temperature, all electrical accessories must be Off, the idle stabilization system must be okay, and the exhaust system must be leak-free.

- With the engine off, disconnect the crankcase ventilation hose from the emission control valve atop the valve cover, then plug the hose. Start the engine and let it idle. Disconnect the blue coolant temperature sensor harness connector.

- Let the engine idle for one minute, then reconnect the harness connector. CAUTION: DO NOT reconnect the coolant temperature sensor harness connects, while the radiator cooling fan is running.

- Turn the engine off and reconnect the crankcase ventilation hose. Erase the OBD fault memory.

NOTE: Disconnecting the coolant temperature sensor with the engine running generates fault code 2312.

Retrieving Codes (Bosch Motronic CIS-E)

The Bosch Motronic system (CIS-E) requires a scanner to retrieve and clear the trouble codes.

### Trouble Codes

CIS-E Motronic & Digifant Systems

VR-6 Motronic System

96–97 All & 98 Jetta - Diesel Turbo Direct Injection

96–00 Motronic OB II System

98–00 New Beetle & Passat - Diesel Turbo Direct Injection

99–00 Jetta - Diesel Turbo Direct Injection

## CIS-E Motronic & Digifant Systems

| Code |  |
| --- | --- |
| CIS-E MOTRONIC SYSTEM |  |
| 1111 | Control Unit Defective |
| 1231 | Transmission Speed Sensor (No Signal) |
| 2112 | Ignition Reference Sender |
| 2113 | Hall Sender |
| 2121 | Idle Switch and/or Circuit |
| 2141 | First Knock Regulator |
| 2142 | KS 1 (No Signal) |
| 2144 | KS 2 (No Signal) |
| 2231 | Idle Speed Regulation |
| 2232 | Air Flow Sensor |
| 2312 | ECT Sensor |
| 2341 | O2S |
| 2342 | O2S |
| 2411 | EGR System |
| 4431 | Idle Stabilizer and/or Circuit |
| 4444 | No Fault Registered |
| DIGIFANT SYSTEM |  |
| 1111 | Control Unit Defective |
| 4444 | No Fault Registered |
| 2141 | First Knock Control Limit Exceeded |
| 2142 | Knock Sensor |
| 2212 | Throttle Potentiometer |
| 2222 | Manifold Vacuum Sensor |
| 2232 | Air Flow Potentiometer |
| 2242 | CO Potentiometer |
| 2312 | ECT Sensor |
| 2322 | IAT Sensor |
| 2323 | Air Flow Sensor Potentiometer |
| 2341 | O2S Control Limit Exceeded |
| 2342 | O2S |
| 4411 | No. 1 Cyl. Injector Defective |

## VR-6 Motronic System

| MIL | VAG |  |
| --- | --- | --- |
| 1231 | 00281 | VSS Signal |
| 2111 | 00513 | ESS Defective |
| 2113 | 00515 | Hall Sensor Rotor |
| 2212 | 00518 | TVP Circuit |
| 2312 | 00522 | ECT Sender |
| 2141 | 00535 | Knock Regulation |
| 2142 | 00524 | Knock Sensor |
| 2342 | 00525 | O2S Probe |
| 2412 | 00527 | IAT Sender |
| 2234 | 00532 | Battery Voltage Too High/Low |
| 2231 | 00533 | Idle Regulation Adaptation |
| 2341 | 00537 | O2S Probe Heater |
| 2144 | 00540 | KS No. 2 |
| 2214 | 00543 | Maximum Engine Speed Exceeded |
| 2314 | 00545 | Engine/Transaxle Electrical Connection Short To Ground |
| 2324 | 00553 | Air Volume Meter |
| 2413 | 00561 | Fuel Pressure Improper |
| 2411 | 00585 | Temperature Sensor In EGR Valve |
| — | 00586 | EGR Valve |
| — | 01025 | OBD Lamp |
| 4332 | 01242 | Loose Terminal To Output Components Or Control Unit |
| 4343 | 01247 | Carbon Canister System Solenoid Valve No. 1 |
| 4411 | 01249 | Injector No. 1 Cyl. |
| 4412 | 01250 | Injector No. 2 Cyl. |
| 4413 | 01251 | Injector No. 3 Cyl. |
| 4414 | 01252 | Injector No. 4 Cyl. |
| 4421 | 01253 | Injector No. 5 Cyl. |
| 4422 | 01254 | Injector No. 6 Cyl. |
| 4431 | 01257 | Idle Speed Stabilizer Valve |
| 4433 | 01259 | Fuel Pump Relay |
| 4312 | 01265 | EGR Valve |
| 1111 | 65535 | ECM |

## Diesel Turbo Direct Injection (96–97 All & 98 Jetta)

| MIL | VAG |  |
| --- | --- | --- |
| 1414 | 00310 | Temperature Sensor 1 For Catalyst |
| 1422 | 00312 | Temperature Sensor 2 For Catalyst |
| 1423 | 00313 | Catalyst Questionable Signal |
| 2111 | 00513 | ESS |
| 2222 | 00519 | MAP Sensor |
| 2312 | 00522 | ECT Sensor |
| 2412 | 00527 | IAT Sensor |
| 2234 | 00532 | Supply Voltage |
| 2311 | 00539 | Fuel Temperature Sensor |
| 2324 | 00553 | MAF Sensor |
| 2221 | 00575 | MAP Sensor |
| 2211 | 00542 | Needle Lift Sensor |
| 2244 | 00550 | Injection Start Control |
| — | 00625 | VSS |
| — | 00668 | Battery + Signal Low |
|  | 00671 | Speed Control Switch |
| — | 00741 | Brake Pedal Position Monitor |
| — | 00585 | Temperature Sensor In EGR Valve |
| — | 00765 | Modulating Piston Displacement Sensor |
| 3131 | 00777 | TP Sensor |
| — | 01117 | Alternator Load Signal For Terminal DF |
| — | 01208 | Data Record Changed |
| 4321 | 01237 | Fuel Cutoff Valve |
| 4332 | 01242 | Final Stage In Control Module |
| 4113 | 01268 | Quantity Adjuster |
| 1111 | 65535 | Direct Fuel Injection Control Module |

## Motronic OBII System (96–00)

| OBD2 | VAG |  |
| --- | --- | --- |
| — | 00524 | Knock Sensor 1, Signal Too Low |
| — | 00532 | Supply Voltage (B+) Signal Too Low |
| — | 00543 | RPM Upper Limit Exceeded |
| — | 00545 | Engine/Transmission Electrical Connection |
| — | 00577 | Knock Control Sensor Cylinder 1 Control Limit Exceeded |
| — | 00578 | Knock Control Sensor Cylinder 2 Control Limit Exceeded |
| — | 00579 | Knock Control Sensor Cylinder 3 Control Limit Exceeded |
| — | 00580 | Knock Control Sensor Cylinder 4 Control Limit Exceeded |
| — | 00652 | Transmission Range Controller Open/Short Circuit To B+ |
| — | 00668 | Battery Positive Voltage (B+) Terminal 30, Signal Too High/Too Low/Interrupted/Implausible |
| — | 00750 | Malfunction Indicator Lamp Faulty |
| — | 01165 | Throttle Body Control Module |
| — | 01177 | Engine Control Module Faulty |
| P0102 | 16486 | MAF/VAF Circuit Input Low |
| P0103 | 16487 | MAF/VAF Circuit Input High |
| P0107 | — | Manifold Absolute Pressure Or Barometric Pressure Low Input |
| P0108 | — | Manifold Absolute Pressure Or Barometric Pressure High Input |
| P0112 | 16496 | IAT Circuit Input Low |
| P0113 | 16497 | IAT Circuit Input High |
| P0116 | 16500 | ECT Circuit Range/Performance |
| P0117 | 16501 | ECT Circuit Input Low |
| P0118 | 16502 | ECT Circuit Input High |
| P0120 | 16504 | TP Sensor Circuit ‘‘A’’ Malfunction |
| P0121 | 16505 | TP Sensor Circuit ‘‘A’’ Range/Performance |
| P0122 | 16506 | Throttle Position Circuit ‘‘A’’ Input Low |
| P0123 | — | Throttle Position Circuit ‘‘A’’ Input High |
| P0125 | 16509 | Insufficient Coolant Temperature For Closed Loop Fuel Control |
| P0130 | — | O2S Circuit Bank 1 Sensor 1 Malfunction |
| P0131 | 16515 | O2S Circuit Low Voltage Bank 1 Sensor 1 |
| P0132 | 16516 | O2S Circuit High Voltage Bank 1 Sensor 1 |
| P0133 | 16517 | O2S Circuit Slow Response Bank 1 Sensor 1 |
| P0134 | 16518 | O2S Circuit No Activity Detected Bank 1 Sensor 1 |
| P0135 | 16519 | HO2S Heater Circuit Malfunction Bank 1 Sensor 1 |
| P0136 | — | O2S Circuit Bank 1 Sensor 2 Malfunction |
| P0137 | 16521 | O2S Circuit Low Voltage Bank 1 Sensor 2 |
| P0138 | 16522 | O2S Circuit High Voltage Bank 1 Sensor 2 |
| P0139 | — | O2S Circuit Slow Response Bank 1 Sensor 2 |
| P0140 | 16524 | O2S Circuit No Activity Detected Bank 1 Sensor 2 |
| P0141 | 16525 | HO2S Heater Circuit Malfunction Bank 1 Sensor 2 |
| P0150 | — | O2S Circuit Bank 2 Sensor 1 Malfunction |
| P0151 |  | O2S Circuit Bank 2 Sensor 1 Low Voltage |
| P0152 | — | O2S Circuit Bank 2 Sensor 1 High Voltage |
| P0153 | — | O2S Circuit Bank 2 Sensor 1 Slow Response |
| P0154 | — | O2S Circuit Bank 2 Sensor 1 No Activity Detected |
| P0156 | — | O2S Circuit Bank 2 Sensor 2 Malfunction |
| P0158 | — | O2S Circuit Bank 2 Sensor 2 High Voltage |
| P0160 | — | O2S Circuit Bank 2 Sensor 2 No Activity Detected |
| P0157 | — | O2S Circuit Bank 2 Sensor 2 Low Voltage |
| P0170 | — | Fuel Trim Bank 1 Malfunction |
| P0171 | 16555 | System Too Lean Bank 1 |
| P0172 | 16556 | System Too Rich Bank 1 |
| P0300 | 16684 | Random Misfire Detected |
| P0301–P0306 | 16685–16690 | Misfire Detected Cyls. 1–6 |
| P0321 | 16705 | Ignition/Distributor Engine Speed Inspection Circuit Range Performance |
| P0322 | 16706 | Ignition/Distributor Engine Speed Inspection Circuit No Signal |
| P0327 | 16711 | KS 1 Circuit Input Low Bank 1 Or Single Sensor |
| P0328 | — | KS 1 Signal Too High |
| P0332 | 16716 | KS 2 Circuit Input Low Bank 2 |
| P0333 | — | KS 2 Signal Too High |
| P0341 | 16725 | CMP Sensor Circuit Range/Performance |
| P0342 | — | CMP Sensor Circuit Low Input |
| P0343 | — | CMP Sensor Circuit High Input |
| P0400 | 16784 | EGR Flow Malfunction |
| P0411 | 16795 | Secondary Air Injection System Improper Flow Detected |
| P0420 | — | Catalyst System Efficiency Below Threshold Bank 1 |
| P0422 | 16806 | Main Catalyst Efficiency Below Threshold Bank 1 |
| P0432 | — | Main Catalyst Efficiency Below Threshold Bank 2 |
| P0440 | 16824 | EVAP Control System Malfunction |
| P0441 | — | EVAP Incorrect Purge Flow |
| P0442 | 16826 | EVAP Control System Small Leak Detected |
| P0455 | 16839 | EVAP Control System Large Leak Detected |
| P0501 | 16885 | VSS Range/Performance |
| P0506 | 16890 | ISC RPM Lower Than Expected |
| P0507 | 16891 | ISC RPM Higher Than Expected |
| P0510 | 16894 | CTP Switch Malfunction |
| P0560 | — | System Voltage Malfunction |
| P0562 | — | System Voltage Low |
| P0563 | — | System Voltage High |
| P0571 | — | Cruise/Brake Switch Circuit Malfunction |
| P0601 | — | internal Control Module Memory Check Sum Error |
| P0604 | — | internal Control Module Random Access Memory Error |
| P0605 | 16989 | Internal Control Module Read Only Memory Error |
| P0707 | — | Trans. Range Sensor Circuit Low Input |
| P0708 | — | Trans. Range Sensor Circuit High Input |
| P0715 | 17099 | Input/Turbine Speed Sensor Circuit Malfunction |
| P0722 | 17106 | OSS No Signal |
| P0725 | 17109 | Engine Speed Input Circuit Malfunction |
| P0748 | 17132 | Pressure Control Solenoid Electrical Malfunction |
| P0753 | 17137 | Shift Solenoid ‘‘A’’ Electrical Malfunction |
| P0758 | 17142 | Shift Solenoid ‘‘B’’ Electrical Malfunction |
| P0763 | 17147 | Shift Solenoid ‘‘C’’ Electrical Malfunction |
| P0768 | 17152 | Shift Solenoid ‘‘D’’ Electrical Malfunction |
| P0773 | 17157 | Shift Solenoid ‘‘E’’ Electrical Malfunction |
| P1102 | — | O2S Heating Circuit Bank 1 Sensor 1 Voltage Too Low/Air Leak |
| P1105 | — | O2S Heating Circuit Bank 1 Sensor 2 Short To B+ |
| P1107 | — | O2S Heating Circuit Bank 2 Sensor 1 Short To B+ |
| P1110 | — | O2S Heating Circuit Bank 2 Sensor 2 Short To B+ |
| P1113 | — | O2S Sensor Heater Resistance Too High Bank 1 Sensor 1 |
| P1115 | — | O2S Sensor Heater Circuit Short To Ground Bank 1 Sensor 1 |
| P1116 | — | O2S Sensor Heater Circuit Open Bank 1 Sensor 1 |
| P1117 | — | O2S Sensor Heater Circuit Short To Ground Bank 1 Sensor 2 |
| P1118 | — | O2S Sensor Heater Circuit Open Bank 1 Sensor 2 |
| P1127 | 17535 | Long Term Fuel Trim B1 System Too Rich |
| P1128 | 17536 | Long Term Fuel Trim B1 System Too Lean |
| P1129 | — | Long Term Fuel Trim B2 System Too Rich |
| P1130 | — | Long Term Fuel Trim B2 System Too Lean |
| P1136 | — | Long Term Fuel Trim Add. Fuel B1 System Too Lean |
| P1137 | — | Long Term Fuel Trim Add. Fuel B1 System Too Rich |
| P1138 | — | Long Term Fuel Trim Add. Fuel B2 System Too Lean |
| P1139 | — | Long Term Fuel Trim Add. Fuel B2 System Too Rich |
| P1141 | — | Load Calculation Cross Check Range/Performance |
| P1171 | — | Throttle Actuation Potentiometer Sign. 2 Range/Performance |
| P1172 | — | Throttle Actuation Potentiometer Sign. 2 Signal Too Low |
| P1173 | — | Throttle Actuation Potentiometer Sign. 2 Signal Too High |
| P1176 | — | Rear O2S Correction |
| P1177 | — | O2 Correction Behind Catalyst B1 Limit Attained |
| P1196 | — | O2S Heater Circuit Bank 1 Sensor 1 Electrical Malfunction |
| P1197 | — | O2S Heater Circuit Bank 2 Sensor 1 Electrical Malfunction |
| P1198 | — | O2S Heater Circuit Bank 1 Sensor 2 Electrical Malfunction |
| P1199 | — | O2S Heater Circuit Bank 2 Sensor 2 Electrical Malfunction |
| P1213–P1218 | 17621–17626 | Injector Circuit Cyls. 1–6 Short To B+ |
| P1225–P1230 | 17633–17638 | Injector Circuit Cyls. 1–6 Short To Ground |
| P1237–P1242 | 17645–17650 | Injector Circuit Open Cyls. 1–6 |
| P1300 | 17708 | Misfire Detected, Fuel Level Too Low |
| P1250 | — | Fuel Level Too Low |
| P1325 | — | Cyl. 1 Knock Control Limit Attained |
| P1326 | — | Cyl. 2 Knock Control Limit Attained |
| P1327 | — | Cyl. 3 Knock Control Limit Attained |
| P1328 | — | Cyl. 4 Knock Control Limit Attained |
| P1329 | — | Cyl. 5 Knock Control Limit Attained |
| P1330 | — | Cyl. 6 Knock Control Limit Attained |
| P1336 | — | Engine Torque Adaption At Limit |
| P1337 | — | CMP Sensor Bank 1 Short To Ground |
| P1338 | — | CMP Sensor Bank 1 Open Circuit Or Short To B+ |
| P1340 | 17748 | CKP/CMP Sensor Signals Out Of Sequence |
| P1341 | — | Ignition Coil Output Stage 1 Short To Ground |
| P1343 | — | Ignition Coil Output Stage 2 Short To Ground |
| P1345 | — | Ignition Coil Output Stage 3 Short To Ground |
| P1386 | — | Internal Control Module Knock Control Error |
| P1387 | — | Control Unit Internal Altitude Sensor |
| P1391 | — | CMP Sensor Bank 2 Short To Ground |
| P1392 | — | CMP Sensor Bank 2 Open Circuit/Short To B+ |
| P1393 | — | Ignition Coil Power Output Stage 1 Malfunction |
| P1394 | — | Ignition Coil Power Output Stage 2 Malfunction |
| P1395 | — | Ignition Coil Power Output Stage 3 Malfunction |
| P1401 | 17809 | EGR Valve Power Stage Short To Ground |
| P1402 | 17810 | EGR Valve Circuit Short To B+ |
| P1407 | 17815 | EGR Temperature Sensor Signal Too Low |
| P1408 | 17816 | EGR Temperature Sensor Signal Too High |
| P1410 | 17818 | Tank Ventilation Valve Circuit Short To B+ |
| P1420 | 17828 | Secondary Air Injection Control Module Electrical Malfunction |
| P1421 | 17829 | Secondary Air Injection Valve Circuit Short To Ground |
| P1422 | 17830 | Secondary Air Injection Valve Circuit Short To B+ |
| P1424 | — | Secondary Air Injection System Bank 1 Leak Detected |
| P1425 | 17833 | Tank Ventilation Valve Short To Ground |
| P1426 | 17834 | Tank Ventilation Valve Open Circuit |
| P1432 | — | Secondary Air Injection Valve Open |
| P1433 | — | Secondary Air Injection Pump Relay Circuit Open |
| P1434 | — | Secondary Air Injection Pump Relay Circuit Short To B+ |
| P1435 | — | Secondary Air Injection Pump Relay Circuit Short To Ground |
| P1436 | — | Secondary Air Injection Pump Relay Circuit Electrical Malfunction |
| P1440 | 17848 | EGR Valve Power Stage Open |
| P1450 | 17858 | Secondary Air Injection System Circuit Short To B+ |
| P1451 | 17859 | Secondary Air Injection Circuit Short To Ground |
| P1452 | 17860 | Secondary Air Injection System Circuit Open |
| P1471 | 17879 | EVAP Control System LDP Circuit Short to B+ |
| P1472 | 17880 | EVAP Control System LDP Circuit Short To Ground |
| P1473 | 17881 | EVAP Control System LDP Open Circuit |
| P1475 | 17883 | EVAP Control System LDP Malfunction/Signal Circuit Open |
| P1476 | 17884 | EVAP Control System LDP Malfunction/Insufficient Vacuum |
| P1477 | 17885 | EVAP Control System LDP Malfunction |
| P1478 | 17886 | EVAP Control System LDP Clamped Tube Detected |
| P1500 | 17908 | Fuel Pump Relay Electrical Circuit Malfunction |
| P1501 | — | Fuel Pump Relay Circuit Short To Ground |
| P1502 | 17910 | Fuel Pump Relay Circuit Short To B+ |
| P1505 | — | Closed Throttle Position Does Not Close/Open Circuit |
| P1506 | — | Closed Throttle Position Switch Does Not Open./Short To Ground |
| P1512 | — | Intake Manifold Changeover Valve Circuit Short To B+ |
| P1515 | — | Intake Manifold Changeover Valve Circuit Short To Ground |
| P1516 | — | Intake Manifold Changeover Valve Circuit Open |
| P1519 | — | Intake Camshaft Control Bank 1 Malfunction |
| P1522 | — | Intake Camshaft Control Bank 2 Malfunction |
| P1539 | — | Clutch Pedal Switch Signal Fault |
| P1541 | — | Fuel Pump Relay Circuit Open |
| P1542 | — | Throttle Actuation Potentiometer Range/Performance |
| P1543 | 17951 | Throttle Actuation Potentiometer Signal Too Low |
| P1544 | 17952 | Throttle Actuation Potentiometer Signal Too High |
| P1545 | — | Throttle Position Control Malfunction |
| P1546 | — | Boost Pressure Control Valve Short To B+ |
| P1547 | — | Boost Pressure Control Valve Short To Ground |
| P1548 | — | Boost Pressure Control Valve Open |
| P1555 | — | Charge Pressure Upper Limit Exceeded |
| P1556 | — | Charge Pressure Negative Deviation |
| P1557 | — | Charge Pressure Positive Deviation |
| P1558 | — | Throttle Actuator Electrical Malfunction |
| P1559 | — | Idle Speed Control Throttle Position Adaption Malfunction |
| P1560 | — | Maximum Engine Speed Exceeded |
| P1564 | — | Idle Speed Control Throttle Position Low Voltage During Adaption |
| P1565 | 17973 | Idle Speed Control Throttle Position Lower Limit Not Obtained |
| P1568 | — | Idle Speed Control Throttle Position Mechanical Malfunction |
| P1569 | — | Switch For CCS Signal Faulty |
| P1580 | 17988 | Throttle Actuator B1 Malfunction |
| P1582 | 17990 | Idle Adaptation At Limit |
| P1600 | — | Power Supply Terminal No. 15 Low Voltage |
| P1602 | — | Power Supply Terminal No. 30 Low Voltage |
| P1603 | — | Internal Control Module Self Check |
| P1606 | — | Rough Road Spec. Engine Torque ABS-ECU Electrical Malfunction |
| P1611 | 18019 | MIL Call-Up Circuit/TCM Short To Ground |
| P1612 | — | Engine Control Module Incorrect Coding |
| P1613 | 18021 | MIL Call-Up Circuit Open/Short To B+ |
| P1624 | — | MIL Request Signal Active |
| P1626 | — | Data Bus Drive Missing Command From M/T |
| P1630 | — | Accelerator Pedal Position Sensor 1 Signal Too Low |
| P1631 | — | Accelerator Pedal Position Sensor 1 Signal Too High |
| P1633 | — | Accelerator Pedal Position Sensor 2 Signal Too Low |
| P1634 | — | Accelerator Pedal Position Sensor 2 Signal Too High |
| P1639 | — | Accelerator Pedal Position Sensor 1/2 Range Performance |
| P1640 | — | Internal Control Module (EEPROM) Error |
| P1648 | — | Data Bus Powertrain Malfunction |
| P1649 | — | Data Bus Powertrain Missing Message From Brake Controller |
| P1676 | — | Drive By Wire MIL Circuit Electrical Malfunction |
| P1677 | — | Drive By Wire MIL Circuit Short To B+ |
| P1678 | — | Drive By Wire MIL Circuit Short To Ground |
| P1679 | — | Drive By Wire MIL Circuit Open Circuit |
| P1681 | — | Control Module Programming Not Finished |
| P1686 | 18904 | Control Unit Error, Programming Error |
| P1690 | — | MIL Malfunction |
| P1691 | — | MIL Open Circuit |
| P1692 | — | MIL Short To Ground |
| P1693 | — | MIL Short To B+ |
| P1778 | 18186 | Solenoid EV7 Electrical Malfunction |
| P1780 | 18188 | Engine Intervention Readable |
| P1851 | — | Data Bus Drive Missing Command From ABS |

## Diesel Turbo Direct Injection (98–00 New Beetle & Passat)

| OBD2 | VAG |  |
| --- | --- | --- |
| — | 00532 | Power Supply Signal |
| — | 00545 | Engine/Transmission Electrical Connection Faulty Signal |
| — | 00668 | Power Supply Relay Circuit Faulty |
| — | 00671 | Cruise Control Switch Circuit |
| — | 00741 | Brake Pedal Monitor Signal Fault |
| — | 01117 | Alternator Load Signal Fault |
| — | 01180 | Engine Accessories Electrical Connections Short To B+ |
| — | 01193 | Relay For Preheating Coolant Low Heat Output Short To B+ |
| — | 01194 | Relay For Preheating Coolant High Heat Output Short To B+ |
| P0101 | 00553 | MAF Sensor Signal Fault |
| P0116 | 00522 | ECT Sensor Signal Fault |
| P0121 | 00777 | TP Sensor Signal Fault |
| P0123 | 00777 | TP Sensor Short To B+ |
| P0300 | 01162 | Combustion Misfire |
| P0301 | 01162 | Combustion Misfire Cyl. 1 |
| P0302 | 01162 | Combustion Misfire Cyl. 2 |
| P0303 | 01162 | Combustion Misfire Cyl. 3 |
| P0304 | 01162 | Combustion Misfire Cyl. 4 |
| P0321 | 00513 | ESS Signal Fault |
| P0322 | 00513 | ESS No Signal |
| P0380 | 01050 | Glow Plug Monitoring System |
| P0501 | 00625 | Speed Signal Fault |
| P0560 | 00532 | Voltage Supply Signal Fault |
| P0605 | 65535 | Control Module Faulty |
| P1144 | 00553 | MAF Sensor Short To Ground |
| P1145 | 00553 | MAF Sensor Short To B+ |
| P1146 | 00553 | MAF Sensor Voltage Supply |
| P1155 | 00519 | IAT Sensor Short To B+ |
| P1156 | 00519 | IAT Sensor Short To Ground |
| P1157 | 00519 | IAT Sensor Voltage Supply |
| P1160 | 00527 | IAT Sensor Short To Ground |
| P1161 | 00527 | IAT Sensor Short To B+ |
| P1162 | 00539 | Fuel Temperature Sensor Short To Ground |
| P1163 | 00539 | Fuel Temperature Sensor Short To B+ |
| P1245 | 00542 | Needle Lift Sensor Short To Ground |
| P1246 | 00542 | Needle Lift Sensor Signal Fault |
| P1247 | 00542 | Needle Lift Sensor Short To B+ |
| P1248 | 00550 | Injection Start Control |
| P1251 | 01269 | Cold Start Injector Short To B+ |
| P1252 | 01269 | Cold Start Injector Short To Ground |
| P1255 | 00522 | ECT Sensor Short To Ground |
| P1256 | 00522 | ECT Sensor Short To B+ |
| P1354 | 00765 | Modulating Piston Displacement Sensor Circuit |
| P1402 | 01265 | EGR Vacuum Regulator Solenoid Valve Short To B+ |
| P1403 | 00560 | EGR System |
| P1441 | 01265 | EGR Vacuum Regulator Solenoid Valve Short To Ground |
| P1537 | 01237 | Fuel Cut-Off Valve Mechanical Malfunction |
| P1538 | 01237 | Fuel Cut-Off Valve Short To Ground |
| P1540 | 00625 | Speed Signal Too High |
| P1546 | 01262 | Waste Gate Bypass Regulator Valve Short To B+ |
| P1549 | 01262 | Waste Gate Bypass Regulator Valve Short To Ground |
| P1550 | 00575 | Intake Manifold Pressure Control System |
| P1561 | 01268 | Quality Adjuster Control Difference |
| P1562 | 01268 | Quality Adjuster Control Upper Limit Value |
| P1563 | 01258 | Quality Adjuster Control Lower Limit Value |
| P1612 | 01044 | Control Module Incorrectly Coded |
| P1614 | 00750 | Exhaust Warning Lamp Signal Fault |
| P1616 | 00626 | Glow Plug Indicator Lamp Short To B+ |
| P1617 | 00626 | Glow Plug Indicator Lamp Short To Ground |
| P1618 | 01266 | Glow Plug Relay Short To B+ |
| P1619 | 01266 | Glow Plug Relay Short To Ground |
| P1623 | 00777 | TP Sensor Voltage Supply |
| P1626 | 18034 | Drive Data Bus No Message From Transmission Control Module |
| P1632 | 00777 | TP Sensor Voltage Supply |
| P1693 | 00750 | Exhaust warning Lamp Short To B+ |
| P1694 | 00750 | Exhaust warning Lamp Short To Ground |
| P1851 | 18259 | Drive Data Bus No Message From ABS Control Module |
| P1854 | 18262 | Drive Data Bus Hardware Malfunction |

## Diesel Turbo Direct Injection (99–00 Jetta)

| OBD2 | VAG |  |
| --- | --- | --- |
| P0101 | 16485 | MAF Sensor Signal Fault |
| P0116 | 16550 | ECT Sensor Signal Fault |
| P0321 | 16705 | ESS Signal Fault |
| P0322 | 16706 | ESS No Signal |
| P0501 | 16885 | Speed Signal Fault |
| P0605 | 16989 | Control Module Faulty |
| P1144 | 17552 | MAF Sensor Short To Ground |
| P1145 | 17553 | MAF Sensor Short To B+ |
| P1146 | 17554 | MAF Sensor Voltage Supply |
| P1155 | 17563 | IAT Sensor Short To B+ |
| P1156 | 17564 | IAT Sensor Short To Ground |
| P1157 | 17565 | IAT Sensor Voltage Supply |
| P1160 | 17568 | IAT Sensor Short To Ground |
| P1161 | 17569 | IAT Sensor Short To B+ |
| P1162 | 17570 | Fuel Temperature Sensor Short To Ground |
| P1163 | 17571 | Fuel Temperature Sensor Short To B+ |
| P1164 | 17572 | Fuel Temperature Sensor Signal Fault |
| P1245 | 17653 | Needle Lift Sensor Short To Ground |
| P1246 | 17654 | Needle Lift Sensor Signal Fault |
| P1247 | 17655 | Needle Lift Sensor Short To B+ |
| P1248 | 17656 | Injection Start Control |
| P1251 | 17659 | Cold Start Injector Short To B+ |
| P1252 | 17660 | Cold Start Injector Short To Ground |
| P1255 | 17663 | ECT Sensor Short To Ground |
| P1256 | 17664 | ECT Sensor Short To B+ |
| P1354 | 17762 | Modulating Piston Displacement Sensor Circuit |
| P1387 | 17795 | ECM Inoperative |
| P1402 | 17810 | EGR Vacuum Regulator Solenoid Valve Short To B+ |
| P1403 | 17811 | EGR System |
| P1441 | 17849 | EGR Vacuum Regulator Solenoid Valve Short To Ground |
| P1537 | 17945 | Fuel Cut-Off Valve Mechanical Malfunction |
| P1538 | 17946 | Fuel Cut-Off Valve Short To Ground |
| P1540 | 17948 | Speed Signal Too High |
| P1546 | 17955 | Waste Gate Bypass Regulator Valve Short To B+ |
| P1549 | 17957 | Waste Gate Bypass Regulator Valve Short To Ground |
| P1550 | 17958 | Intake Manifold Pressure Control System |
| P1561 | 17969 | Quality Adjuster Control Difference |
| P1562 | 17970 | Quality Adjuster Control Upper Limit Value |
| P1563 | 17971 | Quality Adjuster Control Lower Limit Value |
| P1600 | 18008 | Supply Voltage Terminal No. 15 Too Low |
| P1612 | 18020 | Control Module Incorrectly Coded |
| P1616 | 18024 | Glow Plug Indicator Lamp Short To B+ |
| P1617 | 18025 | Glow Plug Indicator Lamp Short To Ground |
| P1618 | 18026 | Glow Plug Relay Short To B+ |
| P1619 | 18027 | Glow Plug Relay Short To Ground |
| P1626 | 18034 | Drive Data Bus No Message From Transmission Control Module |
| P1631 | 18039 | TP Sensor Signal Too High |
| P1632 | 18040 | TP Sensor Signal Supply Voltage |
| P1639 | 18047 | TP Sensor Signal Faulty |
| P1640 | 18048 | ECM Inoperative |
| P1648 | 18056 | Drivetrain CAN-Bus System Component Inoperative |
| P1649 | 18057 | Drivetrain CAN-Bus System ABS Control Module Faulty Signal |
| P1693 | 00750 | Exhaust warning Lamp Short To B+ |
| P1851 | 18259 | Drive Data Bus No Message From ABS Control Module |
| P1854 | 18262 | Drive Data Bus Hardware Malfunction |