BRAKE SYSTEM

Return To Main Table of Contents

GENERAL ............................................................................ .	2

BRAKE  PEDAL . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .................................... .  10

MASTER CYLINDER . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	13

BRAKE  BOOSTER .  .  .  .  .  .  .  ........................................................... 16

BRAKE LINE . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	18

PROPORTIONING  VALVE .  .  .  ............................................... 19

FRONT DISC BRAKE ...................................................   20

REAR DRUM BRAKE .........................................................	25

REAR  DISK  BRAKE ............................................................. 28

PARKING  BRAKE ................................................................ .. 32

58-2	GENERAL

GENERAL

SPECIFICATIONS

Master cylinder

Type

I:D.

Fluid level warning sensor

Brake  booster Type Effective dia.

Boosting  ratio

Proportioning valve

Cut-in pressure (Split point) Decompression  ratio

Front brake

Type

Disc O.D.

Disc  thickness Pad thickness Cylinder I.D.

Rear brake Type Drum I.D.

Brake lining thickness

Cylinder I.D. Clearance  adjustment

Parking  brake

Type

Braking type

Cable  arrangement

Tandem

22.22 mm (0.875 in.) Provided

Vacuum

233.1 mm (9.177 in.)

4.0 : 1

3.92 MPa (40 kg/cm2, 569 Psi)

3.7 : 1

Floating type with ventilated disc

257 mm (10.118 in.)

22 mm (0.866 in.)

11 mm (0.433 in.)

54 mm (2.126 in.)

Leading-trailing drum

203.2 mm (8 in.)

4.05 mm (0.159 in.)

19.05 mm (0.75 in.) Automatic

Mechanical brake acting on rear wheels

Lever type (Cam shape) V type

GENERAL	58-3

SERVICE  STANDARD

Standard value

Brake pedal height	178 mm (7.008 in.) Brake pedal stroke	145 mm (5.709 in.)

Stop light switch outer case to pedal arm clearance	0.5-1.0 mm (0.02-0.04 in.) Brake pedal free play	3-8 mm (0.118-0.315 in.) Brake pedal to floorboard clearance	65 mm (2.56 in.) or more Booster push rod to master cylinder piston clearance	0 (at 500 mmHg vacuum) Parking brake lever stroke (When lever assembly is	7-8 clicks

pulled with 196 N (20 Kg, 44 lb) force) Service limit

Front disc brake pad thickness Front disc thickness (minimum) Front disc runout

Drum brake lining thickness

Brake drum I.D. (maximum)

Wheel cylinder to piston clearance

2.0 mm (0.079 in.)

20 mm (0.787 in.)

0.15 mm (0.006 in.)

1.5 mm (0.059 in.)

205.2 mm (8.079 in.)

0.15 mm (0.006 in.)

TIGHTENING  TORQUE

Nm	kg.cm	lb.ft

Master cylinder to booster mounting nut

Brake booster mounting nut

Brake booster hose fitting to manifold

Bleeder  screw

Brake tube flare nut, brake hose Proportioning valve mounting nut Caliper guide rod bolt

Caliper pin bolt

Caliper assembly to knuckle Brake hose to front caliper Backing plate mounting bolt

8-12	80-120	6-9

8-12	80-120	6-9

8-12	80-120	6-9

7-13	70-130	5-9.5

13-17	130-170	9-12

8-12	80-120	6-9

22-32	220-320	16-24

35-45	350-450	26-33

69-85	690-850	44-63

25-30	250-300	18-22

50-60	500-600	37-44

GENERAL

SPECIAL TOOL

GENERAL	58-5

TROUBLESHOOTING

Symptom

Noise or vibration when brakes are applied

Vehicle pulls to one side when brakes are applied

Insufficient braking power

Increased pedal stroke

(Reduced pedal to floorboard clearance)

Probable  cause

Backing plate or caliper improperly mounted

Loose backing plate or caliper mounting bolts Unevenly worn or cracked brake drum or brake disc Foreign material in brake drum

Seized pad or lining contact surface Excessive caliper to pad assembly clearance Uneven pad contact

Lack of lubrication in sliding parts

Loose  suspension  parts

Difference in left and right tire inflation pressure lmproper front wheel alignment

Inadequate contact of pad or lining Grease or oil on pad or lining surface Drum eccentricity or uneven wear Incorrect wheel cylinder installation Auto adjuster malfunction

Low or contaminated brake fluid

Air in brake system

Brake booster malfunction

Inadequate contact of pad or lining Grease or oil on pad or lining surface Auto adjuster malfunction

Overheated brake rotor due to dragging of pad or lining

Restricted brake line Proportioning valve malfunction Air in brake system

Brake fluid leaks

Auto adjuster malfunction

Excessive push rod to master cylinder clearance

Remedy Correct Retighten Replace Clean Replace Correct Correct Lubricate Retighten Adjust Adjust Correct Replace Replace Correct Correct

Replenish or change Bleed the system Correct

Correct Replace Correct Correct

Correct

Replace

Bleed the system

Correct Correct Adjust

58-6	GENERAL

GENERAL	58-7

SERVICE  ADJUSTMENT  PROCEDURE

BRAKE PEDAL INSPECTION AND ADJUSTMENT

1. Pedal height

Pedal height (from top of pedal to toeboard) A ...............................

178 mm (7.008 in.) If the pedal height is out of specification, adjust by the following procedure.

1)  Back off the stop lamp switch to a position where it does not contact the pedal arm.

2) Turn the stop lamp switch until the clearance between the stop lamp switch outer case and the pedal arm is within the standard value, and tighten the lock nut.

Clearance between pedal and stop lamp switch B ........................

0.5 to 1.0 mm (0.02 to 0.04 in.)

2.	Brake pedal free play.

Brake pedal free play C . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

3-8 mm (0.118-0.315 in.)

3.	Start the engine, apply the brake pedal with approximately 50 kg (110 Ibs.) of force, and measure the clearance between the brake pedal and the floor board.

Pedal to floor board clearance D

when pedal is depressed 50 kg (110Ibs.)force ........................

90 mm (3.543 in.) or more when pedal is fully depressed without brake fluid ..................

60 mm (2.362 in.) or more

58-8	GENERAL

BRAKE BOOSTER OPERATION TEST

For a simple check of the brake booster operation, use the following tests.

1.		Run the engine for one or two minutes, then stop it. Depress the brake pedal several times using normal foot pressure. If the pedal goes down further the first time, but gradually rises after the second or third time, the brake booster is functioning properly. Go to step 2.

2.	With the engine stopped, depress the brake pedal several times.

Depress the brake pedal and start the engine.

If the pedal goes down slightly, the booster is in good condition. Go to step 3.

3.	With the engine running, depress the brake pedal and then stop

the engine.

Hold the pedal depressed for 30 seconds. If the pedal height does not change, the booster is in good condition.

If one of the above three tests is not okay, check the vacuum hoses, the check valve and the brake booster. Make any necessary corrections. If all the tests are OK, the unit is good.

BLEEDING OF BRAKE SYSTEM

1.		Remove the reservoir cap and fill the brake fluid reservoir. CAUTION

Do not allow brake fluid to remain on a painted surface. Wash it off immediately.

NOTE

When bleeding by pressured fluid, do not depress the brake  pedal.

2.	Connect a vinyl tube to the wheel cylinder bleeder screw and insert the other end of tube in a half full container of brake fluid.

3.	Slowly pump the brake pedal several times.

4.	While depressing the brake pedal fully, loosen the bleeder screw until fluid starts to run out. Then close the bleeder screw.

GENERAL

5.	Repeat steps 3 and 4 until there are no more bubbles in the fluid.

4.	Tighten the bleeder screw.

Bleeder screw tightening  torque  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

7-9 Nm (70-90 kg.cm, 5.1-6.6 Ib.ft)

7.		Repeat the above procedure for each wheel in the sequence shown in the illustration.

PARKING  BRAKE  STROKE  ADJUSTMENT

1.		Pull the brake lever with a force of approximately 20 kg (44 Ibs.), and count the number of clicks.

Parking brake lever stroke [Standard value] ...................

7-8 clicks

2.		If the number of notches is not within the standard value, adjust the cable length with the adjusting nut of the equalizer.

3.	The indicator light will go out when the brake lever is fully released, and will light with the lever is pulled up one notch. If it does not operate, check the bulb and/or switch.

4.	After the adjustment, check that the rear brakes do not drag with the parking brake lever released.

58-10	BRAKE  PEDAL

BRAKE PEDAL

COMPONENTS

BRAKE  PEDAL	58-11

REMOVAL

1.	Remove the lower crash pad assembly.

2.	Remove the steering column shaft mounting bolts (Refer to

Steering Group).

3.	Remove the clutch pedal assembly.

4.	Remove the console assembly (Refer to Body Group)

5.	Remove the glove box housing cover and RH side lower crash pad.

6.	Remove the blower switch and air duct control assembly.

7.	Drain the coolant, disconnect the heater hose and remove the heater unit assembly (Refer to Air Conditioning Group).

8.	Remove the stop lamp switch connector.

9.	Remove the return spring.

10. Disconnect the operating rod and brake pedal from each other.

11. Remove the stopper and the washer.

12. Loosen the booster mounting nut and remove the right side pedal support member.

13. Remove the pedal support (left) and remove the pedal from the left.

INSPECTION

1.	Check the bushing for wear.

2.	Check the brake pedal for distortion.

3.	Check the brake pedal return spring for damage.

4.	Check the stop lamp switch.

1) With an ohmmeter connected to the stop lamp switch terminals, check for continuity.

2)  If there is no continuity when the plunger is depressed and there is continuity when the plunger is released, the stop lamp switch is normal.

58-12	BRAKE PEDAL

INSTALLATION

1.	Apply chassis grease to the bushing bore and flanges.

NOTE

install the bushing in the correct position as shown in the illustration.

2.	Apply chassis grease to the sliding surface of the brake pedal and operating rod clevis pin.

CAUTION

Be sure to install the split pin on the operating rod clevis pin.

3.	Adjust the brake pedal height and free play.

4.	Adjust the clutch pedal stroke and free play.

5.	Install the steering shaft.

MASTER  CYLINDER	58-13

MASTER  CYLINDER

COMPONENTS

TORQUE : Nm (kg.cm, Ib.ft)

REMOVAL

1.	Remove the fluid level warning device connector.

2.		Disconnect the brake lines from the master cylinder, and plug the open ports.

CAUTION

Do not allow brake fluid to remain on a painted surface. Wash it off immediately.

3.	Remove the master cylinder mounting nuts. Disconnect the proportioning valve mounting bracket, then lift out the master cylinder.

DISASSEMBLY

1.	Remove the reservoir cap and drain the brake fluid into a suitable  container.

2.	Remove the reservoir from the master cylinder.

NOTE

If necessary, support the master cylinder in a vise at its flange not at its bore.

3.	While depressing the piston, remove the snap ring.

58-14	MASTER  CYLINDER

4.		Remove the primary and secondary pistons from the master cylinder body.

NOTE

1) Be careful not to damage the cylinder bore.

2) Do not disassemble the primary and secondary piston assemblies.

INSPECTION

1.	Check the master cylinder bore for rust or scoring.

2.	Check the primary and secondary pistons for rust, scoring, wear, damage or deterioration.

3.	Check the primary and secondary piston springs for deteriora- tion.

ASSEMBLY

1.	Apply the specified brake fluid to the inner surface of the master cylinder body and to the outside of the secondary and primary pistons.

Recommended brake fluid . . . . . . . . . . . . . . . . .  DOT 3 or equivalent

2.		Carefully insert the spring and secondary piston assembly in the master cylinder bore.

3.	Carefully insert the primary piston assembly in the master cylinder bore.

4.	Depress the primary piston and install the snap ring in the cylinder bore groove as illustrated.

5.	Install the reservoir cap on the reservoir.

6.	Lubricate the two grommets at both inside and outside with genuine brake fluid and then insert them into the master cylinder body.

NOTE

Whenever the reservoir is replaced, the grommets must also be replaced.

7.		Press the reservoir into the master cylinder body with the fluid level indicator socket facing inboard. The reservoir should snap in place indicating that it is secure as illustrated.

8.	Connect the fluid level warning connector in the socket on the reservoir.

MASTER  CYLINDER

INSTALLATION

1.	Install the master cylinder on the brake booster with two nuts.

Nut ..............................  8-12  Nm  (80-120  kg.cm,  6-9 Ib.ft)

2.	Connect two brake tubes and fluid level warning connector. Brake tube flare nut . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

13-17 Nm (130-170 kg.cm, 9-12 Ib.ft)

3.	Fill the master cylinder reservoir with brake fluid and bleed the system.

58-16	BRAKE  BOOSTER

BRAKE BOOSTER

CHECK VALVE OPERATION TEST

REMOVAL

1.	Remove the master cylinder.

(Refer to page 58-13)

CAUTION

Do not allow brake fluid to remain on a painted surface. Wash it off immediately

BRAKE  BOOSTERR	58-17

2.	Disconnect the vacuum hose from the booster.

3.	Remove the operating rod from the brake pedal.

4.	Loosen the booster mounting nuts.

5.	Lift out the booster assembly.

INSTALLATION

1.	When the booster assembly is installed, replace the packing at each end of the booster mounting holder, if necessary.

2.	Install brake booster and tighten the mounting nuts.

Tightening  torque  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

8-12 Nm (80-120 kg.cm, 6-9 Ib.ft)

3.	Connect clevis to brake pedal with clevis pin and install the split pin to the clevis pin.

4. Install master cylinder.

5.	Connect vacuum hose to brake booster.

6.	Pay attention to the direction of the check valve when installing.

7.	Fill brake reservoir with brake fluid and bleed the system.

8.	Check for fluid leakage.

9.	Check and adjust the brake pedal (Refer to page 58-7).

10. After installation, apply sufficient grease to the clevis and brake pedal  contacting  points.

58-18	BRAKE LINE

BRAKE LINE

INSPECTION

1.	Check the brake lines for cracks, crimps and corrosion.

2.	Check the brake hoses for cracks, damage and leakage.

3.	Check the brake line flare nuts for damage and leakage.

INSTALLATION

1.	Install the brake hoses without twisting them.

2.   The brake lines should be installed away from sharp edges, weld beads or moving parts.

3.	Tighten the connections to the specified torque.

Flare nuts ......................... 13-17 Nm (130-170 kg.cm, 9-12 Ib.ft) Brake hose to front caliper ............................................

25-30 Nm (250-300 kg.cm, 18-22 Ib.ft)

PROPORTIONING  VALVE	58-19

PROPORTIONING  VALVE

Do not disassemble the proportioning valve. The proportioning valve regulates the distribution of fluid pressure to the front and rear brakes to prevent skidding in the event of rear wheel lock, and to obtain greater braking efficiency.

PROPORTIONING  VALVE  FUNCTION  TEST

1.	Connect two pressure gauges, one the input side and one to the output side of the proportioning valve.

NOTE

Be sure to bleed the system after you connect the pressure gauges.

2.	With the brakes applied, measure the input pressure and the output pressure. If the measured pressures are within the ranges as illustrated, the proportioning valve is good.

3.		Reconnect the brake lines in their original positions and bleed the system.

NOTE

This figure shows characteristics of the proportioning valve during pressure increase.

PROPORTIONING   VALVE   INSTALLATION

1.	Install the brake lines according to the illustration.

2.	Tighten the flare nuts and bleed the system.

Tightening  torque

Brake  tube  flare  nut  ......................................

13-17Nm (130-170 kgcm, 9-12 Ib.ft) Proportioning   valve  mounting  nut  .......................

8-12 Nm (80-120 kg.cm, 6-9 Ib.ft)

58-20	FRONT DISC BRAKE

FRONT DISC BRAKE

COMPONENTS

TORQUE : Nm (kg.cm, Ib.ft)

REPLACEMENT OF BRAKE PADS

The brake pads have wear indicators that contact the brake disc when the brake pad thickness becomes 2 mm (0.08 in.). The wear indicators will generate a squealing sound to warn the driver.

Removal

1.	Remove the lower bolt and lift the caliper assembly up and out of the way. Secure it with wire or some other retaining method.

2.	Remove the pads.

CAUTION

Do not depress the brake pedal while disassembling the pads.

FRONT DISC BRAKE	58-21

Inspection

1.	Check the pads for wear or oil contamination and replace, if necessary.

NOTE

The pads for the right and left wheels should be replaced at the same time. Never “split” or intermix brake pad sets. All four pads must be replaced as a complete set.

Standard value Service limit

Pad thickness mm (in.)	11(0.433)	2.0 (0.079)

2.	Check the shims for damage or deformation.

Installation

1.	Install the pad clips.

2.	Install the pads onto each pad clip.

NOTE

Position the pad with its wear indicator toward the disc side and facing upward.

3.	Seat the piston in the cylinder using special tool.

4.	Install the new pads. The shims are attached to the each pad as  illustrated.

NOTE

Never apply grease to the disc or pads.

58-22	FRONT DISC BRAKE

5.	Install the bolt and tighten to the specified value.

Tightening  torque  ...................................

22-32 Nm (220-320 kg.cm, 16-24 Ib.ft)

CALIPER ASSEMBLY Removal

1.	Remove the wheel and tire.

2.	Disconnect the brake hose.

3.	Remove the pads.

4.	Remove the sleeve.

5.	Remove the pin boot and sleeve boot.

6.	Remove the caliper assembly.

Disassembly

1.	Remove the piston boot.

2.	Remove the piston using compressed air.

CAUTION

1) Do not place fingers in front of the piston when using compressed air.

2) Be careful not to splatter the brake fluid.

3.	Remove the piston seal from the caliper using a screw driver.

Inspection

1.	Check the caliper for wear, damage, cracks and rust.

2.		Check the piston for rust, damage, cracks and wear on the outer surface.

3.	Check the sleeve and pin for damage and rust.

4.	Check the pad spring and boots for damage.

5.	Check the carrier for damage, rust, wear and cracks.

NOTE

1) Do not use sand paper on the piston surface.

2) All rubber parts must be replaced with new ones.

FRONT DISC BRAKE	58-23

6.	Inspect the disc using a caliper and dial gauge.

Standard value Service limit Thickness of disc mm (in.)	22 (0.876)		20 (0.797) Runout of disc mm (in.)		0.15 (0.006)

7.	If necessary, replace the brake disc.

1)  Remove the torque plate from the knuckle and suspend it with a wire.

2)   Using special tool, disconnect the driveshaft from the hub, and remove the brake disc and hub.

3)  Install the brake disc and hub assembly to the axle shaft.

4) Install the axle nut and washer, then tighten the axle nut.

Tightening torque  ..........................................

200-260 Nm (2000-2600 kg.cm, 145-188 Ib.ft)

5)   Install the torque plate onto the knuckle.

Assembly

1.	Clean all components with isopropyl alcohol except the pads and shim.

2.	Apply rubber grease to the piston seal and install the piston seal in the cylinder.

3.	Assemble the piston and piston boots according to the following procedure.

1)  Apply rubber grease to the caliper bore, outside surface of the piston and piston boot.

2) Install the piston boot on the piston as illustrated.

3)   Insert the piston boot in the inner groove of caliper and slide the piston into the caliper.

58-24	FRONT DISC BRAKE

4.	Assemble the sliding parts according to the following proce- dure.

1)  Apply rubber grease to the outside surface of the sleeve and pin, pin and sleeve bore of the caliper, pin boot and sleeve boot.

2)   Insert the boots into the groove of the caliper.

5. Install the pads.

NOTE

Do not apply grease to the disc or pads.

6.   Tighten the brake hose connecting bolt.

Tightening  torque	Nm (kg.cm, Ib.ft)

Sliding pin

Sliding bolt

Carrier mounting bolt

Brake hose mounting oil bolt

35-45 (350-450, 26-33)

22-32 (220-320, 16-24)

69-85 (690-850, 44-63)

25-30 (250-300, 18-22)

NOTE

1) Check that the surfaces of the pin and bolts are not damaged  before  tightening.

2) Bleed the system.

Depress the pedal several times and check for fluid leakage from all connecting parts.

REAR DRUM BRAKE

REAR DRUM BRAKE

COMPONENTS

TORQUE : Nm (kg.cm, Ib.ft)

REMOVAL

1.	Removing the wheel and tire, then remove brake drum.

2.	After removing the nut, remove the rear hub with the brake drum.

3.	Remove the abut end shoe spring, shoe hold down spring.

4.	Remove the shoes and adjuster as an assembly.

58-26	REAR DRUM BRAKE

INSPECTION

1.	Measure the brake drum inside diameter.

Check the runout of the brake drum using a dial indicator.

mm (in.) Standard value	Service limit

Inside diameter

Out-of-rounded of brake drum

203.2 (8)	205.2 (8.079)

0.015 (0.0006)

2.	Measure the brake shoe lining thickness.

Standard value	Service limit Lining  thickness	4.05 mm (0.159 in.)	 1.5 (0.059) mm (in.)

3.	Inspect the brake lining and drum for proper contact.

4.	Inspect the wheel cylinder outside for excessive corrosion and damage.

5.	Inspect the backing plate for wear or damage.

INSTALLATION

1.	Apply a specified grease to the locations indicated in the illustration and to each component.

o Shoe and backing plate contact surfaces o Shoe and anchor plate contact surfaces

Recommended   grease   .................................... Multipurpose grease SAE 3310, NLGI No.2

2.	Install the shoe hold down pin.

REAR DRUM BRAKE	58-27

3.	Assemble the return spring with the push rod shortened.

4.	After assembling the drum components, pull the parking brake lever all the way up several times.

WHEEL  CYLINDER  REPLACEMENT

1.	Remove the brake shoe.

2.	Disconnect the brake tube.

3.	Remove the wheel cylinder assembly.

4.	Remove the duct boot.

5.	Remove the piston and piston cup.

6.	Drive out the return spring.

7.		Before assembling the wheel cylinder, inspect the following points.

1)  Check the cylinder and piston for wear, damage and rust.

2)  Check the cylinder body for damage and cracks.

3)  Check the contact surface of the piston and shoes for wear.

4) Check the piston spring for looseness.

8.	Assembly is the reverse of the removal procedure.

NOTE

1)  Clean the cylinder and inner part with isopropyl alcohol before assembly.

2) Apply enough brake fluid to piston cups and cylinder.

3)  Be sure to use new piston cups and dust boots.

NOTE

Be careful not to lose the steel ball in the bleeder.

REAR DISC BRAKE

REAR DISC BRAKE

COMPONENTS

DISC BRAKE PAD REMOVAL

1.	Remove the wheel

2.		Remove the screw which holds the trailing shoe key onto the anchor plate.

3.	Release the parking cable adjusting nut.

4.	Remove the parking cable.

REAR DISC BRAKE	58-29

5.	Remove the brake fluid hose.

6.	Remove the caliper assembly

NOTE

Support the caliper with a wire or some other means to prevent damage to the brake hose.

7.	Remove the ABS wheel speed sensor.

8.	Remove the wheel bearing nut

9.	Remove the wheel hub and dust cover.

INSPECTION

1.	Check the pads for wear or oil contamination and replace if necessary.

NOTE

The pads for the right and left wheels should be replaced at the same time

Pad thickness wear limit 0.8 mm

2.	Check the leading and trailing shoe keys and retaining screw for damage, or wear. Replace the keys and retaining screw at the same time the pads are replaced.

3.	Check for worn or damaged dust boots if dust or mud has entered the caliper assembly through this seal, the caliper assembly must be replaced or rebuilt.

58-30	REAR DISC BRAKE

INSTALLATION

1.		Before replacing the brake pads, remove brake fluid from the master cylinder reservoir until it is half full.

CALIPER Disassembly

1.	Remove the caliper mounting bolts, and remove the caliper assy.

2.	Remove the carrier bolt, and remove the carrier with brake pad.

3.	Remove the piston boot.

4.	Remove the piston assy with special tool to twist the piston out of the caliper body.

5. Remove piston seal with finger tip.

CAUTION

Do not use sharpened tools to prevent damage to inner cylinder.

REAR DISC BRAKE	58-31

Assembly

1.	Check for worn, damaged, or rusted piston bore and piston. Replace the damaged parts if necessary.

2.	Check for damaged piston seal, boot, and pin insulators.

3.	Apply a recommended lubricant to the fallowing parts

4.	Push the piston into the caliper with special tool.

58-32	PARKING BRAKE

PARKING BRAKE

COMPONENTS

REMOVAL

1.	Remove the rear console.

2.	Loosen the adjusting nut and detach the parking brake cable.

3.	Detach the parking brake switch assembly.

4.	Remove the parking brake lever assembly.

5.	Remove the wheel and tire.

6.	Remove the brake drum.

7.	Remove the brake shoes as outlined before.

8.	Detach the parking brake cable from the parking lever.

PARKING BRAKE	58-33

7.		Remove the parking brake cable retaining ring in the rear of the backing  plate.

8.	Remove the rear seat cushion assembly and roll up the carpet.

9.	Loosen the parking brake cable clamp and remove the parking brake cable assembly.

INSPECTION

1.	Check the parking brake switch operation.

2.	Check the parking brake lever ratchet for wear.

3.	Check the parking brake cable for fraying or damage.

INSTALLATION

1.	Check the parking brake cables for left and right identification marks and install accordingly.

2.	Move the adjuster lever all the way back when installing the shoe-to-shoe  spring.

PARKING BRAKE

3.	Install the grommets in the direction shown in the illustration.

4.	Apply a coating of the specified grease to the sliding parts of the ratchet plate and ratchet pawl.

Specified  grease  .............................................. Multipurpose grease SAE J310, NLGI No. 2

5.	After installing the cable adjuster, adjust the parking brake lever stroke.